Charlie 2G and 3G

Field Test Display Specification

| Status: | Approved |
| --- | --- |
| Version: | 1.0 |
| Doc. number: | DTS08337-EN-1.0 |
| Date: | 8.4.2004 |
| Author: | Jari.P.Jokinen |
| Approval: | Tero Tamminiemi |
| Function: | Field Testing |
| File Name: | Charlie_FTD_specification_1.0.doc |
| File location: | Notes: \\eslns47\NMP\PCP\WCDMA\IBU DocMan |

Change history:

| 1.0 | 8.4.2004 | Jari Jokinen | Approved | Modified and approved |
| --- | --- | --- | --- | --- |
| 0.1 | 6.4.2004 | Jari Jokinen | Draft | Charlie 2G FTD v. 1.0 and Charlie 3G FTD v. 0.4 were put together |

1	INTRODUCTION	9

2	USING THE FIELD TEST DISPLAYS	9

2.1	General	9

2.2	Activating the Field Test Displays	9

2.3	Deactivating the Field Test Displays	9

2.4	Field Test Display modes	9

2.4.1	Modes belonging in the use of the displays	9

2.4.2	Modes belonging in the field test display data	11

Signal and battery level	11

3	Information on the displays	12

Group 01: GSM Signaling displays, which can be visible to network operators	13

Display 01.01: Information of the serving cell (Version: 3.25, Status: Approved)	13

Display 01.02: More information of the serving cell (Version: 1, Status: Approved)	15

Display 01.03: Information of the serving cell, 1st and 2nd neighbor (Version: 3.25, Status: Approved)	16

Display 01.04: Information of the 3rd, 4th and 5th neighbor (Version: 3.25, Status: Approved)	17

Display 01.05: Information of the 6th, 7th and 8th neighbor (Version: 3.25, Status: Approved)	19

Display 01.06: Network selection display (Version: 1, Status: Approved)	20

Display 01.07: System information bits for serving cell (Version: 3.19, Status: Approved).	21

Display 01.08: Paging repeat period, TMSI, periodic location update, AFC and AGC (Version: 3.19, Status: Approved)	22

Display 01.09: Network parameters (Version: 4.0, Status: Approved)	23

Display 01.10: Ciphering, hopping DTX status and IMSI (Version: 1, Status: Approved)	24

Display 01.11: Uplink DTX switching display (Version: 1, Status: Approved)	25

Display 01.12: Storing and removing BTS_TEST carrier (Version: 3.13, Status: Approved)	25

Display 01.13: Toggle Cell Barred Status (Version: 1, Status: Approved)	27

Display 01.14: Modify "last used band" (Version: 1, Status: Approved)	28

Group 02: GSM Registration and Mobility Displays	30

Display 02.01: Reset Registration and Mobility counters (Version: 1, Status: Approved)	30

Display 02.02: Neighbor Measurement Counter Display (Version: 1, Status: Approved)	30

Display 02.03: Search and reselection counter display (Version: 1, Status: Approved)	31

Display 02.04: Location update attempts counters (Version: 1, Status: Approved)	32

Display 02.05: Handover display, INTER CELL (Version: 1, Status: Approved)	32

Display 02.06: Handover display,  INTRA CELL (Version: 1, Status: Approved)	33

Display 02.07: L2 display (Version: 1, Status: Approved)	34

Group 03: GSM memory handling	36

Display 03.01: SIM information 1 (Version: 1, Status: Approved)	36

Display 03.02: SIM information 2 (Version: 1, Status: Approved)	36

Group 04: GSM Test Timers, SMS and Call Counter Displays	38

Display 04.01: Reset test timers to zero and restart them (Version: 1, Status: Approved)	38

Display 04.02: Enable or disable test timers (Version: 1, Status: Approved)	38

Display 04.03: Test timer display (Version: 1, Status: Approved)	39

Display 04.04: Reset SMS and Call Counters to zero (Version: 1, Status: Approved)	40

With this display all counters listed on the Display 04.05: Call attempts counters (Version: 1, Status: Approved),	40

Display 04.05: Call attempts counters (Version: 1, Status: Approved)	40

Display 04.06: Information about reasons for call clearing (Version: 1, Status: Approved)	41

Display 04.07: SMS attempts counters (Version: 1, Status: Approved)	42

Display 04.08: SMS timeout counters (Version: 1, Status: Approved)	42

Display 04.09: Toggle transmitter functionality (Version: 1, Status: Approved)	43

Group 05: GSM Data Displays	45

Display 05.01: HSCSD, Timeslot information display (Version: 3.19, Status: Approved)	45

Display 05.02: HSCSD, Timeslot power control level display (Version: 3.19, Status: Approved)	45

Display 05.03: Transparent data information (Version: 1, Status: Approved)	46

Display 05.04: Non transparent data (Version: 1, Status: Approved)	47

Display 05.05: Facsimile information (Version: 1, Status: Approved)	48

Display 05.06: CSD Server, main data display (Version: 1, Status: Approved)	49

Display 05.07: CSD Server, active DTE info 1/2, call related info (Version: 3.13, Status: Approved)	51

Display 05.08: CSD Server, active DTE info 2/2, GPDS, socket, conference info (Version: 1, Status: Approved)	52

Display 05.09: GPRS Rel99 Display (Version: 1, Status: Approved)	53

Display 05.10: MAC Interface Display (Version: 1, Status: Approved)	55

Display 05.11: CSD Server Main Data Display, Control 2001 (Version: 1, Status : Approved)	55

Display 05.12: Control DTE and Call Info Displays, Control 2001 (Version: 1, Status: Approved)	57

Display 05.13: Active DTE Display xx, Control 2001 (Version: 1, Status: Approved)	58

Display 05.14: Active Call Display (Version: 1, Status: Approved)	59

Display 05.15: Common CSD Display (Version: 1, Status: Approved)	60

Display 05.16: Current BC-IE (Version: 1, Status: Approved)	62

Display 05.17: LLC (Version: 1, Status: Approved)	63

Group 06: GPRS Signaling displays	64

Display 06.01: Information of the current GPRS state and previous TBF configuration  (Version: 3.17, Status: Approved)	64

Display 06.02: Previous UL TBF establishment (Version: 3.15, Status: Approved)	65

Display 06.03: Information of the GMM state (Version: 5.0, Status: Approved)	66

Display 06.04: Values of P-TMSI, RAC, SMS radio priority, Ciphering and Non-DRX parameters  (Version: 3.15, Status: Approved)	67

Display 06.05: GPRS Network parameters  (Version: 3.15, Status: Approved)	68

Display 06.06: Packet control channel parameters (Version: 4.0, Status: Approved)	69

Display 06.07: (Packet) System information parameters (Version: 5.0, Status: Approved)	69

Display 06.08: Cell re-selection parameters (serving cell) (Version: 3.15, Status: Approved)	70

Display 06.09: GPRS information of the serving cell, 1st, 2nd, and 3rd neighbor (Version: 3.19, Status: Approved)	71

Display 06.10: GPRS information of the 4th,5th and 6th neighbor (Version: 3.15, Status: Approved)	73

Display 06.11: GPRS information of the serving cell and 6 neighbors  (Version: 3.16, Status: Approved)	74

Display 06.12: EGPRS specific parameters (Version: 3.24, Status: Approved)	76

Display 06.13: EGPRS BEP parameters (Version: 5.0, Status: Approved)	77

Group 07: GPRS Data protocol displays	78

Display 07.01: Information of the first  active PDP context (Version: 3.22, Status: Approved)	78

Display 07.02: Information of the second active PDP context (Version: 3.22, Status: Approved)	79

Display 07.03: Information of the RLC state (Version: 1, Status: Approved)	80

Display 07.04: RLC parameters (Version: 1, Status: Approved)	81

Display 07.05: RLC data block counters (Version: 1, Status: Approved)	81

Display 07.06: LLC data block counters (Version: 1, Status: Approved)	82

Display 07.07: LLC Ciphering parameters (Version: 1, Status: Approved)	82

Display 07.08: LLC parameters of the first SAPI (Version: 1, Status: Approved)	83

Display 07.09: LLC parameters of the second SAPI (Version: 1, Status: Approved)	83

Display 07.10: SNDC Data counters (Version: 1, Status: Approved)	84

Display 07.11: PPP information (Version: 1, Status: Approved)	85

Display 07.12: ERLC Data information_1 (Version: 3.32, Status: Approved)	85

Display 07.13: ERLC Data information_2 (Version: 3.12, Status: Approved)	86

Display 07.14: ERLC Data counters (Version: 3.12, Status: Approved)	86

Display 07.15: System status (Version: 5.0, Status: Approved)	87

Display 07.16: Information of the selected PDP context (Version: 5.0, Status: Approved)	87

Display 07.17: QoS of the selected PDP context 1 (Version: Charlie spec)	89

Display 07.18: QoS of the selected PDP context 2 (Version: Charlie spec)	90

Display 07.19: PDCP parameters of the selected PDP context (Version: 3.22, Status: Approved)	90

Display 07.20: RFC2507 parameters of the selected PDP context (Version: 3.22, Status: Approved)	91

Group 08: GPRS Measurement displays	93

Display 08.01: Power control parameters (Version: 3.19, Status: Approved)	93

Display 08.02: The previous channel quality report (Version: 3.20, Status: Approved)	94

Display 08.03: NC and EM parameters (Version: 1, Status: REJECTED)	94

Display 08.04: The previous NC report, part 1 (Version: 1, Status: REJECTED)	95

Display 08.05: The previous NC report, part 2 (Version: 1, Status: REJECTED)	96

Display 08.06: The previous EM report, part 1 (Version: 1, Status: REJECTED)	97

Display 08.07: The previous EM report, part 2 (Version: 1, Status: REJECTED)	97

Display 08.08: Downlink power control parameters (Version: 1, Status: REJECTED)	98

Group 09: GPRS Test counter displays	99

Display 09.02: TBF counters (Version: 5.0, Status: Approved)	99

Display 09.10: GPRS attach and detach counters (Version: 5.0, Status:Approved)	100

Display 09.11: Periodic routing area update counters (Version: 5.0, Status: Approved)	101

Display 09.12: Routing area update counters (Version: 5.0, Status: Approved)	101

Display 09.13: PDP context counters (Version: 5.0, Status: Approved)	102

Display 09.14: SMS over GPRS counters (Version: 5.0, Status: Approved)	103

Display 09.17: MS initiated cell re-selection counters (Version: 5.0, Status: Approved)	104

Display 09.18: Network initiated cell re-selection counters (Version: 5.0, Status: Approved)	105

Group 10: GSM DSP displays	106

Display 10.01: DSP Data Display 1 (Version: 1, Status: Approved)	106

Display 10.02: DSP Data Display 2 (Version: 1, Status: Approved)	106

Display 10.03: DSP Data Display 3 (Version: 1, Status: Approved)	107

Display 10.04: FER measuments for sub ch0: REAL, FULL and SUB values (Version: 3.24, Status: Approved)	108

Display 10.05: FER measuments for sub ch1: REAL, FULL and SUB values (Version: 3.25, Status: Approved)	109

Display 10.06: FER measuments for sub ch2: REAL, FULL and SUB values (Version: 3.25, Status: Approved)	110

Group 11: CDSP GSM AMR displays	111

Display 11.01: AMR1:Information of the Active Mode Set (Version: 3.16, Status: Approved)	111

Display 11.02: AMR2: Information of the mode control (Version: 3.16, Status: Approved)	112

Display 11.03: AMR3: Information of the link quality estimation (Version: 3.16, Status: Approved)	113

Display 11.04: AMR4: Control AMR FTD data pre processing (Version: 1, Status: Approved)	115

Display 11.05: AMR5: Control AMR algorithms (Version: 1, Status: Approved)	115

Group 12: CDSP SW Platform	117

Display 12.01: CDSP version and date (Version: 3.13, Status: Approved)	117

Display 12.02: Reset display (Version: 1, Status: Approved)	117

Display 12.03: CDSP error and reset counters and MCU reset time (Version: 3.13, Status: Approved)	118

Display 12.04: Counters for specific CDSP non-fatal errors (Version: 1, Status: Approved)	119

Display 12.05: CDSP fatal error codes (Version: 3.13, Status: Approved)	120

Display 12.06: CDSP non-fatal error codes (Version: 1, Status: Approved)	121

Display 12.07: CDSP fatal error code timestamps (Version: 3.31, Status: Approved)	122

Display 12.08: CDSP non-fatal error code timestamps (Version: 3.31, Status: Approved)	123

Display 12.09: Counters for failed application starts (Version: 1, Status: Approved)	124

Display 12.10: Common CDSP test data display (Version: 3.22, Status: Approved)	125

Group 41: WCDMA CDSP displays	126

Display 41.01: RACH MSG TX profile (Version: 3.19, Status: Approved)	126

Display 41.02: Dedicated uplink channel power control status (Version: 3.24, Status: Approved)	127

Display 41.03: Dedicated downlink channel power control status (Version: 3.19, Status: Approved)	128

Display 41.04: WCDMA AFC status (Version: 3.13, Status: Approved)	129

Display 41.05: CDSP load status (Version: 3.19, Status: Approved)	130

Display 41.06: Downlink receiver gain control status (Version: 3.28, Status: Approved)	131

Display 41.07: WCDMA rake receiver status (Version: 3.13, Status: Approved)	132

Display 41.08: WCDMA decoder status (Version: 3.13, Status: Approved)	133

Display 41.09: Common WCDMA DSP CS scratchpad display (interactive) (Version: 3.19, Status: Approved)	134

Display 41.10: FDD neighbour cell summary (Version: 3.13, Status: Approved)	135

Display 41.11: FDD ranking summary (Version: 3.24, Status: Approved)	136

Display 41.12: FDD Frequency summary (Version: 3.13, Status: Approved)	137

Display 41.13: FDD intra frequency neighbour summary (Version: 3.13, Status: Approved)	138

Displays 41.14 – 41.15: FDD inter frequencies 1 and 2 neighbour summary (Version: 3.13, Status: Approved)	141

Display 41.16: FDD mode GSM cell detection summary  (Version: 3.13, Status: Approved)	143

Display 41.17: FDD detailed cell info (interactive):  (Version: 3.16, Status: Approved)	144

Groups 42-45: WCDMA Layer 1 Field Test Displays	146

Group 46: WCDMA RAN System displays	147

Display Display 46.01: RRC Global Status (Version: 6.2, Status: Approved)	147

Display 46.02: PEER message MSC (Version: 6.2, Status: Approved)	147

Display 46.03: RNTI Values (Version: 6.2, Status: Approved)	149

Display 46.04: Ciphering Capability (Version: 6.2, Status: Approved)	150

Display 46.05: Cell Selection - Screen 2 (Version: 6.2, Status: Approved)	150

Display 46.06: FDD BTS Carrier Lock Mode (Version: 6.2, Status: Approved)	151

Display 46.07: Counter Reset (Version: 6.2, Status: Approved)	151

Display 46.08: Call Failure Reasons (Version: 6.2, Status: Approved)	152

Display 46.09: L1 State (Version: 6.2, Status: Approved)	153

Display 46.10: Cell Reselection - Screen 1 (Version: 6.2, Status: Approved)	153

Display 46.11: Radio Access Bearer Information (Version: 6.2, Status: Approved)	154

Display 46.12: Radio Bearer Information (Version: 6.2, Status: Approved)	155

Display 46.13: Radio Bearer Information (Version: 6.2, Status: Approved)	156

Display 46.14: DPCH Change Count (Version: 6.2, Status: Approved)	156

Display 46.15: Tracker Feature Enable/Disable (Version: 6.2, Status: Approved)	157

Display 46.27: Voice RLC-U Information (Version: 6.2, Status: Approved)	157

Display 46.28: DTX Enable/Disable input (Version: 6.2, Status: Approved)	158

Display 46.60: 3G to 2G Reselections (Version: 6.2, Status: Approved)	159

Display 46.61: 3G to 2G ISHO #1 (Version: 6.2, Status: Approved)	160

Display 46.62: 3G to 2G ISHO #2 (Version: 6.2, Status: Approved)	161

Display 46.63: 3G to 2G ISHO #3 (Version: 6.2, Status: Approved)	161

Group 47: WCDMA RRC displays	163

Display 47.01: RRC Global State Change Counters (Version: 6.2, Status: Approved)	163

Display 47.02: Counters for Timeouts - Screen 1 (Version: 6.2, Status: Approved)	163

Display 47.03: Counters for Timeouts - Screen 2 (Version: 6.2, Status: Approved)	164

Display 47.04: Peer Message Count- Screen1 (Version: 6.2, Status: Approved)	165

Display 47.05: Peer Message Count- Screen2 (Version: 6.2, Status: Approved)	165

Display 47.06: Peer Message Count-Screen3 (Version: 6.2, Status: Approved)	166

Display 47.07: Peer Message Count-Screen4 (Version: 6.2, Status: Approved)	167

Display 47.08: Peer Message Count-Screen5 (Version: 6.2, Status: Approved)	168

Display 47.09: Peer Message Count-Screen6 (Version: 6.2, Status: Approved)	169

Display 47.10: Peer Message Count-Screen7 (Version: 6.2, Status: Approved)	169

Group 48: WCDMA RLC displays	171

Display 48.01: Set AM Bearer ID (Version: 6.2, Status: Approved)	171

Display 48.02: RLC AM State information (Version: 6.2, Status: Approved)	171

Display 48.03: RLC AM Counters 1 (Version: 6.2, Status: Approved)	172

Display 48.04: RLC AM Counters 2 (Version: 6.2, Status: Approved)	172

Display 48.05: RLC AM Counters 3 (Version: 6.2, Status: Approved)	173

Display 48.06: RLC AM Counters 4 (Version: 6.2, Status: Approved)	174

Display 48.07: RLC AM Counters 5 (Version: 6.2, Status: Approved)	174

Display 48.08: RLC AM Counters 6 (Version: 6.2, Status: Approved)	175

Display 48.09: RLC AM Tx Buffer Occupancy (Version: 6.2, Status: Approved)	175

Group 49: WCDMA MAC displays	177

Display 49.01: Basic Radio Bearer Information (Version: 6.2, Status: Approved)	177

Display 49.02: Basic DCH Information (Version: 6.2, Status: Approved)	178

Display 49.04: Radio Bearer ID and SUB ID input (Version: 6.2, Status: Approved)	180

Display 49.05: Radio Bearer Information (Version: 6.2, Status: Approved)	180

Display 49.06: Input DCH ID (Version: 6.2, Status: Approved)	182

Display 49.07: DCH Information (Version: 6.2, Status: Approved)	182

Group 50: WCDMA WPH displays	184

Display 50.01: WPH Connection counts (Version: 6.2, Status: Approved)	184

Display 50.10: WPH L1 Received Error Codes (Version: 6.2, Status: Approved)	184

Display 50.11: WPH L1 Received Error Timestamp (Version: 6.2, Status: Approved)	185

Display 50.12: WPH L1 PId list before error (Version: 6.2, Status: Approved)	186

Display 50.13: WPH L1 Rx signal list before error (Version: 6.2, Status: Approved)	187

Display 50.14: WPH L1 Tx signal list before error (Version: 6.2, Status: Approved)	188

Display 50.15: WPH L1 list of previous warnings received before error (Version: 6.2, Status: Approved)	189

Group 61: Common EM (Energy Management) displays	190

Display 61.01: Basic EM information (Version: 3.16, Status: Approved)	190

Display 61.02: Basic BATMON4 information (Version: 1, Status: Approved)	191

Display 61.03: Advanced BATMON4 algorithm information (Version: 1, Status: Approved)	192

Display 61.04: Battery Monitor setup and log information (Version: 1, Status: Approved)	193

Display 61.05: PSM (Phone State Monitor) information 1 (Version: 3.16, Status: Approved)	194

Display 61.06: PSM (Phone State Monitor) information 2 (Version: 1, Status: Approved)	195

Display 61.07: PSM (Phone State Monitor) information 3 (Version: 3.16, Status: Approved)	195

Display 61.08: Basic charging information (Version: 1, Status: Approved)	196

Display 61.09: Extended charging information (Version: 1, Status: Approved)	197

Display 61.10: Extended charging information 2 (Version: 3.16, Status: Approved)	199

Display 61.11: Configuration parameters etc. (Version: 3.16, Status: Approved)	200

Display 61.12: Varying EM information 1 (Version: 3.15, Status: Approved)	200

Display 61.13: Varying EM information 2 (Version: 3.15, Status: Approved)	201

Group 62: Common general displays	202

Display 62.01: Reasons for SW resets (Version: 3.32, Status: Approved)	202

Display 62.02: Counters for resets 1 (Version: 3.32, Status: Approved)	203

Display 62.03: Counters for resets 2 (Version: 3.19, Status: Approved)	204

Display 62.04: Counters for DSP resets  (Version: 3.32, Status: Approved)	205

Display 62.06: Lights status control (Version: 1, Status: Approved)	206

Display 62.07: Disabling R&D Test Displays (Version: 1, Status: Approved)	206

Display 62.08: Reliability Test Display (Version: 1, Status: Approved)	207

Display 62.10: Information about MCU and DSP software versions (Version: 3.23, Status: Approved)	207

Display 62.11: Information about HW version (Version: 3.18, Status: Approved)	208

Display 62.12: Information about PPM (Version: 3.24, Status: Approved)	208

Group 63: Common OS status displays	210

Display 63.01: OS Block usage statistics (Blocks 1-8) (Version: 3.19, Status: Approved)	210

Display 63.02: OS Block usage statistics (Blocks 9-16) (Version: 3.19, Status: Approved)	211

Display 63.03: OS block allocation errors (Version: 3.32, Status: Approved)	212

Display 63.04: Memory status before reset (Version: 3.19, Status: Approved)	212

Display 63.05: OS pre-reset error status (Version: 3.32, Status: Approved)	213

Display 63.06: OS current error status (Version: 3.31, Status: Approved)	214

Display 63.07: OS pre-reset warning status (Version: 3.31, Status: Approved)	214

Display 63.08: OS current warning status (Version: 3.31, Status: Approved)	215

Display 63.09: OS pre-reset remark status (Version: 3.31, Status: Approved)	216

Display 63.10: OS current remark status (Version: 3.31, Status: Approved)	216

Display 63.11: Heap system status (Version: 3.19, Status: Approved)	217

Display 63.12: Heap top users (Version: 3.31, Status: Approved)	218

Display 63.13: Failed assertions (Version: 3.19, Status: Approved)	218

Display 63.14-63.29: Reserved for future use (Version: 1, Status: - )	219

Display 63.30: Information of OS_SYSTEM_STACK (Version: 3.19, Status: Approved)	219

Display 63.31: Select task set (Version: 3.19, Status: Approved)	219

Displays 63.32 – 63.99: Information about tasks (Version: 3.31, Status: Approved)	220

Group 64: Common Audio displays	222

Display 64.01: Read DSP Memory (Version: 3.13, Status: Approved)	222

Display 64.02: Signal generators (Version: 3.13, Status: Approved)	223

Display 64.03: Accesory mode status (Version: 3.13, Status: Approved)	223

Display 64.04: Downlink audio display (Version: 3.13, Status: Approved)	224

Display 64.05: Uplink audio display (Version: 3.13, Status: Approved)	225

Display 64.06: Microphone Active Gain Control (AGC) (Version: 3.13, Status: Approved)	225

Display 64.07: Acoustic Echo Canceller (AEC) (Version: 3.13, Status: Approved)	226

Display 64.08: Dynamic/Multi-Band Range Controller (DRC/MDRC) (Version: 3.13, Status: Approved)	227

Display 64.09: Control DSP audio enhancements switch 1 (Version: 3.15, Status: Approved)	228

Display 64.10: Control DSP audio enhancements switch 2 (Version: 3.15, Status: Approved)	229

Display 64.11: Control DSP audio enhancements switch 3 (Version: 3.15, Status:Approved)	230

Display 64.12: SpCs (Version: 3.13, Status: Approved)	231

Display 64.13: SpC counters (Version: 3.16, Status: Approved)	231

Group 65: Common PPC (Product Performance Counters) displays	233

Display 65.01: PPC Counters 1 (Version: 3.19, Status: Approved)	233

Display 65.02: PPC Counters 2 (Version: 3.19, Status: Approved)	233

Display 65.03: PPC Counters 3 (Version: 3.19, Status: Approved)	234

Display 65.04: Camera PPCs (Version: 3.27, Status: Approved)	235

Display 65.05: HW UI counters (Version: 3.28, Status: Approved)	235

Group 73: Common Accessory displays	237

Display 73.01: Accessory mode information 1 (Version: 1, Status: Approved)	237

Display 73.02: Accessory mode information 2 (Version: 1, Status: Approved)	237

Display 73.03: Accessory mode information 3 (Version: 1, Status: Approved)	238

Display 73.04: Vibra information (Version: 1, Status: Approved)	238

Display 73.06: Ambient light sensor information 1 (Version: 1, Status: Approved )	239

Display 73.07: Ambient light sensor information  2 (Version: 1, Status: Approved )	240

Group 74: Common Memory Management displays	241

Display 74.01: Information of MeM component versions (Version: 1, Status: Approved)	241

Display 74.02: Information of PMM Plain area (Version: 1, Status: Approved)	241

Display 74.03: Information of PMM Classified area (Version: 1, Status: Approved)	242

Display 74.04: Information of PMM Protected area (Version: 1, Status: Approved)	242

Display 74.05: PMM Storing information (Version: 1, Status: Approved)	243

Display 74.06: PMM Error information 1 (Version: 1, Status: Approved)	244

Display 74.07: PMM Error information 2 (Version: 1, Status: Approved)	244

Display 74.08: Information of PMM Clean-up (Version: 1, Status: Approved)	245

Group 81: Multimode protocol displays	245

Display 81.01: Force protocol between GSM and WCDMA (Version: 3.26, Status: Approved)	245

Display 81.02: Toggle Integrity Protection Mode (Version: 3.26, Status: Approved)	246

Group 99: Charlie specific displays	248

Display 99.01: PPC Counters	248

Display 99.04: Einfo on FTD	248

4	REFERENCES	249

# INTRODUCTION

This document specifies the features of Charlie Field Test Displays. Field Test Display, i.e. engineering test mode, is a configurable software feature that is available in application list.

Field Test Display is useful for testing mobile phones during development or for verifying the operation of the network.

# USING THE FIELD TEST DISPLAYS

## 2.1	General

When the Field Test Display is active, the phone can be used in a similar way than without it.

When application key is pressed FTD will stay on backround and it can be activated again by pressing application key for about one second and after that select.

It is possibly to disable FTD from display 62.07 and after that FTD application icon will be deleted.

FTD can be restored installing it again.

NOTE:  FTD application should not interfere phone operation, because of allmost all field testing will be made so that FTD is in active mode. When phone will go to customer, FTD is not activated and phone should not care of this. This will be a challenge to FTD application

During proper power off (use of power key or RE-CHARGE indication) some valuable data (e.g. the number of current field test display) is stored onto the permanent memory. Next time when the phone is powered on the lastly selected field test display will be automatically re-activated.

## 2.2	Activating the Field Test Displays

The Field Test Display is located in other folder and can be activated as follows:

1. Press application key

2. Scroll in the application list to other folder and from there field test display item

3. Press the Options button and Open or press rocker to select

The field test data display will then appear in a moment and first field test display is shown.

## 2.3	Deactivating the Field Test Displays

Test display can be deactivated by pressing Exit– key or

1. Press the Options button

2. Choose Exit

## 2.4	Field Test Display modes

### 2.4.1	Modes belonging in the use of the displays

There are tree Field Test Displays modes:

- Execute mode

- Data display mode

- Help mode

Different modes are marked in this specification as follows:

Execute mode:		              Data display mode:

**************************        +++++++++++++++++++++++++++

*    Execute    display   *       + Parameter 1   aaaa      +

* With this display it is *       + Parameter 2   bbbb      +

* possible  to  reset     *       + Parameter 3   cccc      +

* registration / mobility *       + Parameter 4   dddd      +

* counters. Use options   *       + Parameter 5   eeee      +

* to change.              *       + Parameter 6   ffff      +

*                         *       + Parameter 7   gggg      +

*                         *       + Pa 8 / Pa 9   hhhh  iiii+

*                         *       + Pa10 / Pa11   jjjj  kkkk+

**************************        +++++++++++++++++++++++++++

OPTIONS	   EXIT           OPTIONS	       EXIT

The execute mode can be selected by choosing options from every execute display. Example, select group 2 and display number 1, select options, select reset handover counters and confirm with Ok button. Timers are reseted as soon as the OK button has been pressed from options menu and the execute mode takes over.

During the data display mode, parameter text will define what parameter is visible, there is no help mode so all needed information from that parameter is in that text field.

The rocker key (<,>^,v) offer an easy way to switch to another test displays without using the options menu. Rocker  (<,>) can be used to change different display groups, like Group1: GSM signaling displays. Rocker (^,v) can be used to change different data displays from one group.

Display will support different colourcodes, all parameters which are in normal range are coloured with green. When some parameter will go to critical level, example tx power is too low parameter is coloured with red.  NOTE Will be implmented if enough time to do that.

Help mode:  selection phase

+++++++++++++++++++++++++++

+ Parameter 1   aaaa      +

+ Parameter 2   bbbb      +

+ Parameter 3   cccc      +

+ Parameter 4   dddd      +

+ Parameter 5   eeee      +

+ Parameter 6   ffff      +

+ Parameter 7   gggg      +

+ Pa 8 / Pa 9   hhhh  iiii+

+ Pa10 / Pa11   jjjj  kkkk+

+++++++++++++++++++++++++++

OPTIONS	    EXIT

It is possible to view help text from every parameter. By pressing “copy-paste” button in Data mode, selection menu appears (highlight- yellow) help text will appear by selecting parameter and by pressing “rocker”. Help text’s are specified in description field for every parameter.

| Abbreviation | Server | Sub Block ID | Mode | Description (HELP TEXT) |
| --- | --- | --- | --- | --- |
| bbb | GSS | FTD_SB_CARRIER | R,I,O | When mobile is on TCH: DCH carrier number of serving cell in decimal. When mobile is NOT on TCH: CH means carrier number of serving cell in decimal. If hopping is on, used channels are scrolled when display is updated |

Help mode:  help text phase, example

++++++++++++++++++++++++++++++

+ When mobile is on TCH:     +

+ DCH carrier number of      +

+ serving cell in decimal.   +

+ When mobile is NOT on TCH: +

+ CH means carrier number    +

+ of serving cell in decimal.+

+ If hopping is on, used     +

+ channels are scrolled when +

+ display is updated.        +

++++++++++++++++++++++++++++++

BACK

### 2.4.2	Modes belonging in the field test display data

The displayed data itself has three different modes:

- R = R&D Mode (allows all data presented on the display)

- O = Operator Mode  (only network related displays presented on the display)

- I = Infra vendor mode (only network related and few R&D displays presented on the display)

The presented data are divided into displays so, that data with same mode are located in the own displays. So, some displays can be activated/deactivated to be visible or not by the R&D people.

## Signal and battery level

When Field Test Display is active, normal signal and battery level bars are visible.

# Information on the displays

Length of a field is shown in the display picture by amount of the letters. Two notations are currently used. Examples of an old and new notations:

|  | Old notation | New notation |
| --- | --- | --- |
| Field with 5 letters, field name a | aaaaa | a(5) |
| Field with 4 letters, field name h | hhhh | h(4) |

If a value is not available, 'x' is shown for all the digits. If data to be displayed is shorter than the field reserved to show that data, text  is aligned left and numbers right.

In this specification the layouts of field test displays is drawn and all needed data items are shown in each display. In the descriptive tables are mentioned the server, from which the data is got, the sub block ID of the server message for the data, description of the data and the mode (R, I or O) for each data item.

As so many field test display as possible are defined to be common. The rest of the displays are divided based on protocol (GSM, TDMA, CDMA, PDC, WCDMA, TETRA). All displays are divided into different groups based on that and in every group display numbering starts from 01.

Format of the field is shown in Format column using next symbols:

| Symbol | Description |
| --- | --- |
| DW:D | double word, 32bit value shown in decimal |
| DW:H | double word, 32bit value shown in hexadecimal |
| W:D | word, 16bit value shown in decimal |
| W:H | word, 16bit value shown in hexadecimal |
| B:D | byte, 8bit value shown in decimal |
| B:H | byte, 8bit value shown in hexadecimal |
| S | String |

- Group 01: GSM Signaling displays, which can be visible to network operators

The network operators can activate these signaling displays. Before field test displays are visible, mobile has to be modified. With the normal production mobile, field test displays are not working.

Display 01.01: Information of the serving cell (Version: 3.25, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Channel num   abbb      +

+ Rx level      ccc       +

+ Tx power lev  ddd       +

+ GPRS attach   qq        +

+ TS / TA       e    ff   +

+ Rx / R time   g    mmmm +

+ C1 / C2       nnn  ppp  +

+ Band/ CHty    rr   oooo +

+ Amr U/AMR D   x    y    +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.25	Approved	Note about “ERR” text added to n and p fields / Inspection 11-Nov-02, SWAM ADB 02111944685

3.22	Approved	Approved / Inspection 15-Oct-02, SWAM ADB TS02101549586

3.21	Proposal	Q-field explanation changed / ISA RM-RIM tool 009-28

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	B-field explanation corrected, approved/ Inspection 09-Aug-02, SWAM ADB TS02073044970

3.12	Proposal	05-Jun-2002	AnttiE	O-field modified by Tapio Saarinen.

1	Approved	24-Apr-2002	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | GSS | FTD_SB_HOPPING |  | R,I,O | H, if carrier numbers are scrolled when hopping is on. Otherwise '.' |
| bbb | GSS | FTD_SB_CARRIER |  | R,I,O | When mobile is on TCH: DCH carrier number of serving cell in decimal. When mobile is NOT on TCH: CH means carrier number of serving cell in decimal. If hopping is on, used channels are scrolled when display is updated ERR means that mobile is out of band. |
| ccc | GSS | FTD_SB_RX_LEVEL |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100 |
| ddd | GSS | FTD_SB_TX_LEVEL |  | R,I,O | Tx power level. If transmitter is on, symbol * is shown in front of the power level value. |
| e | GSS | FTD_SB_TIME_SLOT |  | R,I,O | Time Slot, range is 0 – 7 |
| ff | GSS | FTD_SB_TIMING_ADV |  | R,I,O | Timing advance, range is 0 - 63 |
| g | GSS | FTD_SB_RX_QUALITY |  | R,I,O | Rx quality (sub), range is 0 - 7 |
| mmmm | GSS | FTD_SB_RADIO_LINK_TIMEOUT |  | R,I,O | Radio Link Timeout value. If value is negative, 0 is shown. Maximum value is 64. When mobile is NOT on TCH then xx is show |
| nnn | GSS | FTD_SB_C1 |  | R,I,O | Value of the path loss criteria (C1). Range is -99 - 999. ERR means that the value is out of range, e.g. because it was not able to be calculated |
| ppp | GSS | FTD_SB_C2 |  | R,I,O | Value of the cell reselection criteria (C2). Range is -99 - 999. If phone is phase 1 then C1 value is shown. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| rr | GSS | FTD_SB_CURR_BAND |  | R,I,O | Currently used band.Values: 9 = GSM900, 18 = GSM1800, 19 = GSM1900 ….. extensible for future |
| oooo | GSS | FTD_SB_TYPE_OF_CURR_CH |  | R,I,O | Type of current channel THR0: TCH HR subchannel 0 THR1: TCH HR subchannel 1 AHS0: TCH AHS subchannel 0 AHS1: TCH AHS subchannel 1 TFR: TCH FR AFS: TCH AFS TEFR: TCH EFR F144: TCH FR data channel, speed 14.4 kbps F96: TCH FR data channel, speed 9.6 kbps F72: TCH FR data channel, speed 7.2 kbps F48 : TCH FR data channel, speed 4.8 kbps F24 : TCH FR data channel, speed 2.4 kbps H480 : TCH HR data channel, speed 4.8 kbps, subch 0 H481 : TCH HR data channel, speed 4.8 kbps, subch 1 H240 : TCH HR data channel, speed 2.4 kbps, subch 0 H241 : TCH HR data channel, speed 2.4 kbps, subch 1 FA: TCH FR signaling only (FACCH) channel FAH0 : TCH HR signaling only (FACCH) channel, subch 0 FAH1 : TCH HR signaling only (FACCH) channel, subch 1 PCCC, PBCC, PAGC : GPRS packet control channel PNDR: GPRS non-DRX mode (in PCCCH) PDTC: GPRS traffic channel NDRX: GPRS non-DRX mode (in CCCH) SDCC : SDCCH AGCH : AGCH CCCH : CCCH CBCH : CCCH and cell broadcast receiving on BCCH : BCCH SEAR : SEARCH NSPS : MS is in No Serv Power Save state |
| qq | GPDS | FTD_SB_GPDS_ATT_AND_PDP |  | R,I,O | G if MS is GSM-GPRS attached, GP if attached and PDP context created, U if UMTS-GPRS attached, UP if attached and PDP context created, otherwise empty. |
| x | GSM_DSP | FTD_SB_AMR_UL_MODE |  | R,I,O | Current absolute uplink mode on AMR channels, (0-7). 0=4.75, 1=5.15, 2=5.9, 3=6.7, 4=7.4, 5=7.95, 6=10.2, 7=12.2. |
| y | GSM_DSP | FTD_SB_AMR_DL_MODE |  | R,I,O | Current absolute downlink mode on AMR channels. See definition above. |

Display 01.02: More information of the serving cell (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Paging mode    aa       +

+ max RACH retr  b        +

+ Roaming ind    c        +

+ BSIC value     bdd      +

+ CC cause       ee       +

+ Rx quality     f        +

+ CRO / Hopping  ggg   hh +

+ PenT / HCh     iii   j  +

+ MAIO / HSN     mm    nn +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | GSS | FTD_SB_PAGING_MODE |  | R,I,O | Paging mode NO : normal paging EX : extended paging RO : paging reorganization SB : same as before |
| b | GSS | FTD_SB_MAX_RAND_ACC_TRA |  | R,I,O | Maximum number of Random Access retransmission |
| c | GSS | FTD_SB_ROAMING_IND |  | R,I,O | Roaming indicator, values are R or empty. |
| bdd | GSS | FTD_SB_BSIC |  | R,I,O | Letter B and BSIC value, range is 0 - 63. |
| ee | GSS | FTD_SB_LAST_CALL_REL_REAS |  | R,I,O | Reason of last call release. Cause from messages disconnect and release complete. |
| f | GSS | FTD_SB_ RX_QUALITY |  | R,I,O | RX quality (full), range is 0 – 7 |
| ggg | GSS | FTD_SB_CELL_RESEL_OFFSET |  | R,I,O | Cell reselect offset, range 0 - 126 dB. 0 - 63 * 2 dB. 'xxx' in active mode. |
| hh | GSS | FTD_SB_TEMP_OFFSET |  | R,I,O | Temporary offset, range 0 - 60 dB. 0 - 7 * 10 dB. 70 dB means infinite time. 'xx' in active mode. |
| iii | GSS | FTD_SB_PENALTY_TIME |  | R,I,O | Penalty time, range 0 - 620 s. 0 - 31 * 20 s. 'xxx' in active mode |
| j | GSS | FTD_SB_HOPP_CH |  | R,I,O | Hopping channel 0 Single RF channel 1 RF hopping channel |
| mm | GSS | FTD_SB_MOB_ALLOC_INDEX |  | R,I,O | Mobile allocation index offset, MAIO Range: 00 to 63 / xx when H=0 |
| nn | GSS | FTD_SB_HSN |  | R,I,O | Hopping sequence number, HSN Range: 00 to 63 / xx when H=0 |

Display 01.03: Information of the serving cell, 1st and 2nd neighbor (Version: 3.25, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+  Serving cell info      +

+ CH:    C1:    Rx:   C2: +

+ aaa    bbb    ccc   ddd +

+  1. Neighbor  info      +

+ eee    fff    ggg   hhh +

+  2. Neighbor  info      +

+ iii    jjj    kkk   lll +

+ 1. Neigbor ar mn        +

+ 2. Neigbor ar op        +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.25	Approved	Note about “ERR” text added to b, d, f, h, j and l fields / Inspection 11-Nov-02, SWAM ADB 02111944685

3.19	Approved	Sub block id for a-field changed / Inspection 30-Sep-02, SWAM ADB TS02093045811

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

1. row: serving cell information

2. row: 1. neighbor information

3. row: 2. neighbor information

4. row, mn: 1. neighbor information

4. row, op: 2. neighbor information

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | GSS | FTD_SB_BCCH_CARRIER_SERV |  | R,I,O | Carrier number of serving cell in decimal |
| bbb | GSS | FTD_SB_C1S |  | R,I,O | C1 value of serving cell, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| ccc | GSS | FTD_SB_RX_LEVEL |  | R,I,O | Rx level of serving cell in dBm, minus sign not shown if <=-100 |
| ddd | GSS | FTD_SB_C2S |  | R,I,O | C2 value of serving cell, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| eee | GSS | FTD_SB_CARRIER_1_NEIGH |  | R,I,O | Carrier number of 1. neighbor in decimal |
| fff | GSS | FTD_SB_C1_1_NEIGH |  | R,I,O | C1 of 1. neighbor value, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| ggg | GSS | FTD_SB_RX_LEVEL_1_NEIGH |  | R,I,O | Rx level of 1. neighbor in dBm, minus sign not shown if <=-100 |
| hhh | GSS | FTD_SB_C2_1_NEIGH |  | R,I,O | C2 of 1. neighbor value, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| iii | GSS | FTD_SB_CARRIER_2_NEIGH |  | R,I,O | Carrier number of 2. neighbor in decimal |
| jjj | GSS | FTD_SB_C1_2_NEIGH |  | R,I,O | C1 value of 2. neighbor, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| kkk | GSS | FTD_SB_RX_LEVEL_2_NEIGH |  | R,I,O | Rx level of 2. neighbor in dBm, minus sign not shown if <=-100 |
| lll | GSS | FTD_SB_C2_2_NEIGH |  | R,I,O | C2 value of 2. neighbor, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| m | GSS | FTD_SB_LOCATION_AREA_INFO_1 |  | R,I,O | F is shown if cell is in a forbidden location area, otherwise location is empty. |
| n | GSS | FTD_SB_CELL_PRIORITY_1 |  | R,I,O | B is Barred, N is normal priority and L is low priority, otherwise location is empty. |
| o | GSS | FTD_SB_LOCATION_AREA_INFO_2 |  | R,I,O | F is shown if cell is in a forbidden location area, otherwise location is empty. |
| p | GSS | FTD_SB_CELL_PRIORITY_2 |  | R,I,O | B is Barred, N is normal priority and L is low priority, otherwise location is empty. |

Display 01.04: Information of the 3rd, 4th and 5th neighbor (Version: 3.25, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+   3. Neighbor info      +

+ aaa    bbb    ccc   ddd +

+   4. Neighbor info      +

+ eee    fff    ggg   hhh +

+   5. Neighbor info      +

+ iii    jjj    kkk   lll +

+ 3. Neigbor ar mn        +

+ 4. Neigbor ar op        +

+ 5. Neigbor ar QR        +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.25	Approved	Note about “ERR” text added to b, d, f, h, j and l fields / Inspection 11-Nov-02, SWAM ADB 02111944685Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

1. row: 3. neighbor information

2. row: 4. neighbor information

3. row: 5. neighbor information

4. row, mn: 3. neighbor information

5. row, op: 4. neighbor information

6. row, qr: 5. neighbor information

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | GSS | FTD_SB_CARRIER_3_NEIGH |  | R,I,O | Carrier number of 3. neighbor in decimal |
| bbb | GSS | FTD_SB_C1_3_NEIGH |  | R,I,O | C1 value of 3. neighbor, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| ccc | GSS | FTD_SB_RX_LEVEL_3_NEIGH |  | R,I,O | Rx level of 3. neighbor in dBm, minus sign not shown if <=-100 |
| ddd | GSS | FTD_SB_C2_3_NEIGH |  | R,I,O | C2 value of 3. neighbor, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| eee | GSS | FTD_SB_CARRIER_4_NEIGH |  | R,I,O | Carrier number of 4. neighbor in decimal |
| fff | GSS | FTD_SB_C1_4_NEIGH |  | R,I,O | C1 value of 4. neighbor, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| ggg | GSS | FTD_SB_RX_LEVEL_4_NEIGH |  | R,I,O | Rx level of 4. neighbor in dBm, minus sign not shown if <=-100 |
| hhh | GSS | FTD_SB_C2_4_NEIGH |  | R,I,O | C2 value of 4. neighbor, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| iii | GSS | FTD_SB_CARRIER_5_NEIGH |  | R,I,O | Carrier number of 5. neighbor in decimal |
| jjj | GSS | FTD_SB_C1_5_NEIGH |  | R,I,O | C1 value of 5. neighbor, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| kkk | GSS | FTD_SB_RX_LEVEL_5_NEIGH |  | R,I,O | Rx level of 5. neighbor in dBm, minus sign not shown if <=-100 |
| lll | GSS | FTD_SB_C2_5_NEIGH |  | R,I,O | C2 value of 5. neighbor, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| m | GSS | FTD_SB_LOCATION_AREA_INFO_3 |  | R,I,O | F is shown if cell is in a forbidden location area, otherwise location is empty. |
| n | GSS | FTD_SB_CELL_PRIORITY_3 |  | R,I,O | B is Barred, N is normal priority and L is low priority, otherwise location is empty. |
| o | GSS | FTD_SB_LOCATION_AREA_INFO_4 |  | R,I,O | F is shown if cell is in a forbidden location area, otherwise location is empty. |
| p | GSS | FTD_SB_CELL_PRIORITY_4 |  | R,I,O | B is Barred, N is normal priority and L is low priority, otherwise location is empty. |
| Q | GSS | FTD_SB_LOCATION_AREA_INFO_5 |  | R,I,O | F is shown if cell is in a forbidden location area, otherwise location is empty. |
| R | GSS | FTD_SB_CELL_PRIORITY_5 |  | R,I,O | B is Barred, N is normal priority and L is low priority, otherwise location is empty. |

### Display 01.05: Information of the 6th, 7th and 8th neighbor (Version: 3.25, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+   6. Neighbor info      +

+ aaa    bbb    ccc   ddd +

+   7. Neighbor info      +

+ eee    fff    ggg   hhh +

+   8. Neighbor info      +

+ iii    jjj    kkk   lll +

+ 6. Neigbor ar mn        +

+ 7. Neigbor ar op        +

+ 8. Neigbor ar qr        +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.25	Approved	Note about “ERR” text added to b, d, f, h, j and l fields / Inspection 11-Nov-02, SWAM ADB 02111944685

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

1. row: 6. neighbor information

2. row: 7. neighbor information

3. row: 8. neighbor information

4. row, ef: 6. neighbor information

5. row, gh: 7. neighbor information

6. row, ij: 8. neighbor information

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | GSS | FTD_SB_CARRIER_6_NEIGH |  | R,I,O | Carrier number of 6. neighbor in decimal |
| bbb | GSS | FTD_SB_C1_6_NEIGH |  | R,I,O | C1 value of 6. neighbor, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| ccc | GSS | FTD_SB_RX_LEVEL_6_NEIGH |  | R,I,O | Rx level of 6. neighbor in dBm, minus sign not shown if <=-100 |
| ddd | GSS | FTD_SB_C2_6_NEIGH |  | R,I,O | C2 value of 6. neighbor, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| eee | GSS | FTD_SB_CARRIER_7_NEIGH |  | R,I,O | Carrier number of 7. neighbor in decimal |
| fff | GSS | FTD_SB_C1_7_NEIGH |  | R,I,O | C1 value of 7. neighbor, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| ggg | GSS | FTD_SB_RX_LEVEL_7_NEIGH |  | R,I,O | Rx level of 7. neighbor in dBm, minus sign not shown if <=-100 |
| hhh | GSS | FTD_SB_C2_7_NEIGH |  | R,I,O | C2 value of 7. neighbor, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| iii | GSS | FTD_SB_CARRIER_8_NEIGH |  | R,I,O | Carrier number of 8. neighbor in decimal |
| jjj | GSS | FTD_SB_C1_8_NEIGH |  | R,I,O | C1 value of 8. neighbor, range is -99 - 999, displayed only in idle mode. ERR means that the value is out of range, e.g. because it was not able to be calculated. Instead of C1 value, letter 'B' and BSIC value will be displayed in active mode. |
| kkk | GSS | FTD_SB_RX_LEVEL_8_NEIGH |  | R,I,O | Rx level of 8. neighbor in dBm, minus sign not shown if <=-100 |
| lll | GSS | FTD_SB_C2_8_NEIGH |  | R,I,O | C2 value of 8. neighbor, range is -99 – 999. ERR means that the value is out of range, e.g. because it was not able to be calculated. |
| m | GSS | FTD_SB_LOCATION_AREA_INFO_6 |  | R,I,O | F is shown if cell is in a forbidden location area, otherwise location is empty. |
| n | GSS | FTD_SB_CELL_PRIORITY_6 |  | R,I,O | B is Barred, N is normal priority and L is low priority, otherwise location is empty. |
| o | GSS | FTD_SB_LOCATION_AREA_INFO_7 |  | R,I,O | F is shown if cell is in a forbidden location area, otherwise location is empty. |
| p | GSS | FTD_SB_CELL_PRIORITY_7 |  | R,I,O | B is Barred, N is normal priority and L is low priority, otherwise location is empty. |
| q | GSS | FTD_SB_LOCATION_AREA_INFO_8 |  | R,I,O | F is shown if cell is in a forbidden location area, otherwise location is empty. |
| r | GSS | FTD_SB_CELL_PRIORITY_8 |  | R,I,O | B is Barred, N is normal priority and L is low priority, otherwise location is empty. |

### Display 01.06: Network selection display (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

This display shows the last registered network country code and network code as well as the codes for four forbidden networks and the first 3 preferred networks.

Data Display:

+++++++++++++++++++++++++++

+ Last reg  1st forbidd   +

+   aaaaaa      eeeeee    +

+ 1st pref  2nd forbidd   +

+   bbbbbb      ffffff    +

+ 2nd pref  3rd forbidd   +

+   cccccc      gggggg    +

+ 3rd pref  4th forbidd   +

+   dddddd      hhhhhh    +

+                         +

+++++++++++++++++++++++++++

1. row: last registered network -  1st forbidden network

2. row: 1st preferred network   -  2nd forbidden network

3. row: 2nd preferred network   -  3rd forbidden network

4. row: 3rd preferred network   -  4th forbidden network

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | GSS | FTD_SB_LAST_REG |  | R,I,O | Last registred network |
| b(6) | GSS | FTD_SB_1_PREF |  | R,I,O | 1. preferred network |
| c(6) | GSS | FTD_SB_2_PREF |  | R,I,O | 2. preferred network |
| d(6) | GSS | FTD_SB_3_PREF |  | R,I,O | 3. preferred network |
| e(6) | GSS | FTD_SB_1st_FORBIDDEN_NW |  | R,I,O | 1. forbidden PLMN in SIM |
| f(6) | GSS | FTD_SB_2nd_FORBIDDEN_NW |  | R,I,O | 2. forbidden PLMN in SIM |
| g(6) | GSS | FTD_SB_3rd_FORBIDDEN_NW |  | R,I,O | 3. forbidden PLMN in SIM |
| h(6) | GSS | FTD_SB_4th_FORBIDDEN_NW |  | R,I,O | 4. forbidden PLMN in SIM |

### Display 01.07: System information bits for serving cell (Version: 3.19, Status: Approved).

Data Display:

+++++++++++++++++++++++++++

+ Emergency cal  a        +

+ Attach detach  b        +

+ Half rate CH   c        +

+ C2 broadcast   d        +

+ SI 7&8 broad   e        +

+ Cell broad     f        +

+ Call Re-est    g        +

+ ECSC / 2-Ter   h     i  +

+ Mband / GPRS   j     k  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Formats added, k-field completed, approved / Inspection 30-Sep-02, SWAM ADB TS02093045811

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | GSS | FTD_SB_SYSTEM_INFO_BITS1 | S | R,I,O | 1 is shown if emergency calls are supported, else 0 |
| b | GSS | FTD_SB_SYSTEM_INFO_BITS1 | S | R,I,O | 1 is shown if attach-detach-procedure is allowed, else 0 |
| c | GSS | FTD_SB_SYSTEM_INFO_BITS1 | S | R,I,O | 1 is shown if half rate channels are supported, else 0 |
| d | GSS | FTD_SB_SYSTEM_INFO_BITS1 | S | R,I,O | 1 is shown if C2 values are broadcast, else 0 |
| e | GSS | FTD_SB_SYSTEM_INFO_BITS1 | S | R,I,O | 1 is shown if system information 7 and 8 are broadcast, else 0 |
| f | GSS | FTD_SB_SYSTEM_INFO_BITS1 | S | R,I,O | 1 is shown if cell broadcast is supported, else 0 |
| g | GSS | FTD_SB_SYSTEM_INFO_BITS1 | S | R,I,O | 1 is shown if re-establishment is supported, else 0 |
| h | GSS | FTD_SB_SYSTEM_INFO_BITS2 | S | R,I,O | In idle mode 1 is shown if Early Classmark (ECSC) sending is supported, else 0. In dedicated mode (conversation) X is shown. |
| i | GSS | FTD_SB_SYSTEM_INFO_BITS2 | S | R,I,O | In idle mode 1 is shown if 2-Ter messages are supported, else 0. In dedicated mode (conversation) X is shown. |
| j | GSS | FTD_SB_SYSTEM_INFO_BITS2 | S | R,I,O | MultiBand reporting decimal value (0,1,2,3) is shown if supported. This is shown both in idle and dedicated mode. |
| k | GSS | FTD_SB_GPRS_SUPPORT | B:D | R,I,O | 1 is shown if GPRS is supported, else 0 |

[/FTD_FIELDS]

The following is picked from Phase2+ ETSI GSM 05.08 version 5.4.0, Section 8.4.3 ""Additional cell reporting requirements for multiband MS"".

For a multi band MS the number of cells, for each frequency band supported, which shall be included in the measurement report is indicated by the parameter, MULTIBAND_REPORTING. The meaning of different values of the parameter is specified as follows:

| Value | Meaning |
| --- | --- |
| 0 (00) | Normal reporting of the six strongest cells, with known and allowed NCC part of BSIC, irrespective of the band used. |
| 1 (01) | The MS shall report the strongest cell, with known and allowed NCC part of BSIC, in each of the frequency bands in the BA list, excluding the frequency band of the serving cell. The remaining positions in the measurement report shall be used for reporting of cells in the band of the serving cell. If there are still remaining positions, these shall be used to report the next strongest identified cells in the other bands irrespective of the band used. |
| 2 (10) | The MS shall report the two strongest cells, with known and allowed NCC part of BSIC, in each of the frequency bands in the BA list, excluding the frequency band of the serving cell. The remaining positions in the measurement report shall be used for reporting of cells in the band of the serving cell. If there are still remaining positions, these shall be used to report the next strongest identified cells in the other bands irrespective of the band used. |
| 3 (11) | The MS shall report the three strongest cells, with known and allowed NCC part of BSIC, in each of the frequency bands in the BA list, excluding the frequency band of the serving cell. The remaining positions in the measurement report shall be used for reporting of cells in the band of the serving cell. If there are still remaining positions, these shall be used to report the next strongest identified cells in the other bands irrespective of the band used. |

Display 01.08: Paging repeat period, TMSI, periodic location update, AFC and AGC (Version: 3.19, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ TMSI value     aaaaaaaa +

+ Curr T3212     bbb      +

+ Initial T3212  ccc      +

+ Paging repeat  d        +

+ Downlink fail  ee       +

+ TCH/SDCCH Ga   ff       +

+ VCTCXO AFC     ggggg    +

+ Channel num    hhh      +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Many changes to server and sub block id columns, formats added, approved / Inspection 30-Sep-02, SWAM ADB TS02093045811

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | GSS | FTD_SB_TMSI | S | R,I,O | TMSI value in hex format |
| b(3) | GSS | FTD_SB_T3212 | B:D | R,I,O | Current value of T3212 counter (range is 000 - 'ccc', where 1 means 6 min time. So, if this value is 2 less than 'ccc' then next periodic location updating will be made within 2 * 6 min = 12 minutes |
| c(3) | GSS | FTD_SB_T3212_TIMEOUT | B:D | R,I,O | Timeout value of T3212 counter (range is 000 - 240, where 1 means 6 min time between location updates and 240 means 240 * 6 min = 24 h between location updates. 000 means that periodic location update is not in use.) This value is received from the network. |
| d | GSS | FTD_SB_PRP | B:D | R,I,O | Value of paging repeat period (range is 2 - 9, when paging is in every second multiframe, mobile takes more current than if it were in every 9th multiframe) |
| e(2) | GSS | FTD_SB_DOWNLINK_SIGN_FAIL | B:D | R,I,O | Downlink signaling failure value. If value is negative, 0 is shown. Maximum value is 45. When mobile is on TCH then xx is shown |
| f(2) | GSS | FTD_SB_GAIN_ON_TCH | B:D | R,I,O | Gain value on TCH/SDCCH, range is 0 - 93 |
| g(5) | GSS | FTD_SB_VCTCXO_AFC_DAC | S | R,I,O | VCTCXO AFC DAC control, range is -1024 - 1023 |
| h(3) | GSS | FTD_SB_BCCH_CARRIER_SERV | S | R,I,O | Serving cell channel number |

### Display 01.09: Network parameters (Version: 4.0, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+   Network parameters    +

+ Country code   aaa      +

+ Network code   bbb      +

+ Location area  ccccc    +

+                         +

+ Serving cell            +

+ Channel num    ddd      +

+ Cell                    +

+ identifier   eeeeeeeeee +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.35/4.0	Approved	Approved / Inspection 16-May-03, Email comment from Tapio Askeli

3.34	Proposal	E-field changed to double word / PCP Errors EV02121832117

3.19	Approved	Sub block id and size for d-field changed / Inspection 30-Sep-02, SWAM ADB TS02093045811

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | GSS | FTD_SB_REG_MCC |  | R,I,O | MCC value in decimal (MCC=Mobile Country Code) |
| b(3) | GSS | FTD_SB_REG_MNC |  | R,I,O | MNC value in decimal (MNC=Mobile Network Code). Three digits are shown only in DCS1900. In other systems only two digits are shown. |
| c(5) | GSS | FTD_SB_LAC |  | R,I,O | LAC value in decimal (LAC=Location Area Code) |
| d(3) | GSS | FTD_SB_BCCH_CARRIER_SERV |  | R,I,O | Serving cell channel number |
| e(10) | GSS | FTD_SB_CELL_ID | DW:D | R,I,O | Cell Identifier in decimal format |

[/FTD_FIELDS]

LAC and CID may be in hex format in some projects. Or even both decimal and hexadecimal formats may be shown on the same line.

Display 01.10: Ciphering, hopping DTX status and IMSI (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+  Updated only on TCH    +

+                         +

+ Cipherin val   aaa      +

+                         +

+ Hopping val    bbb      +

+                         +

+ DTX value      ccc      +

+                         +

+ IMSI attach    ddd      +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | GSS | FTD_SB_CIPH |  | R,I,O | Ciphering value, OFF/A51/A52 |
| bbb | GSS | FTD_SB_HOPP_STATUS |  | R,I,O | Hopping value, ON/OFF |
| ccc | GSS | FTD_SB_DTX_VALUE |  | R,I,O | DTX value ON/OFF |
| ddd | GSS | FTD_SB_IMSI_ATT |  | R,I,O | IMSI attach ON : IMSI attach on OFF : IMSI attach off |

[/FTD_FIELDS]

These values are updated only on the TCH.

Display 01.11: Uplink DTX switching display (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

With this display it is possible to change MS to use DTX or not, if BS allows MS to decide it. This display must be activated from MENU to change DTX state. When MENU is not active and the user is scrolling field test displays with NEXT and PREVIOUS, the DTX state will not change.

Executive Display:

***************************

*    Execute    display   *

* With this display it is *

* possible to change MS   *

* to use DTX or not. Use  *

* options to change       *

*                         *

* DTX mode      aaaaaaaaaa*

* Default DTX   bbb       *

* DTX from BS   ccc       *

***************************

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | GSS | FTD_SB_DTX_STATUS_CHANGE | N/A | R,I,O | Toggle DTX status. |
| a(10) | GSS | FTD_SB_DTX_STATUS |  | R,I,O | Status of switched mode. DTX:ON : MS uses DTX DTX:OFF : MS does not use DTX DTX:DEF : MS use default state of DTX. Defined in MS_PAR.H NOTALLOWED: BS does not allow MS to decide if it uses DTX or not. |
| bbb | GSS | FTD_SB_DTX_DEF |  | R,I,O | Default state of DTX. Defined in MS_PAR.H. The value is either ON or OFF |
| ccc | GSS | FTD_SB_DTX_FROM_BS |  | R,I,O | Is DTX value from BS MAY : BS allows MS to decide if it uses dtx or not on uplink. USE : BS controls MS to use dtx (on uplink) NOT : BS controls MS not to use dtx (on uplink) |

### Display 01.12: Storing and removing BTS_TEST carrier (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Display corrected / Inspection 14-Jun-02, SWAM ADB SJ02061836705

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

This display is used to change BTS_TEST carrier on permanent memory. If BTS_TEST carrier is stored on permanent memory each time the mobile sends a search list it uses only the carrier number stored on Mobile Station permanent memory. Also the neighbor information from system information messages is ignored. If the BTS_TEST carrier is not stored, then the mobile behaves normally (i.e. does the neighbor measurements according the GSM specifications).

Executive Displays:

***************************

*     Execute   display   *

* With this display it is *

* possible  to   change   *

* BTS_TEST carrier.  Use  *

* options to change       *

*                         *

*   BTS TEST locked on    *

*     channel   xxxx      *

*                         *

***************************

Mobile is searching only one frequency. Neighbor measurements are not done.

***************************

*     Execute   display   *

* With this display it is *

* possible  to  change    *

* BTS_TEST carrier.  Use  *

* options to change       *

*                         *

*     BTS TEST  OFF       *

*                         *

*                         *

***************************

Mobile is behaving normally. Neighbor measurements are done.

To store BTS test carrier, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 0112 (TEST GROUP;TEST DISPLAY) to the query prompt.

- Test input promt will be shown on the display, enter the channel number in display.

- Confirm with the Ok button.

- Switch power off and on.

- If activation succeeded, there is text "BTS TEST LOCKED, CH:…" in display 12 of group 1.

To deactivate BTS tests, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 0112 (TEST GROUP;TEST DISPLAY) to the query prompt.

- Test input promt will be shown on the display, enter the 3333 as a channel number in display

- Confirm with the Ok button.

- Switch power off and on.

- If deactivation succeeded, there is text "BTS TEST OFF" in display 12 of group 1.

The legal carrier numbers:

| Band | Carrier Numbers |
| --- | --- |
| GSM450 | 259-293 |
| GSM480 | 306-340 |
| GSM 850 | 128-251 |
| GSM900 | 1-124 |
| GSM1800 | 512-885 |
| GSM1900 | 512-810 |
| E-GSM | 0, 975 – 1023 |
| R-GSM | 955-974 |

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| xxxx | NETWORK | FTD_SB_BTS_STATUS |  | R,I,O | BTS test status |
| INPUT | NETWORK | FTD_SB_BTS_TEST | DW:D | R,I,O | BTS test |

### Display 01.13: Toggle Cell Barred Status (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

Executive Displays:

***************************

*     Execute   display   *

* With this display it is *

* possible  to  change    *

* Call bearer status.Use  *

* options to change       *

*                         *

*       CALL BEARER       *

*        xxxxxxxx         *

*                         *

***************************

This test is meant to be used when some cells are tested prior taking them into commercial use. By setting the barring on in the base station normal GSM phones will not try to register these barred cells. By selecting cell barring reversed, the MS will only use the cells to be tested. However, if at the same time it is wanted that MS will be capable to use normal network cell barring ignored can be set. This display will show the cell barring mode.

NOTE! If a cell has been selected before barring state is changed the selected cell may have different barring state than what the display shows. After reselection the cell barring state is working for sure.

To change Cell Barring status, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in executive mode by entering 0113 (TEST GROUP;TEST DISPLAY) to the query prompt.

- Confirm with the Ok button.

- If activation succeeded, there is text "CELL BARR …" in display 13 of group 1.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| Cell barr exe | GSS | FTD_SB_CELL_BARR_IND_STATUS_CHANGE |  | R,I,O | Toggle Cell Barring status |
| Cell barr stat | GSS | FTD_SB_CELL_BARR_STATUS_IND |  | R,I,O | Cell Barring status |

### Display 01.14: Modify "last used band" (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

This display is applicable only in phones having both European and American frequencies.

With this display it is possible to manipulate the "last used band". In multi-band products this paramater is used at startup to decide which band shall be searched first. In most cases the last used band is the most promising band to find networks in. Nevertheless, for testing purposes it is helpful to have influence on this decision.

If at startup no "last used  band" is found in the memory, the "Default" setting (Europe or America) will be effective.

Executive Display:

***************************

*     Execute   display   *

* With this display it is *

* possible to set band    *

* which will be searched  *

* first after startup     *

*                         *

* Use options to set.     *

*                         *

*                         *

***************************

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 01.14 (TEST GROUP;TEST DISPLAY) to the query prompt.

- Test input prompt will activate, enter the desired band code (see the table below) in display.

- Confirm with the Ok button.

- REMOVE BATTERY or POWER SUPPLY

NOTE: In normal operation the last used band is saved to the memory at power-down. If this value is written via Test Display 01.14, the phone MUST NOT be powerd off normally afterwards, because then the user-set value will be overwritten. The power supply has be interrupted instead.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GSS | FTD_SB_LAST_USED_BAND_READ | B:D | R,I,O | Last used band, see the table below |
| INPUT | GSS | FTD_SB_LAST_USED_BAND_SET | DW:D | R,I,O | Write value to memory |

[/FTD_FIELDS]

Band codes:

| Band Code | Meaning |
| --- | --- |
| 9 | GSM900 |
| 18 | GSM1800 |
| 19 | GSM1900 |
| Other | Delete stored value |

- Group 02: GSM Registration and Mobility Displays

Display 02.01: Reset Registration and Mobility counters (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

With this display all timers listed on the displays in this group can be reset.

Executive Display:

***************************

*     Execute   display   *

* With this display it is *

* possible  to  reset     *

* registration / mobility *

* counters. Displays      *

* 02/02 – 02/07           *

* Use options to change.  *

*                         *

*                         *

***************************

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | GSS | FTD_SB_RESET_HANDOVERS | N/A | R | Reset handover counters. |

Display 02.02: Neighbor Measurement Counter Display (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Neighbor counter display+

+ PSW meas                +

+ attempts       aaaa     +

+ Sync meas               +

+ attempts       bbbb     +

+ BCCH meas               +

+ attempts       cccc     +

+ BCCH meas               +

+ ext att        dddd     +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSS | FTD_SB_NEIGH_PSW_ATT |  | R | Counter for neighbor PSW measurement attempts |
| bbbb | GSS | FTD_SB_NEIGH_SYNC_ATT |  | R | Counter for neighbor synchronization measurement attempts |
| cccc | GSS | FTD_SB_NEIGH_BCCH_ATT |  | R | Counter for neighbor BCCH measurement attempts |
| dddd | GSS | FTD_SB_BCCH_EXT_ATT |  | R | Counter for neighbor BCCH Ext measurement attempts |

[/FTD_FIELDS]

Counter values are shown in hexadecimal form. On poweroff the values of the counter displays are stored onto the permanent memory, where they will be read during power on. To initialize the counters to zero, select Display 02.01: Reset Registration and Mobility counters. Counters are automatically reset to zero when they exceed their maximum value.

Display 02.03: Search and reselection counter display (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Search/reselect counters+

+ GSM 900  PSW   aaaaa    +

+ GSM 1800 PSW   bbbbb    +

+ GSM 900  sync  ccccc    +

+ GSM 1800 sync  ddddd    +

+ 900-900  res   eeeee    +

+ 1800-1800 res  fffff    +

+ 900-1800 res   ggggg    +

+ 1800-900 res   hhhhh    +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaaa | GSS | FTD_SB_NO_PSW_FOUND_GSM |  | R | GSM900 counter for MDI_NO_PSW_FOUND message received from DSP in decimal form (max 99999). |
| bbbbb | GSS | FTD_SB_NO_PSW_FOUND_DCS |  | R | GSM1800 counter for MDI_NO_PSW_FOUND message received from DSP in decimal form (max 99999). |
| ccccc | GSS | FTD_SB_SYNC_GSM |  | R | GSM900 counter for synchronization measurement attempts in decimal form. If counter value is over 99999 then five x are shown. |
| ddddd | GSS | FTD_SB_SYNC_DCS |  | R | GSM1800 counter for synchronization measurement attempts in decimal form. If counter value is over 99999 then five x are shown. |
| eeeee | GSS | FTD_SB_RESEL_GSM_GSM |  | R | Counter for GSM900->GSM900 cell reselections in decimal form (max 99999). |
| fffff | GSS | FTD_SB_RESEL_DCS_DCS |  | R | Counter for GSM1800->GSM1800 cell reselections in decimal form (max 99999). |
| ggggg | GSS | FTD_SB_RESEL_GSM_DCS |  | R | Counter for GSM900->GSM1800 cell reselections in decimal form (max 99999). |
| hhhhh | GSS | FTD_SB_RESEL_DCS_GSM |  | R | Counter for GSM1800->GSM900 cell reselections in decimal form (max 99999). |

[/FTD_FIELDS]

On power off the values of the counter displays are stored onto the permanent memory, where they will be read during power on. To initialize the counters to zero, select Display 02.01: Reset Registration and Mobility counters. Counters are automatically reset to zero when they exceed their maximum value.

Display 02.04: Location update attempts counters (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Location update counters+

+ Fail reasons:           +

+ Norm LU        Aa       +

+ Periodic&IMSI  dd       +

+ Counters:               +

+ Norm LU total  bbb      +

+ Norm LU ok     ccc      +

+ Periodic tot   eee      +

+ periodic ok    fff      +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| Aa | GSS | FTD_SB_LAST_NORMAL_LOC_UP_REASON |  | R | Reason of last normal location update failure |
| bbb | GSS | FTD_SB_NORMAL_LOC_UP_COUNT |  | R | Count of normal location update attempts |
| ccc | GSS | FTD_SB_SUCC_LOC_UP_COUNT |  | R | Count of succeeded normal location updates |
| dd | GSS | FTD_SB_LAST_IMSI_ATT_FAIL_REASON |  | R | Reason of last periodic or IMSI attach location update failure |
| eee | GSS | FTD_SB_LOC_UP_COUNT_IMSI |  | R | Count of all periodic and IMSI attach location update attempts |
| fff | GSS | FTD_SB_SUCC_LOC_UP_COUNT_IMSI |  | R | Count of succeeded periodic and IMSI attach location updates |

[/FTD_FIELDS]

On poweroff the values of the counter displays are stored onto the permanent memory, where they will be read during power on. To initialize the counters to zero, select Display 02.01: Reset Registration and Mobility counters. Counters are automatically reset to zero when they exceed their maximum value.

Display 02.05: Handover display, INTER CELL (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+  INTER CELL   Handovers +

+ 900->900   OK  aaaa     +

+ 1800->1800 OK  bbbb     +

+ 900->1800  OK  cccc     +

+ 1800->900  OK  dddd     +

+ 9-9/18-18FAIL  eee fff  +

+ 9-18/18-9FAIL  ggg  hhh +

+ 9-9/18-18prev  iii  jjj +

+ 9-18/18-9prev  kkk  lll +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSS | FTD_SB_HO_GSM_GSM |  | R | Counter of successful handovers (max 9999) from GSM900 to GSM900 |
| bbbb | GSS | FTD_SB_HO_DCS_DCS |  | R | Counter of successful handovers (max 9999) from GSM1800 to GSM1800 |
| cccc | GSS | FTD_SB_HO_GSM_DCS |  | R | Counter of successful handovers (max 9999) from GSM900 to GSM1800 |
| dddd | GSS | FTD_SB_HO_DCS_GSM |  | R | Counter of successful handovers (max 9999) from GSM1800 to GSM900 |
| eee | GSS | FTD_SB_FAILED_HO_GSM_GSM |  | R | Counter for failed handovers (max 999) from GSM900 to GSM900 |
| fff | GSS | FTD_SB_FAILED_HO_DCS_DCS |  | R | Counter for failed handovers (max 999) from GSM1800 to GSM1800 |
| ggg | GSS | FTD_SB_FAILE_HO_GSM_DCS |  | R | Counter for failed handovers (max 999) from GSM900 to GSM1800 |
| hhh | GSS | FTD_SB_FAILED_HO_DCS_GSM |  | R | Counter for failed handovers (max 999) from GSM1800 to GSM900 |
| iii | GSS | FTD_SB_BACK_TO_PREV_GSM_GSM |  | R | Counter of successful back to previous channel attempts (max 999) from GSM900 to GSM900 |
| jjj | GSS | FTD_SB_BACK_TO_PREV_DCS_DCS |  | R | Counter of successful back to previous channel attempts (max 999) from GSM1800 to GSM1800 |
| kkk | GSS | FTD_SB_BACK_TO_PREV_GSM_DCS |  | R | Counter of successful back to previous channel attempts (max 999)from GSM900 to GSM1800 |
| lll | GSS | FTD_SB_BACK_TO_PREV_DCS_GSM |  | R | Counter of successful back to previous channel attempts (max 999) from GSM1800 to GSM900 |

[/FTD_FIELDS]

Counters will stop when they reach their maximum. To initialize the counters to zero Display 02.01: Reset Registration and Mobility counters.

Display 02.06: Handover display,  INTRA CELL (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ INTRA CELL   Handovers  +

+ 900->900   OK  aaaa     +

+ 1800->1800 OK  bbbb     +

+ 900->1800 OK   cccc     +

+ 1800->900 OK   dddd     +

+ 9-9/18-18FAIL  eee  fff +

+ 9-18/18-9FAIL  ggg  hhh +

+ 9-9/18-18prev  iii  jjj +

+ 9-18/18-9prev  kkk  lll +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSS | FTD_SB_INTRA_HO_GSM_GSM |  | R | Counter of successful INTRA CELL handovers (max 9999) from GSM900 to GSM900 |
| bbbb | GSS | FTD_SB_INTRA_HO_DCS_DCS |  | R | Counter of successful INTRA CELL handovers (max 9999) from GSM1800 to GSM1800 |
| cccc | GSS | FTD_SB_INTRA_HO_GSM_DCS |  | R | Counter of successful INTRA CELL handovers (max 9999) from GSM900 to GSM1800 |
| dddd | GSS | FTD_SB_INTRA_HO_DCS_GSM |  | R | Counter of successful INTRA CELL handovers (max 9999) from GSM1800 to GSM900 |
| eee | GSS | FTD_SB_FAILED_INTRA_HO_GSM_GSM |  | R | Counter of failed INTRA CELL handovers (max 999) from GSM900 to GSM900 |
| fff | GSS | FTD_SB_FAILED_INTRA_HO_DCS_DCS |  | R | Counter of failed INTRA CELL handovers (max 999) from GSM1800 to GSM1800 |
| ggg | GSS | FTD_SB_FAILED_INTRA_HO_GSM_DCS |  | R | Counter of failed INTRA CELL handovers (max 999) from GSM900 to GSM1800 |
| hhh | GSS | FTD_SB_FAILED_INTRA_HO_DCS_GSM |  | R | Counter of failed INTRA CELL handovers (max 999) from GSM1800 to GSM900 |
| iii | GSS | FTD_SB_INTRA_BACK_TO_PREV_GSM_GSM |  | R | Counter of successful back to previous normal INTRA CELL channel attempts (max 999) from GSM900 to GSM900 |
| jjj | GSS | FTD_SB_INTRA_BACK_TO_PREV_DCS_DCS |  | R | Counter of successful back to previous normal INTRA CELL channel attempts (max 999) from GSM1800 to GSM1800 |
| kkk | GSS | FTD_SB_INTRA_BACK_TO_PREV_GSM_DCS |  | R | Counter of successful back to previous normal INTRA CELL channel attempts (max 999) from GSM900 to GSM1800 |
| lll | GSS | FTD_SB_INTRA_BACK_TO_PREV_DCS_GSM |  | R | Counter of successful back to previous normal INTRA CELL channel attempts (max 999) from GSM1800 to GSM900 |

[/FTD_FIELDS]

Counters will stop when they reach their maximum. To initialize the counters to zero, select Display 02.01: Reset Registration and Mobility counters.

Display 02.07: L2 display (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+    L2 retransmission    +

+ T200 MS 900    aaaa     +

+                         +

+ T200 BS 900    bbbb     +

+                         +

+ T200 MS 1800   cccc     +

+                         +

+ T200 BS 1800   dddd     +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSS | FTD_SB_T200_EXP_MS_GSM |  | R | GSM900: Counts how many times T200 in MS has expired and therefore L2 transmission has been repeated. |
| bbbb | GSS | FTD_SB_T200_EXP_BS_GSM |  | R | GSM900: Counts how many times T200 in BS (network) has expired and therefore L2 transmission has been repeated |
| cccc | GSS | FTD_SB_T200_EXP_MS_DCS |  | R | GSM1800: Counts how many times T200 in MS has expired and therefore L2 transmission has been repeated (for dualband phones). |
| dddd | GSS | FTD_SB_T200_EXP_BS_DCS |  | R | GSM1800: Counts how many times T200 in BS (network) has expired and therefore L2 transmission has been repeated (for dualband phones) |

[/FTD_FIELDS]

Counters will stop when they reach their maximum. To initialize the counters to zero Display 02.01: Reset Registration and Mobility counters.

- Group 03: GSM memory handling

Display 03.01: SIM information 1 (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+    SIM information 1    +

+                         +

+ SIM voltage   aaaaa     +

+ SIM baudrate  bbb       +

+ Clk stop allo c         +

+ Clk stop cond dddddddd  +

+ Pin1/Pin2 att f      g  +

+ Puk1/Puk2 att hh     ii +

+ ATR retrans   j         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | SIM | FTD_SB_SIM_VOL_SEL | S | R | Sim voltage selection type (3, 1V8 or 1V8/3) |
| b(3) | SIM | FTD_SB_SIM_BAUDRATE | W:D | R | Sim baudrate (372, 64, 32 or 0) |
| c | SIM | FTD_SB_CLOCK_STOP_ALL | S | R | Clock stop allowed, Y or N |
| d(8) | SIM | FTD_SB_CLOCK_STOP_COND | S | R | Clock stop condition, High or low (preferred) |
| f | SIM | FTD_SB_PIN1_ATT | B:D | R | Pin1 attempts left (0,1,2,3) |
| g | SIM | FTD_SB_PIN2_ATT | B:D | R | Pin2 attempts left (0,1,2,3) |
| hh | SIM | FTD_SB_PUK1_ATT | B:D | R | Puk1 attempts left (0-10) |
| ii | SIM | FTD_SB_PUK2_ATT | B:D | R | Puk2 attempts left (0-10) |
| j | SIM | FTD_SB_ATR_RETRANS | B:D | R | ATR retransmission counter (0-9) |

Display 03.02: SIM information 2 (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+    SIM information 2    +

+ Service table           +

+ from  6F38    aaaaaaaaaa+

+ Customer serv           +

+ profile 6F15  bbbbbbbbbb+

+ Customer serv           +

+ profile 6F98  cccccccccc+

+ ATR historic            +

+ bytes	      dddddddddd+

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(10) | SIM | FTD_SB_SIM_SERV_TAB |  | R | The service table form the sim card file 6F38. The readout of the service table is compressed so that each service is reprecented with one bit. 1 for Allocated Activated otherwise 0. |
| b(10) | SIM | FTD_SB_SIM_CSP1 |  | R | The Customer service profile from file 6F15 on binary form. |
| c(10) | SIM | FTD_SB_SIM_CSP2 |  | R | The Customer service profile from file 6F98 on binary form |
| d(10) | SIM | FTD_SB_SIM_HIST_DATA |  | R | Max. first 5 historical bytes from the ATR on binary form. |

- Group 04: GSM Test Timers, SMS and Call Counter Displays

Display 04.01: Reset test timers to zero and restart them (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

With this display all timers listed on the Display 04.02. can be reset.

Executive Display:

***************************

*     Execute   display   *

* With this display it is *

* possible  to  reset     *

* test timers to zero.    *

* Display 04/03.          *

* Use options to change.  *

*                         *

*                         *

*                         *

***************************

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | GSS | FTD_SB_RESET_TEST_TIMERS | N/A | R | Reset and restart Test timers |

Display 04.02: Enable or disable test timers (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

This display will only use to start or stop the timers. When looking the status of timers user should use the Display 04.03: Test timer display (Version: 1, Status: Approved)

Executive Display:

***************************

*     Execute   display   *

* With this display it is *

* possible  to  start and *

* stop test timers        *

* Display 04/03.          *

* Use options to change.  *

*                         *

*                         *

*          XXX            *

*                         *

***************************

XXX ON or OFF

On power off the values of the timer displays are stored onto the permanent memory, where they will be read during power on. To initialize the counters to zero, use Display 04.01: Reset test timers to zero and restart them.

Also the current state of timer disabling/enabling is stored onto the permanent memory.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | GSS | FTD_SB_ENAB_OR_DISAB_TEST_TIMERS | N/A | R | Enable / DisableTimers |
| b(6) | GSS | FTD_SB_TEST_TIMER_STATE |  | R | State of timers, ON/OFF |

Display 04.03: Test timer display (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Test timer display      +

+                         +

+ Power on time  aaaa     +

+ Service time   bbbb     +

+ No-serv time   cccc     +

+ Trans on time  dddd     +

+ Timers state   eee      +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| AAbbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSS | FTD_SB_POWER_ON_TIMER |  | R | Timer for how long the phone has been powered on |
| bbbb | GSS | FTD_SB_SERV_TIMER |  | R | Timer for how long the phone has been in service |
| cccc | GSS | FTD_SB_NSPS_TIMER |  | R | Timer for NO-SERV POWER-SAVE state |
| dddd | GSS | FTD_SB_TRANSMIT_TIMER |  | R | Timer for how long the transmitter has been on |
| eee | GSS | FTD_SB_TEST_TIMER_STATE |  | R | State of timers, ON/OFF |

[/FTD_FIELDS]

All the values are shown in one minute resolution. The accuracy of the timers is about one second. The display uses following format for timers: HHMM  where HH is hours and MM is minutes.

NOTE: When the maximum usage time of the phone is required (e.g. idle time measurement) then ALL field test displays must be deactivated! See chapter Error: Reference source not found Error: Reference source not found, how to deactivate the field test displays.

Display 04.04: Reset SMS and Call Counters to zero (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

With this display all counters listed on the Display 04.05: Call attempts counters (Version: 1, Status: Approved),  and Display 04.08: SMS timeout counters can be reset.

Executive Display:

***************************

*     Execute   display   *

* With this display it is *

* possible  to  reset     *

* SMS and Call counters   *

* to zero. Displays 04/05 *

* 05/07, 04/08            *

* Use options to change   *

*                         *

*                         *

*                         *

***************************

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | GSS | FTD_SB_RESET_TEST_COUNTERS | N/A | R | Reset SMS & Call counter displays |

Display 04.05: Call attempts counters (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+  Call attempts counters +

+                         +

+ CC cause      Aa        +

+ Release dire  bb        +

+ MO attempts   ccc       +

+ MO call ok    ddd       +

+ MT attempts   eee       +

+ MT call ok    fff       +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| Aa | GSS | FTD_SB_LAST_CALL_REL_REAS |  | R | Reason of last call release: Cause from messages disconnect and release complete |
| bb | GSS | FTD_SB_LAST_CALL_REL_DIRECTION |  | R | Direction of last call release: UN : Unknown MO : Mobile originated MT : Mobile terminated IN : Internal (ME CS sw) |
| ccc | GSS | FTD_SB_MO_CALL_COUNT |  | R | Count of all MO call attempts made |
| ddd | GSS | FTD_SB_MO_CALL_OK_COUNT |  | R | Count of succeeded MO calls |
| eee | GSS | FTD_SB_MT_CALL_COUNT |  | R | Count of all call setups received |
| fff | GSS | FTD_SB_MT_CALL_OK_COUNT |  | R | Count of succeeded MT calls |

[/FTD_FIELDS]

On poweroff the values of the counter displays are stored onto the permanent memory, where they will be read during power on. To initialize the counters to zero, select Display 04.04: Reset SMS and Call Counters to zero. Counters are automatically reset to zero when they exceed their maximum value.

Display 04.06: Information about reasons for call clearing (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Information from call   +

+ clearing reasons        +

+                         +

+ CC cause      aaaa      +

+                         +

+ MM cause      bbbb      +

+                         +

+ RR cause      cccc      +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSS | FTD_SB_CC_CAUSE |  | R | CC cause value, see section 10.5.4.11/GSM 04.08. '*' is shown in front of cause value if cause is made up by CC layer in MS |
| bbbb | GSS | FTD_SB_MM_CAUSE |  | R | MM cause value, see section 10.5.3.6/GSM 04.08. '*' is shown in front of cause value if cause is made up by MM layer in MS |
| cccc | GSS | FTD_SB_RR_CAUSE |  | R | RR cause value, see section 10.5.2.31/GSM 04.08. '*' is shown in front of cause value if cause is made up by RR layer in MS |

[/FTD_FIELDS]

All cause values are shown in decimal form.

Display 04.07: SMS attempts counters (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+  SMS attempts counters  +

+ Last MO fail  aa        +

+ MO all        bbb       +

+ MO Ok         ccc       +

+ Last MT fail  dd        +

+ MT all        eee       +

+ MT Ok         fff       +

+ Broad sche    gggg      +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | GSS | FTD_SB_LAST_MO_SMS_FAIL |  | R | Reason of last sending failure |
| bbb | GSS | FTD_SB_MO_SMS_COUNT |  | R | Count of all MO short message attempts |
| ccc | GSS | FTD_SB_MO_SMS_OK_COUNT |  | R | Count of succeeded MO short message attempts |
| dd | GSS | FTD_SB_LAST_MT_SMS_FAIL |  | R | Reason of last receiving failure |
| eee | GSS | FTD_SB_MT_SMS_COUNT |  | R | Count of all MT short message attempts |
| fff | GSS | FTD_SB_MT_SMS_OK_COUNT |  | R | Count of succeeded MT short message attempts |
| gggg | GSS | FTD_SB_CB_SCHEDULE_MSG |  | R | Count of all received cell broadcast schedule messages |

[/FTD_FIELDS]

On poweroff the values of the counter displays are stored onto the permanent memory, where they will be read during power on. To initialize the counters to zero Display 04.04: Reset SMS and Call Counters to zero. Counters are automatically reset to zero when they exceed their maximum value.

Display 04.08: SMS timeout counters (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+  SMS timeout counters   +

+                         +

+ TR1M           aaa      +

+ TR2M           bbb      +

+ TRAM           cc       +

+ TC1M           ddd      +

+ TC2M           eee      +

+ CB schedule    ff       +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | GSS | FTD_SB_TR1M_COUNT |  | R | Counter for TR1M timeouts |
| bbb | GSS | FTD_SB_TR2M_COUNT |  | R | Counter for TR2M timeouts |
| cc | GSS | FTD_SB_TRAM_COUNT |  | R | Counter for TRAM timeouts |
| ddd | GSS | FTD_SB_TC1M_COUNT |  | R | Counter for TC1M timeouts |
| eee | GSS | FTD_SB_TC2M_COUNT |  | R | Counter for TC2M timeouts |
| ff | GSS | FTD_SB_CB_SCHEDULE_COUNT |  | R | Counter for CB schedule timeouts |

[/FTD_FIELDS]

On poweroff the values of the counter displays are stored onto the permanent memory, where they will be read during power on. To initialize the counters to zero, select Display 04.04: Reset SMS and Call Counters to zero. Counters are automatically reset to zero when they exceed their maximum value.

Display 04.09: Toggle transmitter functionality (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

When selected, disables transmitter functionality if enabled and vice versa.  New setting is valid until next power off or until new execute of this display.

Executive Display:

***************************

*     Execute   display   *

* With this display it is *

* possible  to  disable   *

* or enable transmitter   *

* Use options to change.  *

*                         *

*       xxxxxxxx          *

*                         *

*                         *

***************************

Options

Enabled or disabled

This FTD can be used to simulate easily situations when the MS can hear the network (i.e. receiving signal is good enough), but the network can not receive any messages from the MS.

Location updating attempts or MO call establishment attempts can be failed (random access failure) by this FTD and field testing of these failures is much easier now.

Next periodic location updating can be checked from the Display 01.08: Paging repeat period, TMSI, periodic location update, AFC and AGC (Version: 3.19, Status: Approved) by taking the difference of current T3212 counter value and T3212 timeout value.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaaaaaa | GSS | FTD_SB_TOGGLE_TRANS_STATUS |  | R | Transmitter status |
| EXE | GSS | FTD_SB_TOGGLE_TRANS_STATUS_CHANGE | N/A | R | Toggle transmitter functionality |

- Group 05: GSM Data Displays

Display 05.01: HSCSD, Timeslot information display (Version: 3.19, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+     HSCSD     Timeslot  +

+   information display   +

+                         +

+ Timeslot       01234567 +

+ RX             rrrrrrrr +

+ TX             tttttttt +

+ Main CH Ts     x        +

+ Main CH power  yy       +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Mode changed / Inspection 30-Sep-02, SWAM ADB AE02092038099

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| rrrrrrrr | GSS | FTD_SB_RX_TIMESLOT_ALLOC |  | R,I,O | Each bit can have value 1 or 0, 1 indicating that timeslot is part of multislot allocation in downlink direction. |
| tttttttt | GSS | FTD_SB_TX_TIMESLOT_ALLOC |  | R,I,O | Each bit can have value 1 or 0, 1 indicating that timeslot is part of multislot allocation in uplink direction. |
| x | GSS | FTD_SB_MAIN_CH_PLACE |  | R,I,O | Value from 0 to 7 indicating the place of main channel. In case of GPRS multislot allocation, this is not valid "x". |
| yy | GSS | FTD_SB_MAIN_CH_PWR_CONTROL |  | R,I,O | Value from 0-31 indicating main channel power control level. In case of GPRS multislot allocation, this is not valid "xx". |

Display 05.02: HSCSD, Timeslot power control level display (Version: 3.19, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Main CH TS     x        +

+ Main CH power  yy       +

+ Pwr level TS0  aa       +

+ Pwr level TS1  bb       +

+ Pwr level TS2  cc       +

+ Pwr level TS3  dd       +

+ Pwr level TS4  ee       +

+ Pwr level TS5  ff       +

+ Pwr TS6 / TS7  gg    hh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Mode changed / Inspection 30-Sep-02, SWAM ADB AE02092038099

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| x | GSS | FTD_SB_MAIN_CH_PLACE |  | R,I,O | Value from 0 to 7 indicating the place of main channel. In case of GPRS multislot allocation, this is not valid "x". |
| yy | GSS | FTD_SB_MAIN_CH_PWR_CONTROL |  | R,I,O | Value from 0-31 indicating main channel power control level. In case of GPRS multislot allocation, this is not valid "xx". |
| aa | GSS | FTD_SB_PWR_CONTROL_TS_0 |  | R,I,O | Power control level for timeslot 0. |
| bb | GSS | FTD_SB_PWR_CONTROL_TS_1 |  | R,I,O | Power control level for timeslot 1. |
| cc | GSS | FTD_SB_PWR_CONTROL_TS_2 |  | R,I,O | Power control level for timeslot 2. |
| dd | GSS | FTD_SB_PWR_CONTROL_TS_3 |  | R,I,O | Power control level for timeslot 3. |
| ee | GSS | FTD_SB_PWR_CONTROL_TS_4 |  | R,I,O | Power control level for timeslot 4. |
| ff | GSS | FTD_SB_PWR_CONTROL_TS_5 |  | R,I,O | Power control level for timeslot 5. |
| gg | GSS | FTD_SB_PWR_CONTROL_TS_6 |  | R,I,O | Power control level for timeslot 6. |
| hh | GSS | FTD_SB_PWR_CONTROL_TS_7 |  | R,I,O | Power control level for timeslot 7. |

Display 05.03: Transparent data information (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Transparent data info   +

+ Num of CH's    Aaa      +

+ Connect speed  Bbbb     +

+ Connect type   cccc     +

+ Connect mode   dddd     +

+ Mode status    eee      +

+ SB stat bit    fff      +

+ X stat bit     gg       +

+ V.80           hhhh     +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| Aaa | CSD | FTD_SB_TB_NUM_CHAN |  | R | Number of channels. Value 1+1 or 2+2. Applicable only in GSM mode, otherwise show xxx. |
| Bbbb | CSD | FTD_SB_TB_CON_SPEED |  | R | Connection speed in kbit/s. 2.4 : 2400 bit/s 4.8 : 4800 bit/s 9.6 : 9600 bit/s 14.4 : 14400 bit/s 19.2 : 19200 bit/s 28.8 : 28800 bit/s 32: 32000 bits/s 33.6: 33600 bits/s 56: 56000 bits/s 64: 64000 bits/s |
| cccc | CSD | FTD_SB_TB_CON_TYPE |  | R | Connection type. Asyn :When connection is asynchronous Syn :When connection is synchronous |
| dddd | CSD | FTD_SB_TB_CON_MODE |  | R | Connection mode. Buff : Connection is in buffered mode V.80 : Connection is in V.80 mode |
| eee | CSD | FTD_SB_TB_STATUS_CON_MODE |  | R | Status of connection mode. SCN : When synchronizing CON : When connected Applicable only in GSM mode, otherwise show xxx. |
| fff | CSD | FTD_SB_TB_STATUS_SB |  | R | Status bit SB value 0 or 1. SB0: Status ON SB1: Status OFF Applicable only in GSM mode, otherwise show xxx. |
| gg | CSD | FTD_SB_TB_STATUS_X |  | R | Status bit X value 0 or 1. X0 : Status ON X1 : Status OFF Applicable only in GSM mode, otherwise show xx. |
| hhhh | CSD | FTD_SB_TB_V80_SM |  | R | When V.80 is used the sub mode is shown. Tran : Transparent Fram : Framed |

Display 05.04: Non transparent data (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Non Transparent data    +

+                         +

+ Sent I-farmes  aaaaaaaa +

+ X-bit out      bb       +

+ Re-trans       cccccccc +

+ Uplink buff    dd       +

+ Received I-fr  eeeeeeee +

+ X-bit in       ff       +

+ SREJ frames    gggggggg +

+ Downlink buff  hh       +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaaaaaa | CSD | FTD_SB_NTB_ NUM_SENT_I_FRAME |  | R | Number of sent I-frames. 32-bit hex number and range is 0 - FFFFFFFF. |
| bb | CSD | FTD_SB_NTB_ STATUS_XOUT |  | R | X-bit out value 0 or 1. X0 : Status OFF X1 : Status ON |
| cccccccc | CSD | FTD_SB_NTB_ NUM_RETRANS_FRAME |  | R | Number of retransmitted frames. 32-bit hex number and range is 0 - FFFFFFFF. |
| dd | CSD | FTD_SB_NTB_UPLINK_ BUF |  | R | Uplink buffer overflow. 8-bit hex number and range is 0 – FF. If not supported xx is shown. |
| eeeeeeee | CSD | FTD_SB_NTB_ NUM_RECV_I_FRAME |  | R | Number of received I-frames. 32-bit hex number and range is 0 - FFFFFFFF. |
| ff | CSD | FTD_SB_NTB_ STATUS_XIN |  | R | X-bit in. Value is 0 or 1. X0 : Status OFF X1 : Status ON |
| gggggggg | CSD | FTD_SB_NTB_ NUM_SREJ_FRAMES |  | R | Number of SREJ'ed frames. 32-bit hex number and range is 0 - FFFFFFFF. |
| hh | CSD | FTD_SB_NTB_ DOWNLINK_BUF |  | R | Downlink buffer overflow. 8-bit hex number and range is 0 - FF. If not supported xx is shown. |

Display 05.05: Facsimile information (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

Note! This display is not yet updated for 3G phones.

Data Display:

+++++++++++++++++++++++++++

+ Fax call pha  aa        +

+ Fax class     bb        +

+ Err corr mode c         +

+ MO or MT      dd        +

+ TrSpd / DTE   eee  ff   +

+ Cts / Cls2St  ggg  hh   +

+ Cls2Us / Cls1 ii   j    +

+ Flow / OtDT   k    llll +

+ OfDT / GSM st mmmm n    +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | CSD | FTD_SB_FAX_PHASE |  | R | Shows the phase of the facsimile call when Class is 2 or 2.0. 0 : no call (default and in Class 1) BR : Phase B receive BT :Phase B transmit CR : Phase C receive CT : Phase C transmit DR : Phase D receive DT : Phase D transmit E : Phase E |
| bb | CSD | FTD_SB_FAX_CLASS |  | R | Selected fax class. 0 : Not selected yet (default) 1 : Class 1 2 : Class 2 20 : Class 2.0 |
| c | CSD | FTD_SB_FAX_ECM |  | R | Error correction mode. 1 : Used 0 : Not used |
| dd | CSD | FTD_SB_FAX_DIR |  | R | MO: Mobile originated MT: mobile terminated. |
| eee | CSD | FTD_SB_FAX_ IMAGE_TRANS_SPEED |  | R | Negotiated image transfer speed. 0 : not yet negotiated (default) 24 : 2400 bit/s 48 : 4800 bit/s 72 : 7200 bit/s 96 : 9600 bit/s 120 : 12000 bit/s 144 : 14400 bit/s |
| ff | CSD | FTD_SB_FAX_ NUM_SESSIONS |  | R | Number of different DTE sessions for which fax has created tables. Range is 0 – 99, default is 0. |
| ggg | CSD | FTD_SB_FAX_ CALL_TERM_STATUS |  | R | Call termination status, value range is 0 – FF as specified in Appendix A tables A.1 & A.2 for Class 2.0 and 0-255 for Class 2 (tables A.3 & A.4). Default is 0. |
| hh | CSD | FTD_SB_FAX_ CLASS2_SUBSTATES |  | R | Class 2/2.0 substate. 8-bit hex number and range is 0 – 2C. (Appendix A, Table A.5 ) |
| ii | CSD | FTD_SB_FAX_ CLASS2_UTIL_STATE |  | R | Class 2/2.0 utility state. 8-bit hex number and range is 0 –1C. (Appendix A, Table A.6) |
| j | CSD | FTD_SB_FAX_ CLASS1_SUBSTATE |  | R | Class 1 substate. Range is 0 – 9. (Appendix A, Table A.7) |
| k | CSD | FTD_SB_FAX_ FLOW_CTRL |  | R | Flow control enabled by fax. 1 : ON 0 : OFF |
| llll | CSD | FTD_SB_FAX_ TO_DTE_BUF |  | R | Buffer towards DTE overflowed, number of lost octets shown. Range is 0 – 9999. |
| m(4) | CSD | FTD_SB_FAX_ FROM_DTE_BUF |  | R | Buffer from DTE overflowed, number of lost octets shown. Range is 0 – 9999. |
| n | CSD | FTD_SB_FAX_ GSM_STATE |  | R | State of the GSM fax module. Range is 0 – 8. (Appendix A, table A.8) |

Display 05.06: CSD Server, main data display (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+   CSD Server  main data +

+ DTE ID      aaaaaaaaaaaa+

+ TCH ID        b         +

+ DTE context   c         +

+ Conf call st  def       +

+ State machin  gggg      +

+ GPDS connect  h         +

+ Pend trans    iii       +

+ Pend timeout  jjjjjj    +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(12) | CSD | FTD_SB_CSDS_ MDD1_DTE_ID |  | R | Existing DTE IDs. Each character represents a DTE (upto 12 separate sessions). DTE ID is 4-bit hex number and range is 0 – C and x. x : session doesn't exists Example: the value 01345xxxxxxx means that there are five DTE sessions with DTE IDs a0 = 0, a1 = 1, a2 = 3, a3 = 4 and a4 = 5. F means that the session ai , i  [5 - 11] doesn't exists. |
| b | CSD | FTD_SB_CSDS_ MDD1_TCH_ID |  | R | TCH ID. 4-bit hex number and range is 0 – C and x. x : Not active |
| c | CSD | FTD_SB_CSDS_ MDD1_NUM_OF_CALLS |  | R | Number of call contexts within DTEs. Range is 0 - 6. |
| def | CSD | FTD_SB_CSDS_ MDD1_CONFERENCE |  | R | d = Conference call active. A : Active, x : Not active. e = Conference call state. A : Active H : On hold x : Not active. f = DTE ID of conference owner DTE. 4-bit hex number and range is 0 - C and x. x : Conference not active. |
| g(4) | CSD | FTD_SB_CSDS_ MDD1_STATE |  | R | State Machine state information. 4-bit hex numbers. g0 = Main state. Range is 0 – 8. g1 = State branching condition. Range is 0 – F. g2 = Next state. Range is 0 – 8. g3 = Call stack depth. Range is 0 – 4. |
| h | CSD | FTD_SB_CSDS_ MDD1_NUM_ GPDS_ CONNECTIONS |  | R | Number of GPDS connections. 4-bit hex number and range is 0 – B. |
| iii | CSD | FTD_SB_CSDS_ MDD1_NUM_OF_TRANSACTIONS |  | R | Number of pending transactions. Range is 0 - 999. |
| j(6) | CSD | FTD_SB_CSDS_ MDD1_MB_ TIMER_STATUS |  | R | Number of delta elements (pending time-outs) for a timer. There are six timers and each of the timers use one digit (k). Range is 0 – Z (0 – 40). j0 : DTR. j1 : HANGUP. j2 : ALERT. j3 : GENERIC. j4 : DIAL_DELAY. j5 : USSD_REPLY. |

Display 05.07: CSD Server, active DTE info 1/2, call related info (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ DTE ID        a         +

+ Num of calls  b         +

+ Csta / Cmode  cccc   dd +

+ TCH ID / Cdi  e      ff +

+ AcID / HcID   gg     hh +

+ PeT / Ct inf  iii    ll +

+ Ctr / NCtr    jj     kk +

+ SM st / LeID  mmmm  nnnn+

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	O-field removed / Inspection 14-Jun-02, SWAM ADB AV02061036011

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | CSD | FTD_SB_CSDS_ ADTE1_DTE_ID |  | R | DTE ID. 8-bit hex number and range is 0 – C and x. x : Not active |
| b | CSD | FTD_SB_CSDS_ ADTE1_NUM_OF_CALLS |  | R | Number of calls. Range is 0 – 2. |
| cccc | CSD | FTD_SB_CSDS_ ADTE1_CALL_STATE |  | R | Current call state. IDLE : IDLE MTCO : MT CONNECTING WCON : WAITING CONNECT CONN : CONNECTED DISC : DISCONNECTING |
| dd | CSD | FTD_SB_CSDS_ ADTE1_CALL_MODE |  | R | Call mode. 0 : EMERGENCY 1 : SPEECH 2 : DATA 3 : FAX A0 : ALT SPEECH DATA A1 : ALT DATA SPEECH A2 : ALT SPEECH FAX A3 : ALT FAX SPEECH A4 : SPEECH THEN DATA A5 : UNDEFINED |
| e | CSD | FTD_SB_CSDS_ ADTE1_TCH_ID |  | R | TCH ID. 8-bit hex number and range is 0 – C and x. x : No TCH channel active. |
| ff | CSD | FTD_SB_CSDS_ ADTE1_CALL_DIR |  | R | Call direction. MO : Mobile Originated MT : Mobile Terminated |
| gg | CSD | FTD_SB_CSDS_ ADTE1_ACTIVE_CALL_ID |  | R | Active call ID. 8-bit hex number and range is 0 – 9 and 80 – 83. 0 : CSDC_CALL_ID_NONE 1 – 9 : Call id from CALL server 80 : CSDC_CALL_ID_ALL 81 : CSDC_CALL_ID_ACTIVE 82 : CSDC_CALL_ID_HOLD 83 : CSDC_CALL_ID_CONFERENCE |
| hh | CSD | FTD_SB_CSDS_ ADTE1_ON_HOLD_CALL_ID |  | R | Held call ID. 8-bit hex number and range is 0 – 9 and 80 – 83. Interpretation, as above. |
| iii | CSD | FTD_SB_CSDS_ ADTE1_NUM_OF_PENDING_TRANS |  | R | Number of pending transactions. Range is 0 – 999. |
| jj/kk | CSD | FTD_SB_CSDS_ ADTE1_CALL_ TERM_REASON |  | R | jj = Call termination reason. 8-bit hex number and range is 0 – 13. kk = Network related call termination reason. 8-bit hex number and range is 1 – 7F. |
| ll | CSD | FTD_SB_CSDS_ ADTE1_CALL_ TERM_ORIG_AND_DIR |  | R | l0 = Call termination origin. C : Client S : Server N : Network l1 = Call termination direction. O : Mobile Originated T : Mobile Terminated U : Unspecified / Unknown |
| mmmm | CSD | FTD_SB_CSDS_ ADTE1_SM_ STATE_INFO |  | R | SM State information. 4-bit hex numbers. m0 = Main state. Range is 0 – 8. m1 = State branching condition. Range is 0 – F. m2 = Next state. Range is 0 – 8. m3 = Call stack depth. Range is 0 – 4. |
| nnnn | CSD | FTD_SB_CSDS_ ADTE1_LAST_EVENT |  | R | Last event id. 16-bit hex value and range is 0 – 7FFF. |

Display 05.08: CSD Server, active DTE info 2/2, GPDS, socket, conference info (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ CSD server DTE,GPDS     +

+ DTE ID        a         +

+ Act call ID   bb        +

+ GPDS ID,stat  cc        +

+ GPRS quality  dddddd    +

+ Socket conn   ee        +

+ DTE ID        f         +

+ Conff call st g         +

+ Conff call ID hhhhh     +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | CSD | FTD_SB_CSDS_ ADTE2_DTE_ID |  | R | DTE ID. 8-bit hex number and range is 0 – C and x. x : No DTE ID available |
| bb | CSD | FTD_SB_CSDS_ ADTE2_ACTIVE_CALL_ID |  | R | Active call ID. 8-bit hex number and range is 0 – 9 and 80 – 83. 0 : CSDC_CALL_ID_NONE 1 – 9 : Call id from CALL server 80 : CSDC_CALL_ID_ALL 81 : CSDC_CALL_ID_ACTIVE 82 : CSDC_CALL_ID_HOLD 83 : CSDC_CALL_ID_CONFERENCE |
| cc | CSD | FTD_SB_CSDS_ ADTE2_GPDS_ CID_AND_STATE |  | R | c0 = GPDS context id. 4-bit hex number and range is 0 – 7. c1 = GPDS status value A or D. A : Attached D : Detached |
| dddddd | CSD | FTD_SB_CSDS_ ADTE2_GPDS_ QOS |  | R | GPRS Quality of service. 4-bit hex numbers. D0 : Precedence class. Range is 0 – 3. D1 : Delay class. Range is 0 – 4. D2 : Reliability class. Range is 0 – 5. D3 : Peak throughput class. Range is 0 – 9. d4 and d5: Mean throughput class. Range is 0 – 13 (8-bit hex). |
| ee | CSD | FTD_SB_CSDS_ ADTE2_SOCKET_INFO |  | R | Socket connection. 00 : No 01 : Yes |
| f | CSD | FTD_SB_CSDS_ ADTE2_ CONFERENCE_ OWNER |  | R | DTE ID of a conference owner. 8-bit hex number and range is 0 – C and x. x : No DTE ID available |
| g | CSD | FTD_SB_CSDS_ ADTE2_ CONFERENCE_ STATUS |  | R | Conference call status. A : Active H : Held |
| h(5) | CSD | FTD_SB_CSDS_ ADTE2_ CONFERENCE_ PARTICIPANTS |  | R | Call ids in a conference. Each hi (0 < i < 4) is a call id of a participating call. Each hi is in range 1 – 9. |

Display 05.09: GPRS Rel99 Display (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Context info  aa b cc dd+

+ Act user cid  ee        +

+ PDP type      fff       +

+ QoS	      gggg      +

+ Max up/down   hhh   iii +

+ SDU error     jj        +

+ Neg QoS up    kkk       +

+ Neg Qos down  lll       +

+ Residual BER  mmm       +

+++++++++++++++++++++++++++

To change GPRS context, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 0509 (TEST GROUP;TEST DISPLAY) to the query prompt.

- Test input prompt will be shown on the display, enter the context number in display.

- Confirm with the Ok button.

- If change succeeded, the new session information are displayed in display 09 of group 05.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | CSD | FTD_SB_GPRS99_ CONTEXT_ID |  | R | User context id. Range is 1 - 10. |
| b | CSD | FTD_SB_GPRS99_ CONTEXT_STATE |  | R | GPRS context state. A : Attached D : Detached |
| cc | CSD | FTD_SB_GPRS99_ GPDS_CONTEXT_ID |  | R | GPDS context id. 8-bit hex number. Range is 0 - FF. |
| dd | CSD | FTD_SB_GPRS99_ NBR_GPDS_CONNECTIONS |  | R | Number of GPDS connections. Range is 0 – 99. |
| ee | CSD | FTD_SB_GPRS99_ ACT_USER_CID |  | R | Active user cid. Range is 1 - 10. |
| fff | CSD | FTD_SB_GPRS99_ PDP_TYPE |  | R | PDP type. IP : Internet Protocol IP6 : Internet Protocol version 6 PPP : Point to Point Protocol |
| gggg | CSD | FTD_SB_GPRS99_NEQ_QOS |  | R | Negotiated Qos profile. g0 : Traffic class (0-3) g1 : Delivery order (0-1) g2 : Delivery of erroneous SDUs (0-2) g3 : Traffic handling priority (0-2) |
| hhh | CSD | FTD_SB_GPRS99_NEQ_MAX_UL |  | R | Negotiated Qos profile's maximum uplink bitrate. Range is 0-64 kbit/s. |
| iii | CSD | FTD_SB_GPRS99_NEQ_MAX_DL |  | R | Negotiated Qos profile's maximum downlink bitrate. Range is 0-384 kbit/s. |
| jj | CSD | FTD_SB_GPRS99_SDER |  | R | SDU error ratio. 00 : 0E0 (Subscribed) 11 : E 12 : E2 13 : E3 14 : E4 15: E5 15 : E6 73 : 7E3 FF : Default |
| kkk | CSD | FTD_SB_GPRS99_NEQ_GUA_UL |  | R | Negotiated Qos profile's quaranteed uplink bitrate. Range is 0-64 kbit/s. |
| lll | CSD | FTD_SB_GPRS99_NEQ_GUA_DL |  | R | Negotiated Qos profile's quaranteed downlink bitrate. Range is 0-384 kbit/s. |
| mm | CSD | FTD_SB_GPRS99_RBER |  | R | Residual bit error rate. 00 : 0E0 (Subscribed) 12 : E2 13 : E3 14 : E4 15 : E5 16 : E6 43 : 4E3 52 : 5E2 53 : 5E3 68 : 6E8 FF : Default |
| INPUT | CSD | FTD_SB_GPRS99_CHANGE_CONTEXT | DW:D | R | Change GPRS context. |

Display 05.10: MAC Interface Display (Version: 1, Status: Approved)Display 05.10: MAC Interface Display (Version: 1, Status: Approved) Display 05.10: MAC Interface Display (Version: 1, Status: Approved)

Display 05.10: MAC Interface Display (Version: 1, Status: Approved)

Data Display:

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

+++++++++++++++++++++++++++

+ MAC Interface display   +

+                         +

+ Allowed frame           +

+ combinations            +

+                         +

+ aaaaaaaaa               +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(9) | CSD | FTD_SB_MAC_ALLOWED_COMB |  | R | Allowed frame combinations for CSD-MAC to use. Each character represents one combination, 0 means that combination is not allowed. If zero frame is allowed, then it will be marked as 1. an : n = associated transmission frame a1 = zeroth frame a9 = eighth frame Example: 112040008 Means that legal combination of frames are 0,1,2,4 and 8. |

Display 05.11: CSD Server Main Data Display, Control 2001 (Version: 1, Status : Approved)

Data Display:

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

This display is applicable for Kenny.

+++++++++++++++++++++++++++

+ Call mode     A         +

+ DTE/Calls     bb  cc    +

+ Pending trans     ddd   +

+ RS Irda       e   f     +

+ UAU UAN       ggg hhh   +

+ UCU UCN       iii jjj   +

+ UDU UDN       kkk lll   +

+ BT/Socket     m   n     +

+ DCP/Unknown   o   p     +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| A | CSD | FTD_SB_CSDS_ MAIN_ACT_CALL_MODE |  | R | Active call mode. G : GSM W : WCDMA |
| bb | CSD | FTD_SB_CSDS_ MAIN_NBR_DTE |  | R | Number of DTE sessions. Range: 0 – 50. |
| cc | CSD | FTD_SB_CSDS_ MAIN_NBR_CALL |  | R | Number of connected calls. Range: 0 – 99. |
| ddd | CSD | FTD_SB_CSDS_ MAIN_PENDING_TRANS |  | R | Number of pending transactions. Range is 0-999. |
| e | CSD | FTD_SB_CSDS_ MAIN_RS232 |  | R | A DTE of the type RS232 connected. R : If connected x : No RS232 connection |
| f | CSD | FTD_SB_CSDS_ MAIN_IRDA |  | R | A DTE of the type IrDA connected. I : If connected x : No IrDa connection |
| ggg | CSD | FTD_SB_CSDS_ MAIN_USB_UAU |  | R | A DTE of the type USB connected. (AT cmd, unsolicited responses) UAU : If connection xxx : No connection |
| hhh | CSD | FTD_SB_CSDS_ MAIN_USB_UAN |  | R | A DTE of the type USB connected. (AT cmd, no unsolicited responses) UAN : If connection xxx : No connection |
| iii | CSD | FTD_SB_CSDS_ MAIN_USB_UCU |  | R | A DTE of the type USB connected. (Modem ctrl, unsolicited responses) UCU : If connection xxx : No connection |
| jjj | CSD | FTD_SB_CSDS_ MAIN_USB_UCN |  | R | A DTE of the type USB connected. (Modem ctrl, no unsolicited responses) UCN : If connection xxx : No connection |
| kkk | CSD | FTD_SB_CSDS_ MAIN_USB_UDU |  | R | A DTE of the type USB connected. (Modem data, unsolicited responses) UDU : If connection xxx : No connection |
| lll | CSD | FTD_SB_CSDS_ MAIN_USB_UDN |  | R | A DTE of the type USB connected. (Modem data, no unsolicited responses) UDN : If connection xxx : No connection |
| m | CSD | FTD_SB_CSDS_MAIN_BT |  | R | A DTE of the type Bluetooth connected. B : If connected x : No Bluetooth connection |
| n | CSD | FTD_SB_CSDS_MAIN_SCK |  | R | A DTE of the type Socket connected. S : If connected x : No Socket connection |
| o | CSD | FTD_SB_CSDS_MAIN_DCP |  | R | A DTE of the type DCP connected. D : If connected x : No DCP connection |
| p | CSD | FTD_SB_CSDS_MAIN_UNK |  | R | An unknown DTE type connected. U : Unknown connection x : No unknown connection |

Display 05.12: Control DTE and Call Info Displays, Control 2001 (Version: 1, Status: Approved)

Data Display:

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

Next display is applicable for  Kenny.

+++++++++++++++++++++++++++

+ DTE and Call info       +

+                         +

+ Locked session          +

+                         +

+ DTE           aaa       +

+ Call          bb        +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

To change DTE or Call session, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 0512 (TEST GROUP;TEST DISPLAY) to the query prompt.

- Test input prompt will be shown on the display, enter the session number in display.

- Confirm with the Ok button.

- If change succeeded, the new session information are displayed in group 5 display 13  for DTE and display 14 for Call. Also locked session information are changed in display 12 of group 5.

| Abbreviation | Description |
| --- | --- |
| XX | Locked DTE session. DTE session type. XX = 1 : RS232 XX = 2 : IRDA XX = 3 : Bluetooth XX = 4 : USB AT command, no unsolicited responses XX = 5 : USB AT command, unsolicited responses XX = 6 : USB modem control, no unsolicited responses XX = 7 : USB modem control, unsolicited responses XX = 8 : USB modem data, no unsolicited responses XX = 9 : USB modem data, unsolicited responses XX = 10 : Socket XX = 11 : DCP XX = 12 : Unknown |
| 2y 30 31 | Locked Call session. y : Call ID. Range is 1 – 7. 30 : MO call being created 31 : MT call being created |

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | CSD | FTD_SB_CSDS_INFO_DTE_STATE |  | R | Show locked DTE session. |
| bb | CSD | FTD_SB_CSDS_INFO_CALL_STATE |  | R | Show locked Call session. |
| INPUT | CSD | FTD_SB_CSDS_INFO_CHANGE_DISPLAY | DW:D | R | Change DTE or Call display's session. |

[/FTD_FIELDS]

This display is used for selecting the Call or DTE session the information of which the user wants to browse.

The CSD server Main Display indicates the total number of existing DTE sessions as well as what types of DTEs are included. The user selects the DTE session which he wants to browse by indicating the type of this session.

Call display is selected by entering the Call ID of the call to be browsed. The user finds the correct Call IDs from the display of the DTE session which owns the call. The user may also request the information of an MO or MT call that is in the process of being created by typing 30 (MO) or 31 (MT).

Display 05.13: Active DTE Display xx, Control 2001 (Version: 1, Status: Approved)

Data Display:

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

Display is applicable for Kenny.

+++++++++++++++++++++++++++

+ Active DTE display      +

+                         +

+ DTE session ID  aaa     +

+ DTE type        bbbbb   +

+ Control state   ccc     +

+ Call ID         ddddddd +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | CSD | FTD_SB_CSDS_ DTE_SESSION_ID |  | R | DTE Session ID. Range is 0 – 255. |
| b(5) | CSD | FTD_SB_CSDS_ DTE_TYPE |  | R | DTE type. NONE : Not active RS : RS232 IRDA : Infra Red Data Association BT : Bluetooth SCKT : Socket DCP : Data Control Protocol FF : Unknown Universal Serial Bus USATN : AT, no unsolicited responses USATU : AT, unsolicited responses USMCN: Modem control, no unsolicited responses USMCU: Modem control, unsolicited responses USMDN: Modem data, no unsolicited responses USMDU: Modem data, unsolicited responses |
| c(3) | CSD | FTD_SB_CSDS_ DTE_CTRL_STATE |  | R | Control state. WC: CSDC_WAIT_FOR_COMMAND CP: CSDC_COMMAND_PROCESSING WRA:CSDC_WAIT_FOR_REMOTE_ACK PNC: CSDC_PROCESS_NEXT_CMD SRM: CSDC_SMS_READ_MODE |
| d(7) | CSD | FTD_SB_CSDS_ DTE_CID |  | R | Associated call ids. dn : n = associated call id. Range is 1 - 7. Example: the values 1xxx5xx means that associated call ids are 1 and 5. |

Display 05.14: Active Call Display (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Call ID	      aaa       +

+ Call mode     bb        +

+ Call type     c         +

+ Service       ddd       +

+ Call state    eeee      +

+ Call dir      ff        +

+ DTE ID        ggg       +

+ System        h         +

+ Cid           iii       +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | CSD | FTD_SB_CSDS_ CALL_ID |  | R | Call ID. Range is 1 – 7. |
| bb | CSD | FTD_SB_CSDS_ CALL_CURRENT_ MODE |  | R | Current Call Mode. EM : EMERGENCY SP : SPEECH DA : DATA FA : FAX SF : ALT SPEECH FAX FS : ALT FAX SPEECH UN : UNDEFINED |
| c | CSD | FTD_SB_CSDS_ CALL_TYPE |  | R | Call Type S: Single D: Dual |
| ddd | CSD | FTD_SB_CSDS_ CALL_SERVICE |  | R | Service A: ASYNC S: SYNC F1: Fax Class 1 F2: Fax Class 2 F20: Fax Class 2.0 |
| eeee | CSD | FTD_SB_CSDS_ CALL_STATE |  | R | Current call state. IDLE : CSDC_IDLE OCM : CSDC_ONLINE_CMD_MODE DCG : CSDC_DATA_CONNECTING DCD : CSDC_DATA_CONNECTED MTA : CSDC_MT_CALL_ANSWERED MTCG : CSDC_MT_CALL_CONNECTING MOCG : CSDC_MO_CALL_CONNECTING VC : CSDC_VOICE_CONNECTED FCD : CSDC_FAX_CONNECTED REL : CSDC_CALL_RELEASING DTMF : CSDC_DTMF_TRANSMIT |
| ff | CSD | FTD_SB_CSDS_ CALL_DIR |  | R | Call direction. MO : Mobile Originated MT : Mobile Terminated |
| ggg | CSD | FTD_SB_CSDS_ CALL_SESSION_ID |  | R | DTE session ID. Range is 0 – 255. |
| h | CSD | FTD_SB_CSDS_ CALL_SYSTEM |  | R | System. G : GSM W : WCDMA |
| iii | CSD | FTD_SB_CSDS_ CALL_TCH_RAB_ID |  | R | TCH/RAB ID. RAB ID if system is WCDMA and TCH ID for GSM. In WCDMA system this is left empty if RAB ID has not been received. Range is 0 – 255. |

Display 05.15: Common CSD Display (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Received TCH   aaaaaaaa +

+ Dbug1          bb       +

+ Dbug2          cccccccc +

+ Dbug3          dd       +

+ Dbug4          eeeeeeee +

+ Dbug5          ff       +

+ Dbug6          gggggggg +

+ Dbug7          hh       +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | CSD | FTD_SB_COMMON_NUM_TCH_OVERRUNS |  | R | Number of received TCH overrun and underrun indications from TCH pipe. 32-bit hex number and range is 0 - FFFFFFFF. |
| b(2) | CSD | FTD_SB_COMMON_DEBUG_1 |  | R | For general debugging use. String with max. length 2 +1. (* |
| c(8) | CSD | FTD_SB_COMMON_DEBUG_2 |  | R | For general debugging use. 32-bit hex number and range is 0 - FFFFFFFF. (* |
| d(2) | CSD | FTD_SB_COMMON_DEBUG_3 |  | R | For general debugging use. 8-bit hex number and range is 0 – FF. (* |
| e(8) | CSD | FTD_SB_COMMON_DEBUG_4 |  | R | For general debugging use. 32-bit hex number and range is 0 - FFFFFFFF. (* |
| f(2) | CSD | FTD_SB_COMMON_DEBUG_5 |  | R | For general debugging use. String with max. length 2 +1. (* |
| g(8) | CSD | FTD_SB_COMMON_DEBUG_6 |  | R | For general debugging use. 32-bit hex number and range is 0 - FFFFFFFF. (* |
| h(2) | CSD | FTD_SB_COMMON_DEBUG_7 |  | R | For general debugging use. 8-bit hex number and range is 0 – FF. (* |

[/FTD_FIELDS]

(* See csd_common_ftd.h for more information how to activate debugging fields.

Continuus Server:             tresw15

Database Path:                 /opt/ccm_dbs/tr_dcom

Project Name:                   csd4_sw

Directory:                          csd4_sw/common

Filename:                          csd_common_ftd.h

Display 05.16: Current BC-IE (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+                         +

+ BC-IE1 BC-IE2    aa  bb +

+ BC-IE3 BC-IE4    cc  dd +

+ BC-IE5 BC-IE6    ee  ff +

+ BC-IE7 BC-IE8    gg  hh +

+ BC-IE9 BC-IE10   ii  jj +

+ BC-IE11 BC-IE12  kk  ll +

+ BC-IE13 BC-IE14  mm  nn +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	27-Jun-02	AnttiE	Status updated / Inspection 14-Jun-02, SWAM ADB AV02061036011

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | CSD | FTD_SB_BC_IE_OCTET_1 | B:H | R | 1. octet of the BC-IE, which is length. . Range is 00-FF. |
| bb | CSD | FTD_SB_BC_IE_OCTET_2 | B:H | R | 2. octet of the current BC-IE. Range is 00-FF. |
| cc | CSD | FTD_SB_BC_IE_OCTET_3 | B:H | R | 3. octet of the current BC-IE. Range is 00-FF. |
| dd | CSD | FTD_SB_BC_IE_OCTET_4 | B:H | R | 4. octet of the current BC-IE. Range is 00-FF. |
| ee | CSD | FTD_SB_BC_IE_OCTET_5 | B:H | R | 5. octet of the current BC-IE. Range is 00-FF. |
| ff | CSD | FTD_SB_BC_IE_OCTET_6 | B:H | R | 6. octet of the current BC-IE. Range is 00-FF. |
| gg | CSD | FTD_SB_BC_IE_OCTET_7 | B:H | R | 7. octet of the current BC-IE. Range is 00-FF. |
| hh | CSD | FTD_SB_BC_IE_OCTET_8 | B:H | R | 8. octet of the current BC-IE. Range is 00-FF. |
| ii | CSD | FTD_SB_BC_IE_OCTET_9 | B:H | R | 9. octet of the current BC-IE. Range is 00-FF. |
| jj | CSD | FTD_SB_BC_IE_OCTET_10 | B:H | R | 10. octet of the current BC-IE. Range is 00-FF. |
| kk | CSD | FTD_SB_BC_IE_OCTET_11 | B:H | R | 11. octet of the current BC-IE. Range is 00-FF. |
| ll | CSD | FTD_SB_BC_IE_OCTET_12 | B:H | R | 12. octet of the current BC-IE. Range is 00-FF. |
| mm | CSD | FTD_SB_BC_IE_OCTET_13 | B:H | R | 13. octet of the current BC-IE. Range is 00-FF. |
| nn | CSD | FTD_SB_BC_IE_OCTET_14 | B:H | R | 14. octet of the current BC-IE. Range is 00-FF. |

Display 05.17: LLC (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+                         +

+ LLC1          aa        +

+ LLC2          bb        +

+ LLC3          cc        +

+ LLC4          dd        +

+ LLC5          ee        +

+ LLC6          ff        +

+ LLC7          gg        +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	29-Aug-02	AnttiE	Status updated / Inspection 14-Jun-02, SWAM ADB AV02061036011

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | CSD | FTD_SB_LLC_OCTET_1 | B:H | R | 1. octet of the LLC, which is length. . Range is 00-FF. |
| bb | CSD | FTD_SB_LLC_OCTET_2 | B:H | R | 2. octet of the LLC. Range is 00-FF. |
| cc | CSD | FTD_SB_LLC_OCTET_3 | B:H | R | 3. octet of the LLC. Range is 00-FF. |
| dd | CSD | FTD_SB_LLC_OCTET_4 | B:H | R | 4. octet of the LLC. Range is 00-FF. |
| ee | CSD | FTD_SB_LLC_OCTET_5 | B:H | R | 5. octet of the LLC. Range is 00-FF. |
| ff | CSD | FTD_SB_LLC_OCTET_6 | B:H | R | 6. octet of the LLC. Range is 00-FF. |
| gg | CSD | FTD_SB_LLC_OCTET_7 | B:H | R | 7. octet of the LLC. Range is 00-FF. |

- Group 06: GPRS Signaling displays

Display 06.01: Information of the current GPRS state and previous TBF configuration  (Version: 3.17, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.17	Approved	29-Aug-02	AnttiE	L-field explanation corrected / Follow-up for inspection 09-Aug-02 follow-up, SWAM ADB TS02082865409

3.16	Approved	21-Aug-02	AnttiE	L-field length changed 5->4, approved/ Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, n-field corrected, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

If MS is not GPRS attached, xxx is shown in all displays.

+++++++++++++++++++++++++++

+ GPRS state and TBF conf.+

+ Hopping/Carrier a   b(4)+

+ RX lev / Tadv  c(3) d(2)+

+ DL TS / UP TS   e   f   +

+ DL CS / UP CS   g   h   +

+ Tadv I / TFI    i   j   +

+ MAC / Cha type k(4) l(4)+

+ GPRS State      m       +

+ USF values      n(8)    +

+++++++++++++++++++++++++++

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | GSS | FTD_SB_GPRS_HOPPING | S | R,I,O | H, if carrier numbers are scrolled when hopping is on. Otherwise empty. |
| b(4) | GSS | FTD_SB_GPRS_CARRIER | W:D | R,I,O | Carrier number in decimal. If hopping is on, used channels are scrolled when display is updated |
| c(3) | GSS | FTD_SB_GPRS_RX_LEVEL |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100 |
| d(2) | GSS | FTD_SB_GPRS_TIMING_ADV |  | R,I,O | The latest value for timing advance, range 0-63 |
| e | GPDS | FTD_SB_RLC_DL_SLOTS | B:D | R,I,O | Downlink time slot count in the latest TBF configuration, range 0-8 |
| f | GPDS | FTD_SB_RLC_UL_SLOTS | B:D | R,I,O | Uplink time slot count in the latest TBF configuration, range 0-8. |
| g | GPDS | FTD_SB_RLC_DLRX_CS | B:D | R,I,O | The Channel Coding Scheme of the latest downlink data block, range 1-4. |
| h | GPDS | FTD_SB_RLC_ULTX_CS | B:D | R,I,O | The Channel Coding Scheme of the latest uplink data block, range 1-4. |
| I(2) | GSS | FTD_SB_PH_TAI | S | R,I,O | The latest value for timing advance index, range 0-15. If continuous timing advance is/was not in use, xx is shown. |
| j(2) | GPDS | FTD_SB_MAC_DL_TFI | S | R,I,O | TFI of the latest TBF configuration, range 0-31. If there is/was no downlink TBF in the latest TBF configuration, xx is shown. |
| k(4) | GPDS | FTD_SB_MAC_ALLOC_MODE | S | R,I,O | MAC mode, i.e. resource allocation method, used during the latest TBF configuration: DYN EDYN FIXD DUSF: USF granularity in use EUSF: USF granularity in use F_HD: Fixed Half Duplex |
| l(4) | GSS | FTD_SB_GPRS_TYPE_OF_CURR_CH | S |  | Type of current channel In GPRS idle mode, if PBCCH supported in cell: PCCC, PBCC, PAGC, PNDR (non-DRX state): In GPRS idle mode, if PBCCH not supported in cell: AGCH, NDRX (non-DRX state), CCCH, BCCH In GPRS transfer mode: PDTC Other possibilities are the same as in Display 1 in group 1. |
| m | GPDS | FTD_SB_GMM_R_AND_S | S | R,I,O | R if MS is in READY state, S if GPRS is suspended and otherwise empty, |
| n(8) | GSS | FTD_SB_PH_USF |  | R,I,O | USF values of the current TBF configuration, the first value corresponding timeslot 0, next timeslot 1 and so on. The range of each USF value is 0-7. If there is no uplink TBF, or dynamic allocation was not used in the latest TBF configuration, xxxxxxxx is shown. If there is an uplink TBF but not all the 8 time slots are used, x is shown in place of the unused time slots. |

Display 06.02: Previous UL TBF establishment (Version: 3.15, Status: Approved)

+++++++++++++++++++++++++++

+ UL TBF establighment    +

+                         +

+ Estab cause   a(8)      +

+                         +

+ Channel req   b(5)      +

+                         +

+ Result of TBF c(7)      +

+ 		      +

+ Access type   d(7)      +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	a-field description corrected, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | GPDS | FTD_SB_MAC_UL_TBF_EST_CAUSE | S | R,I,O | The establishment cause of the previous TBF establishment: ONE-PHAS: one-phase access request SHORT: short access request TWO-PHAS: two-phase access request PAGE RES: paging response CELL UPD: cell update GMM SIGN: GMM or SM procedure SINGLE B: single block without TBF establishment These correspond the establishment causes in PACKET CHANNEL REQUEST. |
| b(5) | GPDS | FTD_SB_MAC_CH_REQ_TYPE | S | R,I,O | PRACH: if PACKET CHANNEL REQUEST was sent (PCCCH) RACH: if CHANNEL REQUEST was sent (CCCH) |
| e | GPDS | FTD_SB_MAC_RADIO_PRIORITY | B:D | R,I,O | Radio priority, range 1-4. |
| c(7) | GPDS | FTD_SB_MAC_TBF_EST_RES | S | R,I,O | Result of the TBF establishment: OK: TBF establishment succeeded. REJECT: IMM ASS REJECT (CCCH) or PACKET ACCESS REJECT (PCCCH) received from the network. TIMEOUT: No response to (P)RACH from the network. QUEUE: CS channel description received in IMM ASS (CCCH) or PACKET QUEUING NOTIFICATION received (PCCCH) and the TBF was not established (the final result may still be OK if the TBF was established correctly after queueing). FAILURE: TBF establishment failed due to any other reason. |
| d(7) | GPDS | FTD_SB_MAC_TBF_EST_REALIZED | S | R,I,O | The used access type, 1-PHASE or 2-PHASE access. This informs the finally used method, regardless of the requested access type. If the TBF establishment failed, xxxxxx is shown. |

Display 06.03: Information of the GMM state (Version: 5.0, Status: Approved)

+++++++++++++++++++++++++++

+ GMM state information   +

+                         +

+ NW oper mode  a(3)      +

+ GPRS attach   b(8)      +

+ GMM state     c         +

+ MS state      d(5)      +

+ READY timeout e(3)      +

+ T3312 timer   f(3)      +

+ T3312 / exp   g(3) h    +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Approved / Inspection 24-Jun-03, SWAM ADB TS03062563233

4.1	Proposal	h-field added and length of c-field corrected / ISA RM-RIM tool 9575, additional info

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, descriptions of c,d,f and g-fields corrected, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | GPDS | FTD_SB_GMM_NW_MODE | S | R,I,O | Network operation mode NW1 NW2 NW3 |
| b(8) | GPDS | FTD_SB_GMM_ATTACH_TYPE | S | R,I,O | The status of GPRS attach: (xxxxxxxx displayed in all these displays in case no GPRS attach is made) BOTH: Both IMSI and GPRS attach made GPRS: GPRS attach only |
| c | GPDS | FTD_SB_GMM_ATTACH_STATUS | S | R,I,O | The state of GMM c: main state of GMM X idle R registered D deregistered |
| d(5) | GPDS | FTD_SB_GMM_READY_TMR_STATUS |  | R,I,O | READY: MS is in READY state FORCE: MS is in standby state after receiving "force to standby" indication STAND: MS is in standby state (xxxx shown if in IDLE state) |
| e(3) | GPDS | FTD_SB_GMM_READY_TMR_TIMEOUT |  | R,I,O | The used timeout value for READY timer. Unit seconds. xxx if READY timer is deactivated |
| f(3) | GPDS | FTD_SB_GMM_RAU_TMR_CURRENT |  | R,I,O | Current value of T3312 timer, timer for periodical routing area updates. Range is 000 – 186 and unit is minutes. |
| g(3) | GPDS | FTD_SB_GMM_RAU_TMR_TIMEOUT |  | R,I,O | Timeout value of T3312 timer. Range 000 – 186 and unit is minutes. xxx if T3312 timer is deactivated |
| h | GPDS | FTD_SB_GMM_PREVENT_READY_EXPIRY | S | R,I,O | F if READY timer expiry preventation mechanism is activated, otherwise empty. |

Display 06.04: Values of P-TMSI, RAC, SMS radio priority, Ciphering and Non-DRX parameters  (Version: 3.15, Status: Approved)

+++++++++++++++++++++++++++

+ Ciphering and non-DRX   +

+ parameters              +

+                         +

+ P-TMSI        a(8)      +

+ RAI           b(4)      +

+ Ciphering     c(4)      +

+ SPLIT_PG_CYC  d(3)      +

+ Non-DRX timer e(2)      +

+ SMS radio pri f         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, description of d-field corrected, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | GPDS | FTD_SB_GMM_PTMSI_VALUE |  | R,I,O | P-TMSI value in hex format |
| b(4) | GPDS | FTD_SB_GMM_RAI_VALUE |  | R,I,O | RAI (Routing Area Indicator) in hex format |
| c(4) | GPDS | FTD_SB_GMM_CIPHERING_VALUE |  | R,I,O | Ciphering value (negotiated by GMM): OFF/GEA1/GEA2 Note! Even if the value here would show that ciphering is used, LLC may still send and receive unciphered blocks. |
| d(3) | GPDS | FTD_SB_GMM_SPLIT_PG_CYCLE_VALUE |  | R,I,O | SPLIT_PG_CYCLE value indicated by MS. 704 if non-DRX is used, otherwise range is 1-352. 0 is shown if SPLIT_PG_CYCLE is not in use in CCCH. |
| e(2) | GPDS | FTD_SB_GMM_NON_DRX_TMR |  | R,I,O | non-DRX timer indicated by MS. (The actually used value is a minimum of this and the network parameter.) 0 if no non-DRX mode is used after transfer mode, otherwise range is 1-64 seconds. |
| f | GPDS | FTD_SB_GMM_SMS_PRIORITY |  | R,I,O | SMS radio priority, range 1-4. If MO SMS via GPRS is not allowed, x is shown. |

Display 06.05: GPRS Network parameters  (Version: 3.15, Status: Approved)

+++++++++++++++++++++++++++

+ GPRS network parameters +

+                         +

+ CTRL_ACK type a(4)      +

+ ACCESS_BURST  b(2)      +

+ DRX_TIMER_MAX c(2)      +

+ SPLIT support d         +

+ Paging mode   e(2)      +

+ NC mode       f(3)      +

+ T3168 timer   g         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970 TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) |  | FTD_SB_MAC_CTRL_ACK_TYPE |  | R,I,O | The value of CONTROL_ACK_TYPE 4ACC: PACKET CONTROL ACKNOWLEDGEMENT is sent as 4 access bursts. CTRL: PACKET CONTROL ACKNOWLEDGEMENT is sent as RLC/MAC control block. |
| b(2) |  | FTD_SB_MAC_ACC_BURST_TYPE |  | R,I,O | The value of ACCESS_BURST_TYPE, either 8 or 11 [bits]. |
| c(2) |  | FTD_SB_MAC_DRX_TIMER_MAX |  | R,I,O | The value of DRX_TIMER_MAX, range 1 – 64 seconds. |
| d |  | FTD_SB_MAC_SPLIT_PG_CYCLE_ON_CCCH |  | R,I,O | The value of SPLIT_PG_CYCLE_CCCH_SUPPORTED, range 0-1. If PBCCH is supported, x is shown. |
| e(2) |  | FTD_SB_MAC_PAGE_MODE |  | R,I,O | Paging mode: NO: normal paging EX: extended paging RO: paging reorganization SB: same as before |
| f(3) |  | FTD_SB_MAC_NCMODE_BROADCAST |  | R,I,O | NC mode indicated in system information. If GPRS not supported, xxx is shown. NC0: No measurement reports, MS decides of the cell re-selections NC1: Measurement reports. MS decides of the cell re-selections NC2: Measurement reports. The network commands the cell re-selections. |
| g |  | FTD_SB_MAC_T3168_VALUE |  | R,I,O | The value of T3168 timer, range 0-7, corresponding 0.5s, 1s,…, 4s. |

Display 06.06: Packet control channel parameters (Version: 4.0, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.35/4.0	Approved	Approved / Inspection 16-May-03, AS03051652289

3.34	Proposal	Server to GPDS in a, b, c, d and e fields / PCP Errors, HJ01120531465

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, sub block id and server changed in most fields, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

If PCCCH is not supported, xxx is shown in all fields

+++++++++++++++++++++++++++

+ PCCCH parameters        +

+                         +

+ BS_PBCCH_BLKS       a   +

+ BS_PCC_CHANS        b(2)+

+ BS_PAG_BLCKS_RES    c(2)+

+ BS_PCC_REL          d   +

+ BS_PRACH_BLKS       e(2)+

+ Hopping of PBCCH    g   +

+ Hopping of PCCCH    h   +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | GPDS | FTD_SB_BS_PBCCH_BLKS |  | R,I,O | Value of BS_PBCCH_BLKS, range 1-4. |
| g | GSS | FTD_SB_PBCCH_HOPPING |  | R,I,O | Hopping of PBCCH H if PBCCH is hopping, otherwise empty |
| b(2) | GPDS | FTD_SB_BS_PCC_CHANS |  | R,I,O | Value of BS_PCC_CHANS, range 1-16. |
| h | GSS | FTD_SB_PCCCH_HOPPING |  | R,I,O | Hopping of PCCCH H if PCCCH is hopping, otherwise empty (also if BS_PCC_CHANS=1) |
| c(2) | GPDS | FTD_SB_BS_PAG_BLKS_RES |  | R,I,O | Value of BS_PAG_BLCKS_RES, range 0-12. |
| d | GPDS | FTD_SB_BS_PCC_REL |  | R,I,O | Value of BS_PCC_REL, range 0-1. |
| e(2) | GPDS | FTD_SB_BS_PRACH_BLKS |  | R,I,O | Value of BS_PRACH_BLKS, range 0-12. |

Display 06.07: (Packet) System information parameters (Version: 5.0, Status: Approved)

+++++++++++++++++++++++++++

+    System information   +

+                         +

+ Sys 13 / Sys 5  a(4)  b +

+ PSI repeat/ LR  c(2)d(2)+

+ PSI_HR/ Status  e(2)  f +

+ MSCR / SGSNR    g     h +

+ SI ind/NW EXT   i     j +

+ CCN / PFC mode  k     l +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Approved / Inspection 24-Jun-03, SWAM ADB TS03062563233

4.1	Proposal	i- j- k- and l-fields added and a note added to f-field description / ISA RM-RIM tool 9575, additional info

3.32	Approved	Approved / Inspection 06-Feb-03, SWAM ADB TS03020558808

3.31	Proposal	g and h fields added / ISA RM-RIM tool 009-2391

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, sub block id in a-field corrected, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | GSS | FTD_SB_PH_SI13_LOCATION |  | R,I,O | The location of System information 13. NORM: SI 13 available at BCCH Norm EXT: SI 13 available at BCCH Ext |
| b | GPDS | FTD_SB_MAC_PSI5_BROADCAST |  | R,I,O | The availability of Packet system information 5 (optional) 0: PSI 5 not supported 1: PSI 5 supported |
| c(2) | GPDS | FTD_SB_MAC_PSI1_REPEAT_PERIOD |  | R,I,O | The value of PSI_REPEAT_PERIOD, range 1-16. If PBCCH is not supported, xx is shown. |
| d(2) | GPDS | FTD_SB_MAC_PSI_COUNT_LR |  | R,I,O | The value of PSI_COUNT_LR, range 0-63. If PBCCH is not supported, xx is shown. |
| e(2) | GPDS | FTD_SB_MAC_PSI_COUNT_HR |  | R,I,O | The value of PSI_COUNT_HR, range 0-63. If PBCCH is not supported, xx is shown. |
| f | GPDS | FTD_SB_MAC_PSI_STATUS |  | R,I,O | The value of PSI_STATUS_IND, range 0-1. If 1, network supports the PACKET PSI_STATUS message. If PBCCH is not supported, x is shown. |
| g | GSS | FTD_SB_PH_MSCR | B:D | R,I,0 | Value of MSCR bit, range 0-1. If 0, MSC is Release ’98 or older, or info not availaible If 1, MSC is Release ’99 onwards |
| h | GSS | FTD_SB_PH_SGSNR | B:D | R;I;O | Value of SGSNR bit, range 0-1 If 0, SGSN is Release ’98 or older, or info not available If 1, SGSN is Release ’99 onwards |
| i | GSS | FTD_SB_PH_SI_STATUS_IND | B:D | R;I;O | The value of SI_STATUS_IND, range 0-1. If 1, network supports the PACKET SI_STATUS message. If PBCCH is supported, x is shown. |
| j | GSS | FTD_SB_PH_EXT_UTBF | B:D | R;I;O | The value of NW_EXT_UTBF, range 0-1. If 1, network supports the extended UL TBF mode (Rel4). |
| k | GSS | FTD_SB_PH_CCN_ACTIVE | B:D | R;I;O | The value of CCN_ACTIVE, range 0-1. If 1, network supports Cell Change Notification procedures (Rel4) |
| l | GSS | FTD_SB_PH_PFC_FEATURE_MODE | B:D | R;I;O | The value of PFC_FEATURE_MODE, range 0-1. If 1, network supports packet flow context procedures. |

Display 06.08: Cell re-selection parameters (serving cell) (Version: 3.15, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, server and sub block id fields filled, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

xx is shown in all fields if MS performs cell re-selections with GSM cell re-selection algorithm, i.e. if PBCCH is not supported and if neither PACKET MEAS ORDER nor PACKET CELL CHANGE ORDER has been received. xx is also shown in a specific field if the value is not available

+++++++++++++++++++++++++++

+ Cell re-selection param +

+                         +

+ RXLEV_ACCESS_MIN   a(2) +

+ CELL_RESELECT_HYS  b(2) +

+ RA_RESELECT_HYSTE  c(2) +

+ C31_HYST           d    +

+ C32_QUAL           e    +

+ RANDOM_ACC_RETRY   f    +

+ T_RESEL            g(4) +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GSS | FTD_SB_GPRS_RXLEV_ACC_MIN |  | R,I,O | The value of GPRS_RXLEV_ACCESS_MIN, range 0-63. |
| b(2) | GSS | FTD_SB_ GPRS_CELL_RESEL_HYST |  | R,I,O | The value of GPRS_CELL_RESELECT_HYSTERESIS, range 0-14 [dB]. |
| c(2) | GSS | FTD_SB_ RA_RESEL_HYST |  | R,I,O | The value of RA_RESELECT_HYSTERESIS, range 0-14 [dB]. |
| d | GSS | FTD_SB_ C31_HYST |  | R,I,O | The value of C31_HYST, range 0-1. |
| e | GSS | FTD_SB_ C32_QUAL |  | R,I,O | The value of C32_QUAL, range 0-1. |
| f | GSS | FTD_SB_ RAND_ACC_RETRY |  | R,I,O | The value of RANDOM_ACCESS_RETRY, range 0-1. |
| g(4) | GSS | FTD_SB_ T_RESEL |  | R,I,O | The value of T_RESEL, range 5-300 [seconds]. |

Display 06.09: GPRS information of the serving cell, 1st, 2nd, and 3rd neighbor (Version: 3.19, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.19	Approved	Sub block ids for a-,b-,c- and d-fields changed / Inspection 30-Sep-02, SWAM ADB TS02093045811

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, server and sub block id fields filled, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

2. row: serving cell information

4. row: 1. neighbor information

6. row: 2. neighbor information

8. row: 3. neighbor information

xx is shown in all fields if MS performs cell re-selections with GSM cell re-selection algorithm, i.e. if PBCCH is not supported and if neither PACKET MEAS ORDER nor PACKET CELL CHANGE ORDER has been received.

+++++++++++++++++++++++++++

+ GPRS Serving cell info  +

+ CH:    Rx:    PRI:  C32:+

+ aaa    bbb     c    ddd +

+ 1. Neighbor  info       +

+ eee    fff     g    hhh +

+ 2. Neighbor  info       +

+ iii    jjj     k    lll +

+ 3. Neighbor  info       +

+ mmm    nnn     o    ppp +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | GSS | FTD_SB_GPRS_CARRIER_SERV |  | R,I,O | Carrier number in decimal. |
| b(3) | GSS | FTD_SB_GPRS_RX_LEVEL_SERV |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100. |
| c | GSS | FTD_SB_PRIOR_CLASS_SERV |  | R,I,O | PRIORITY_CLASS, range 0-7. Shown only when C31>=0, otherwise left empty. If HCS parameters are not available (optional), x is shown. |
| d(3) | GSS | FTD_SB_C32_SERV |  | R,I,O | C32 value. For the serving cell, C32 equals C1. The neighbours are sorted first based on the priority class (biggest priority on top). Within each priority class the neighbours are sorted based on C32 value. The neighbors are sorted based on only C32, if all the C31 values are negative (c-fields empty) or if priorities are not available (x shown in c-fields). |
| e(3) | GSS | FTD_SB_GPRS_CARRIER_1_NEIGH |  | R,I,O | Carrier number in decimal. |
| f(3) | GSS | FTD_SB_GPRS_RX_LEVEL_1_NEIGH |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100. |
| g | GSS | FTD_SB_PRIOR_CLASS_1_NEIGH |  | R,I,O | PRIORITY_CLASS, range 0-7. Shown only when C31>=0, otherwise left empty. If HCS parameters are not available (optional), x is shown. |
| h(3) | GSS | FTD_SB_C32_1_NEIGH |  | R,I,O | C32 value. For the serving cell, C32 equals C1. The neighbours are sorted first based on the priority class (biggest priority on top). Within each priority class the neighbours are sorted based on C32 value. The neighbors are sorted based on only C32, if all the C31 values are negative (c-fields empty) or if priorities are not available (x shown in c-fields). |
| i(3) | GSS | FTD_SB_GPRS_CARRIER_2_NEIGH |  | R,I,O | Carrier number in decimal. |
| j(3) | GSS | FTD_SB_GPRS_RX_LEVEL_2_NEIGH |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100. |
| k | GSS | FTD_SB_PRIOR_CLASS_2_NEIGH |  | R,I,O | PRIORITY_CLASS, range 0-7. Shown only when C31>=0, otherwise left empty. If HCS parameters are not available (optional), x is shown. |
| l(3) | GSS | FTD_SB_C32_2_NEIGH |  | R,I,O | C32 value. For the serving cell, C32 equals C1. The neighbours are sorted first based on the priority class (biggest priority on top). Within each priority class the neighbours are sorted based on C32 value. The neighbors are sorted based on only C32, if all the C31 values are negative (c-fields empty) or if priorities are not available (x shown in c-fields). |
| m(3) | GSS | FTD_SB_GPRS_CARRIER_3_NEIGH |  | R,I,O | Carrier number in decimal. |
| n(3) | GSS | FTD_SB_GPRS_RX_LEVEL_3_NEIGH |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100. |
| o | GSS | FTD_SB_PRIOR_CLASS_3_NEIGH |  | R,I,O | PRIORITY_CLASS, range 0-7. Shown only when C31>=0, otherwise left empty. If HCS parameters are not available (optional), x is shown. |
| p(3) | GSS | FTD_SB_C32_3_NEIGH |  | R,I,O | C32 value. For the serving cell, C32 equals C1. The neighbours are sorted first based on the priority class (biggest priority on top). Within each priority class the neighbours are sorted based on C32 value. The neighbors are sorted based on only C32, if all the C31 values are negative (c-fields empty) or if priorities are not available (x shown in c-fields). |

Display 06.10: GPRS information of the 4th,5th and 6th neighbor (Version: 3.15, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Abbr notation changed, server and sub block id fields filled, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

2. row: 4. neighbor information

4. row: 5. neighbor information

6. row: 6. neighbor information

xx is shown in all fields if MS performs cell re-selections with GSM cell re-selection algorithm, i.e. if PBCCH is not supported and if neither PACKET MEAS ORDER nor PACKET CELL CHANGE ORDER has been received.

+++++++++++++++++++++++++++

+                         +

+ 4. Neighbor  info       +

+ CH:    Rx:    PRI:  C32:+

+ aaa    bbb     c    ddd +

+ 5. Neighbor  info       +

+ eee    fff     g    hhh +

+ 6. Neighbor  info       +

+ iii    jjj     k    lll +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | GSS | FTD_SB_GPRS_CARRIER_4_NEIGH |  | R,I,O | Carrier number in decimal. |
| b(3) | GSS | FTD_SB_GPRS_RX_LEVEL_4_NEIGH |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100. |
| c | GSS | FTD_SB_PRIOR_CLASS_4_NEIGH |  | R,I,O | PRIORITY_CLASS, range 0-7. Shown only when C31>=0, otherwise left empty. If HCS parameters are not available (optional), x is shown. |
| d(3) | GSS | FTD_SB_C32_4_NEIGH |  | R,I,O | C32 value. The neighbours are sorted first based on the priority class (biggest priority on top). Within each priority class the neighbours are sorted based on C32 value. The neighbors are sorted based on only C32, if all the C31 values are negative (c-fields empty) or if priorities are not available (x shown in c-fields). |
| e(3) | GSS | FTD_SB_GPRS_CARRIER_5_NEIGH |  | R,I,O | Carrier number in decimal. |
| f(3) | GSS | FTD_SB_GPRS_RX_LEVEL_5_NEIGH |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100. |
| g | GSS | FTD_SB_PRIOR_CLASS_5_NEIGH |  | R,I,O | PRIORITY_CLASS, range 0-7. Shown only when C31>=0, otherwise left empty. If HCS parameters are not available (optional), x is shown. |
| h(3) | GSS | FTD_SB_C32_5_NEIGH |  | R,I,O | C32 value. The neighbours are sorted first based on the priority class (biggest priority on top). Within each priority class the neighbours are sorted based on C32 value. The neighbors are sorted based on only C32, if all the C31 values are negative (c-fields empty) or if priorities are not available (x shown in c-fields). |
| i(3) | GSS | FTD_SB_GPRS_CARRIER_6_NEIGH |  | R,I,O | Carrier number in decimal. |
| j(3) | GSS | FTD_SB_GPRS_RX_LEVEL_6_NEIGH |  | R,I,O | Rx level in dBm, minus sign not shown if <=-100. |
| k | GSS | FTD_SB_PRIOR_CLASS_6_NEIGH |  | R,I,O | PRIORITY_CLASS, range 0-7. Shown only when C31>=0, otherwise left empty. If HCS parameters are not available (optional), x is shown. |
| l(3) | GSS | FTD_SB_C32_6_NEIGH |  | R,I,O | C32 value. The neighbours are sorted first based on the priority class (biggest priority on top). Within each priority class the neighbours are sorted based on C32 value. The neighbors are sorted based on only C32, if all the C31 values are negative (c-fields empty) or if priorities are not available (x shown in c-fields). |

Display 06.11: GPRS information of the serving cell and 6 neighbors  (Version: 3.16, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	Sub headers in table corrected, approved / Inspection 09-Aug-02, SWAM ADB TS02073044970

3.15	Proposal	11-Jul-02	AnttiE	Server, sub block id and mode information added, proposed / Corrections proposed by Tapio Saarinen

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

1. row: serving cell information

2. row: 1. neighbor on the left, 4. neighbor on the right

3. row: 2. neighbor on the left, 5. neighbor on the right

4. row: 3. neighbor on the left, 6. neighbor on the right

xx is shown in all fields if MS performs cell re-selections with GSM cell re-selection algorithm, i.e. if PBCCH is not supported and if neither PACKET MEAS ORDER nor PACKET CELL CHANGE ORDER has been received.

+++++++++++++++++++++++++++

+ Serving cell  a   c   d +

+ 1. neighbor   b   c   d +

+ 2. neighbor   b   c   d +

+ 3. neighbor   b   c   d +

+ 4. neighbor   b   c   d +

+ 5. neighbor   b   c   d +

+ 6. neighbor   b   c   d +

+  a=MS READY  b=Same RA  +

+  c=CELL_BAR  d=EXC_ACC  +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| Display row 1: serving cell information |  |  |  |  |  |
| a | GSS | FTD_SB_GMM_STATE |  | R,I,O | R if MS is in READY state and therefore applies hysteresis for cell re-selection. |
| c | GSS | FTD_SB_CELL_BAR_ACC_2 |  | R,I,O | The value of CELL_BAR_ACCESS_2. x is shown if PBCCH is not supported. |
| d | GSS | FTD_SB_EXC_ACC |  | R,I,O | The value of EXC_ACC. x is shown if PBCCH is not supported. |
| Display row 2, left side: 1st neighbor |  |  |  |  |  |
| b | GSS | FTD_SB_SAME_RA_AS_SRV_CELL_1_NEIGH |  | R,I,O | The value of SAME_RA_AS_SERVING_CELL: 1: cell belongs to the same routing area as the serving cell 0: cell belongs to the different routing area as the serving cell |
| c | GSS | FTD_SB_CELL_BAR_ACC_2_1_NEIGH |  | R,I,O | The value of CELL_BAR_ACCESS_2. x is shown if PBCCH is not supported. |
| d | GSS | FTD_SB_EXC_ACC_1_NEIGH |  | R,I,O | The value of EXC_ACC. x is shown if PBCCH is not supported. |
| Display row 3, left side: 2nd neighbor |  |  |  |  |  |
| b | GSS | FTD_SB_SAME_RA_AS_SRV_CELL_2_NEIGH |  | R,I,O | The value of SAME_RA_AS_SERVING_CELL: 1: cell belongs to the same routing area as the serving cell 0: cell belongs to the different routing area as the serving cell |
| c | GSS | FTD_SB_CELL_BAR_ACC_2_2_NEIGH |  | R,I,O | The value of CELL_BAR_ACCESS_2. x is shown if PBCCH is not supported. |
| d | GSS | FTD_SB_EXC_ACC_2_NEIGH |  | R,I,O | The value of EXC_ACC. x is shown if PBCCH is not supported. |
| Display row 4, right side: 3rd neighbor |  |  |  |  |  |
| b | GSS | FTD_SB_SAME_RA_AS_SRV_CELL_3_NEIGH |  | R,I,O | The value of SAME_RA_AS_SERVING_CELL: 1: cell belongs to the same routing area as the serving cell 0: cell belongs to the different routing area as the serving cell |
| c | GSS | FTD_SB_CELL_BAR_ACC_2_3_NEIGH |  | R,I,O | The value of CELL_BAR_ACCESS_2. x is shown if PBCCH is not supported. |
| d | GSS | FTD_SB_EXC_ACC_3_NEIGH |  | R,I,O | The value of EXC_ACC. x is shown if PBCCH is not supported. |
| Display row 2, right side: 4th neighbor |  |  |  |  |  |
| b | GSS | FTD_SB_SAME_RA_AS_SRV_CELL_4_NEIGH |  | R,I,O | The value of SAME_RA_AS_SERVING_CELL: 1: cell belongs to the same routing area as the serving cell 0: cell belongs to the different routing area as the serving cell |
| c | GSS | FTD_SB_CELL_BAR_ACC_2_4_NEIGH |  | R,I,O | The value of CELL_BAR_ACCESS_2. x is shown if PBCCH is not supported. |
| d | GSS | FTD_SB_EXC_ACC_4_NEIGH |  | R,I,O | The value of EXC_ACC. x is shown if PBCCH is not supported. |
| Display row 3, right side: 5th neighbor |  |  |  |  |  |
| b | GSS | FTD_SB_SAME_RA_AS_SRV_CELL_5_NEIGH |  | R,I,O | The value of SAME_RA_AS_SERVING_CELL: 1: cell belongs to the same routing area as the serving cell 0: cell belongs to the different routing area as the serving cell |
| c | GSS | FTD_SB_CELL_BAR_ACC_2_5_NEIGH |  | R,I,O | The value of CELL_BAR_ACCESS_2. x is shown if PBCCH is not supported. |
| d | GSS | FTD_SB_EXC_ACC_5_NEIGH |  | R,I,O | The value of EXC_ACC. x is shown if PBCCH is not supported. |
| Display row 4, right side: 6th neighbor |  |  |  |  |  |
| b | GSS | FTD_SB_SAME_RA_AS_SRV_CELL_6_NEIGH |  | R,I,O | The value of SAME_RA_AS_SERVING_CELL: 1: cell belongs to the same routing area as the serving cell 0: cell belongs to the different routing area as the serving cell |
| c | GSS | FTD_SB_CELL_BAR_ACC_2_6_NEIGH |  | R,I,O | The value of CELL_BAR_ACCESS_2. x is shown if PBCCH is not supported. |
| d | GSS | FTD_SB_EXC_ACC_6_NEIGH |  | R,I,O | The value of EXC_ACC. x is shown if PBCCH is not supported. |

Display 06.12: EGPRS specific parameters (Version: 3.24, Status: Approved)

+++++++++++++++++++++++++++

+    EGPRS parameters     +

+                         +

+ EDGE Supported      a   +

+ Packet Channel          +

+ request supported   b   +

+ BEP period or           +

+ Bep period2        c(2) +

+ Used link qual          +

+ measurement mode   d(2) +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.24	Approved	C-field length changed, approved / Inspection 11-Nov-02, SWAM ADB TS02111141480

3.23	Proposal	Replaced GPDS with GSS in a, b and c fields / SWAM ADB PL02101736947

Version	Status	Date	Author	Description

3.12	Approved	27-Jun-02	AnttiE	Approved / Inspection 14-Jun-02, HA02061839266

3.12	PROPOSAL	07-Jun-02	AnttiE	Display added / RM tool 5287

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | GSS | FTD_SB_EGPRS_SUPPORT | B:D | R,I,O | 1=EGPRS supported in cell; 0=not supported |
| b | GSS | FTD_SB_EGPRS_PACKET CHANNEL _REQUEST | B:D | R,I,O | 1=EGPRS packet channel request supported in cell; 0=not supported |
| c(2) | GSS | FTD_SB_EGPRS_BEP_PER | B:H | R,I,O | BEP_PERIOD, or BEP_PERIOD2 when received |
| d(2) | GPDS | FTD_SB_EGPRS_LQ_MEAS_MODE | ? | R,I,O | Used link quality measurement mode |

### Display 06.13: EGPRS BEP parameters (Version: 5.0, Status: Approved)

+++++++++++++++++++++++++++

+    EGPRS BEP parameters +

+                         +

+ GMSK Bit error          +

+ probapility          a  +

+ 8-PSK Bit error         +

+ probapility          b  +

+ GMSK Bit error          +

+ variation          c(2) +

+ 8-PSK Bit error         +

+ variation          d(2) +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Server added, approved / Inspection 24-Jun-03, SWAM ADB MM03061350494, TS03062563233

4.1	Rejected	Added. Rejected because servers are missing. / ISA RIM-RIM tool 010-1713

/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GPDS | FTD_SB_EGPRS_BEP_MEAN_GPSK | B:D | R | This field contains the mean value of the Bit Error Probability of the channel averaged over all time slots in the TBF for GMSK, refer to 3GPP TS 05.08. Range 0-31 |
| b | GPDS | FTD_SB_EGPRS_BEP_CV_GPSK | B:D | R | This field contains the mean value of the Bit Error Probability of the channel averaged over all time slots in the TBF for 8 PSK, refer to 3GPP TS 05.08. Range 0-7 |
| c(2) | GPDS | FTD_SB_EGPRS_BEP_MEAN_8PSK | B:D | R | This field contains the variation co-efficient for the Bit Error Probability averaged over all time slots of the TBF for GMSK, refer to 3GPP TS 05.08. Range 0-31 |
| d | GPDS | FTD_SB_EGPRS_BEP_CV_8PSK | B:D | R | This field contains the variation co-efficient for the Bit Error Probability averaged over all time slots of the TBF for 8 PSK, refer to 3GPP TS 05.08. Range 0-7 |

[/FTD_FIELDS]

- Group 07: GPRS Data protocol displays

Display 07.01: Information of the first  active PDP context (Version: 3.22, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.22	Approved	Approved / Inspection 15-Oct-02, SWAM ADB AS02101548729

3.21	Proposal	Note about field contents in 2G/3G products added / ISA RM-RIM tool 009-28

Version	Status	Date	Author	Description

1	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB AS02081437614

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

If no PDP context is active, xx is shown in all fields. In 2G/3G products, xx is shown in all fields except NSAPI and SAPI.

+++++++++++++++++++++++++++

+ 1. active PDP context   +

+                         +

+ NSAPI / SAPI   a(2) b   +

+ Relia / Delay  c    d   +

+ Prece / peak   e    f   +

+ Mean  / prior  g(2) h   +

+ VanJac/V.42bis i    j   +

+ PDP addr of PDP context +

+     k(3).l(3).m(3).n(3) +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | GPDS | FTD_SB_GPDS_NSAPI_1 |  | R,I,O | NSAPI of the PDP context, range 5-15. |
| b | GPDS | FTD_SB_GPDS_SAPI_1 |  | R,I,O | SAPI of the PDP context: 3,5,9 or B (=11) |
| c | GPDS | FTD_SB_SN_RELC_1 |  | R,I,O | The negotiated reliability class of the PDP context, range 1-5. |
| d | GPDS | FTD_SB_SN_DELC_1 |  | R,I,O | The negotiated delay class of the PDP context, range 1-4. |
| e | GPDS | FTD_SB_SN_PREC_1 |  | R,I,O | The negotiated precedence class of the PDP context, range 1-3 |
| f | GPDS | FTD_SB_SN_PTPC_1 |  | R,I,O | The negotiated peak throughput, range 1-9. |
| gg | GPDS | FTD_SB_SN_MTPC_1 |  | R,I,O | The negotiated mean throughput, range 1-18 or B (=best effort). |
| h | GPDS | FTD_SB_SN_RPRI_1 |  | R,I,O | The radio priority of the PDP context, range 1-4. |
| i | GPDS | FTD_SB_SN_HCOMP_1 |  | R,I,O | P if protocol compression (VanJacobsen) isnegotiated to be usedotherwise empty. |
| j | GPDS | FTD_SB_SN_DCOMP_1 |  | R,I,O | D if data compression (V.42bis) is negotiated to be used otherwise empty. |
| kkk | GPDS | FTD_SB_GPDS_PDP_ADDRESS_1_1 |  | R,I,O | The PDP address of the PDP context, part 1. Format IPv4 address. |
| lll | GPDS | FTD_SB_GPDS_PDP_ADDRESS_1_2 |  | R,I,O | The PDP address of the PDP context, part 2. Format IPv4 address. |
| mmm | GPDS | FTD_SB_GPDS_PDP_ADDRESS_1_3 |  | R,I,O | The PDP address of the PDP context, part 3. Format IPv4 address. |
| nnn | GPDS | FTD_SB_GPDS_PDP_ADDRESS_1_4 |  | R,I,O | The PDP address of the PDP context, part 4. Format IPv4 address. |

Display 07.02: Information of the second active PDP context (Version: 3.22, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.22	Approved	Approved / Inspection 15-Oct-02, SWAM ADB AS02101548729

3.21	Proposal	Note about field contents in 2G/3G products added / ISA RM-RIM tool 009-28

Version	Status	Date	Author	Description

1	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB AS02081437614

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

If no PDP context is active, xx is shown in all fields. In 2G/3G products, xx is shown in all fields except NSAPI and SAPI.

+++++++++++++++++++++++++++

+ 2. active PDP context   +

+                         +

+ NSAPI / SAPI   a(2) b   +

+ Relia / Delay  c    d   +

+ Prece / peak   e    f   +

+ Mean  / prior  g(2) h   +

+ VanJac/V.42bis i    j   +

+ PDP addr of PDP context +

+     k(3).l(3).m(3).n(3) +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | GPDS | FTD_SB_GPDS_NSAPI_2 |  | R,I,O | NSAPI of the PDP context, range 5-15. |
| b | GPDS | FTD_SB_GPDS_SAPI_2 |  | R,I,O | SAPI of the PDP context: 3,5,9 or B (=11) |
| c | GPDS | FTD_SB_SN_RELC_2 |  | R,I,O | The negotiated reliability class of the PDP context, range 1-5. |
| d | GPDS | FTD_SB_SN_DELC_2 |  | R,I,O | The negotiated delay class of the PDP context, range 1-4. |
| e | GPDS | FTD_SB_SN_PREC_2 |  | R,I,O | The negotiated precedence class of the PDP context, range 1-3 |
| f | GPDS | FTD_SB_SN_PTPC_2 |  | R,I,O | The negotiated peak throughput, range 1-9. |
| gg | GPDS | FTD_SB_SN_MTPC_2 |  | R,I,O | The negotiated mean throughput, range 1-18 or B (=best effort). |
| h | GPDS | FTD_SB_SN_RPRI_2 |  | R,I,O | The radio priority of the PDP context, range 1-4. |
| i | GPDS | FTD_SB_SN_HCOMP_2 |  | R,I,O | P if protocol compression (VanJacobsen) is negotiated to be used otherwise empty. |
| j | GPDS | FTD_SB_SN_DCOMP_2 |  | R,I,O | D if data compression (V.42bis) is negotiated to be used otherwise empty. |
| kkk | GPDS | FTD_SB_GPDS_PDP_ADDRESS_2_1 |  | R,I,O | The PDP address of the PDP context, part 1. Format IPv4 address. |
| lll | GPDS | FTD_SB_GPDS_PDP_ADDRESS_2_1 |  | R,I,O | The PDP address of the PDP context, part 2. Format IPv4 address. |
| mmm | GPDS | FTD_SB_GPDS_PDP_ADDRESS_2_3 |  | R,I,O | The PDP address of the PDP context, part 3. Format IPv4 address. |
| nnn | GPDS | FTD_SB_GPDS_PDP_ADDRESS_2_4 |  | R,I,O | The PDP address of the PDP context, part 4. Format IPv4 address. |

Display 07.03: Information of the RLC state (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

If no TBF is active, xx is shown in all fields.

+++++++++++++++++++++++++++

+ RLC state information   +

+ Downlink TS   a         +

+ Coding scheme b         +

+ RLC mode down c(c)      +

+ Uplink TS     d         +

+ Coding scheme e         +

+ RLC mode up   f(4)      +

+ UL TBF        g(g)      +

+ N3102 state   i(2)      +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | GPDS | FTD_SB_RLC_DL_SLOTS |  | R,I,O | Downlink time slot count, range 1-8. If no downlink TBF is active, x is shown. |
| b | GPDS | FTD_SB_RLC_DL_CH_CODE |  | R,I,O | Channel coding scheme in downlink direction, range 1-4. If no downlink TBF is active, x is shown. |
| cccc | GPDS | FTD_SB_RLC_DL_MODE |  | R,I,O | RLC mode in downlink direction. If no downlink TBF is active, xxxxx is shown. ACK: RLC in acknowledged mode UNAC: RLC in unacknowledged mode |
| d | GPDS | FTD_SB_RLC_UL_SLOTS |  | R,I,O | Upink time slot count, range 1-8. If no upink TBF is active, x is shown. |
| e | GPDS | FTD_SB_RLC_UL_CH_CODE |  | R,I,O | Channel coding scheme in uplink direction, range 1-4. If no uplink TBF is active, x is shown. |
| ffff | GPDS | FTD_SB_RLC_UL_MODE |  | R,I,O | RLC mode in uplink direction. If no uplink TBF is active, xxxx is shown. ACK: RLC in acknowledged mode UNAC: RLC in unacknowledged mode |
| ggggg | GPDS | FTD_SB_RLC_ENDING_MODE |  | R,I,O | xxxxx when no UL TBF is active CLOSE when UL TBF is close-ended TBF OPEN when UL TBF is open-ended TBF |
| ii | GPDS | FTD_SB_RLC_N3102 |  | R,I,O | The state of N3102 counter, range 0-PAN_MAX (negative values are also shown as 0). If the counter is disabled, x is shown. |

Display 07.04: RLC parameters (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB AS02081437614

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

xx is shown in the fields when the values are not available (e.g. no GPRS support in the cell, no GPRS attach performed).

+++++++++++++++++++++++++++

+ RLC parameters          +

+                         +

+ T3192 timer   a(2)      +

+ BS_CV_MAX     c(2)      +

+ PAN_DEC       d         +

+ PAN_INC       e         +

+ PAN_MAX       f(f)      +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | GPDS | FTD_SB_RLC_T3192 |  | R,I,O | The value of T3192 timer, range 0-7, corresponding 0.5s, 1s,…, 4s. |
| Bb | GPDS | FTD_SB_RLC_BS_CV_MAX |  | R,I,O | The value of BS_CV_MAX, range 1-15. Tells how many radio block periods the countdown procedure lasts in the end of UL TBF. |
| c | GPDS | FTD_SB_RLC_PAN_DEC |  | R,I,O | The value of PAN_DEC, range 0-7. If PAN_DEC and PAN_INC are equal to 0, N3102 counter is disabled. |
| d | GPDS | FTD_SB_RLC_PAN_INC |  | R,I,O | The value of PAN_INC, range 0-7. If PAN_DEC and PAN_INC are equal to 0, N3102 counter is disabled. |
| ee | GPDS | FTD_SB_RLC_PAN_MAX |  | R,I,O | The value of PAN_MAX, range 4-32. |

Display 07.05: RLC data block counters (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+ RLC data block counters +

+                         +

+ Received blocks    b(4) +

+ Sent blocks        d(4) +

+ Missing DL blocks  f(4) +

+ Req retrans uplink h(4) +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| bbbb | GPDS | FTD_SB_RLC_BLOCKS_RECV |  | R,I,O | Counter for received RLC data blocks |
| dddd | GPDS | FTD_SB_RLC_BLOCKS_SENT |  | R,I,O | Counter for sent RLC data blocks |
| ffff | GPDS | FTD_SB_RLC_BLOCKS_MISS |  | R,I,O | Counter for detected missing downlink data blocks |
| hhhh | GPDS | FTD_SB_RLC_BLOCKS_RESENT |  | R,I,O | Counter for requested re-transmissions of the uplink RLC data blocks |

Display 07.06: LLC data block counters (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+ LLC data block counters +

+                         +

+ Received                +

+ PDU'S         bbbb      +

+ Sent PDU’s    dddd      +

+ Missing                 +

+ downl PDU's   ffff      +

+ Re-trans                +

+ uplink PDU'S  hhhh      +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| bbbb | GPDS | FTD_SB_LLC_PDU_RECV |  | R,I,O | Counter for received LLC PDUs |
| dddd | GPDS | FTD_SB_LLC_PDU_SENT |  | R,I,O | Counter for sent LLC PDUs |
| ffff | GPDS | FTD_SB_LLC_PDU_MISS |  | R,I,O | Counter for detected missing or erroneous?? downlink LLC PDUs |
| hhhh | GPDS | FTD_SB_LLC_PDU_RESENT |  | R,I,O | Counter for re-transmitted uplink LLC PDUs |

Display 07.07: LLC Ciphering parameters (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+ LLC ciphering parameters+

+                         +

+ LSB of Kc      a(8)     +

+ MSB of Kc      b(8)     +

+ UL overflow I  c(4)     +

+ DL overflow I  d(4)     +

+ UL overflow UI e(4)     +

+ DL overflow UI f(4)     +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaaaaaa | GPDS | FTD_SB_LLC_KC1 |  | R,I,O | LSB of Kc (bits 0-31) in hex format |
| bbbbbbbb | GPDS | FTD_SB_LLC_KC2 |  | R,I,O | MSB of Kc (bits 32-63) in hex format |
| cccc | GPDS | FTD_SB_LLC_OC_I_UL |  | R,I,O | Uplink overflow counter for I frames |
| dddd | GPDS | FTD_SB_LLC_OC_I_DL |  | R,I,O | Downlink overflow counter for I frames |
| eeee | GPDS | FTD_SB_LLC_OC_UI_UL |  | R,I,O | Uplink overflow counter for UI frames |
| ffff | GPDS | FTD_SB_LLC_OC_UI_DL |  | R,I,O | Downlink overflow counter for UI frames |

Display 07.08: LLC parameters of the first SAPI (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB AS02081437614

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

xx is shown in all fields if there is no LLC SAPIs activated?

+++++++++++++++++++++++++++

+ LLC parameters 1. SAPI  +

+                         +

+ SAPI          a(2)      +

+ N201-I        b(4)      +

+ N201-U        c(4)      +

+ N200 / T200   d(2) e(4) +

+ kU   / kD     f(3) g(3) +

+ mU            h(5)      +

+ mD            I(5)      +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | GPDS | FTD_SB_GPDS_SAPI_1 |  | R,I,O | The SAPI identity: 3,5,9 or 11. |
| bbbb | GPDS | FTD_SB_LLC_N201_I_1 |  | R,I,O | The value of N201-I (maximum information field for I frames), range 140-1520. |
| cccc | GPDS | FTD_SB_LLC_N201_U_1 |  | R,I,O | The value of N201-U (maximum information field for U and UI frames), range 140-1520. |
| dd | GPDS | FTD_SB_LLC_N200_1 |  | R,I,O | The value of N200 (maximum number of retransmissions), range 1-15. |
| eeee | GPDS | FTD_SB_LLC_T200_1 |  | R,I,O | The value of T200 (retransmission timeout), range 1-4095. |
| fff | GPDS | FTD_SB_LLC_KU_1 |  | R,I,O | The value of kU (uplink window size), range 1-255. |
| ggg | GPDS | FTD_SB_LLC_KD_1 |  | R,I,O | The value of kD (downlink window size), range 1-255. |

Display 07.09: LLC parameters of the second SAPI (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB AS02081437614

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

xx is shown in all fields if there is no LLC SAPIs activated.

+++++++++++++++++++++++++++

+ LLC parameters 2. SAPI  +

+                         +

+ SAPI          a(2)      +

+ N201-I        b(4)      +

+ N201-U        c(4)      +

+ N200 / T200   d(2) e(4) +

+ kU   / kD     f(3) g(3) +

+ mU            h(5)      +

+ mD            I(5)      +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | GPDS | FTD_SB_GPDS_SAPI_2 |  | R,I,O | The SAPI identity: 3,5,9 or 11. |
| bbbb | GPDS | FTD_SB_LLC_N201_I_2 |  | R,I,O | The value of N201-I (maximum information field for I frames), range 140-1520. |
| cccc | GPDS | FTD_SB_LLC_N201_U_2 |  | R,I,O | The value of N201-U (maximum information field for U and UI frames), range 140-1520. |
| dd | GPDS | FTD_SB_LLC_N200_2 |  | R,I,O | The value of N200 (maximum number of retransmissions), range 1-15. |
| eeee | GPDS | FTD_SB_LLC_T200_2 |  | R,I,O | The value of T200 (retransmission timeout), range 1-4095. |
| fff | GPDS | FTD_SB_LLC_KU_2 |  | R,I,O | The value of kU (uplink window size), range 1-255. |
| ggg | GPDS | FTD_SB_LLC_KD_2 |  | R,I,O | The value of kD (downlink window size), range 1-255. |

Display 07.10: SNDC Data counters (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+ SNDC data counters      +

+                         +

+ Received NPDU’s    b(4) +

+ Sent NPDU’s        d(4) +

+ Aborted NPDU       f(4) +

+ Resent NPDU’s      h(4) +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| bbbb | GPDS | FTD_SB_SN_NPDU_RECV |  | R,I,O | Counter for received NPDUs |
| dddd | GPDS | FTD_SB_SN_NPDU_SENT |  | R,I,O | Counter for sent NPDUs |
| ffff | GPDS | FTD_SB_SN_NPDU_ABORT |  | R,I,O | Counter for aborted NPDU receptions |
| hhhh | GPDS | FTD_SB_SN_NPDU_RESENT |  | R,I,O | Counter for resent NPDUs |

Display 07.11: PPP information (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+ PPP information         +

+                         +

+ PPP HDLC FCS failures   +

+          bbbb           +

+ PPP max receive unit    +

+          dddd           +

+ PPP max transmit unit   +

+          ffff           +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB AS02081437614

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| bbbb | GPDS | FTD_SB_PDI_FCS_FAIL |  | R,I,O | Counter for PPP HDLC FCS failures |
| dddd | GPDS | FTD_SB_PDI_MRU |  | R,I,O | Negotiated PPP Maximum Receive Unit Value. |
| ffff | GPDS | FTD_SB_PDI_MTU |  | R,I,O | Negotiated PPP Maximum Transmit Unit Value. |

Display 07.12: ERLC Data information_1 (Version: 3.32, Status: Approved)

+++++++++++++++++++++++++++

+ ERLC Data information 1 +

+                         +

+ Used MCS UL	B(4)  +

+                         +

+ Used MCS DL	d(4)  +

+                         +

+ Resegmentation bit f    +

+ 		      +

+ Pre-emptive bit    h    +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.32	Approved	Changes were missing from b and d fields, corrected and approved / Inspection 06-Feb-03, SWAM ADB AS03020743614

3.31	Proposal	Format and coding of b and d fields changed / Inspection 06-Feb-03, SWAM ADB AE03013045174

Version	Status	Date	Author	Description

3.12	Approved	27-Jun-02	AnttiE	Approved / Inspection 14-Jun-02, HA02061839266

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 5287

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| b(4) | GPDS | FTD_SB_ERLC_MCS_UPLINK | S | R,I,O | Used MCS for uplink. Coding 1,2,3,4,5,6,7,8,9,3P,6P |
| d(4) | GPDS | FTD_SB_ERLC_MSC_DOWNLINK | S | R,I,O | Used MCS for downlink Coding 1,2,3,4,5,6,7,8,9,3P,6P |
| f | GPDS | FTD_SB_ERLC_RESEGMENT_BIT |  | R,I,O | Value of resegmentation bit |
| h | GPDS | FTD_SB_ERLC_PRE_EMPTIVE |  | R,I,O | Value of pre-emptive transmission bit |

Display 07.13: ERLC Data information_2 (Version: 3.12, Status: Approved)

+++++++++++++++++++++++++++

+ ERLC Data information 2 +

+                         +

+ Used window size for UL +

+          b(5)           +

+                         +

+                         +

+ Used window size for UL +

+          d(5)           +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.12	Approved	27-Jun-02	AnttiE	Approved / Inspection 14-Jun-02, HA02061839266

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 5287

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| bbbbb | GPDS | FTD_SB_ERLC_WINDOW_UPLINK |  | R,I,O | Used window size for uplink |
| ddddd | GPDS | FTD_SB_ERLC_WINDOW_DOWNLIN |  | R,I,O | Used window size for downlink |

Display 07.14: ERLC Data counters (Version: 3.12, Status: Approved)

+++++++++++++++++++++++++++

+ ERLC Data counters      +

+                         +

+ MS out of memory counter+

+          b(5)           +

+                         +

+                         +

+ MCS changes during TBF  +

+          d(5)           +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.12	Approved	27-Jun-02	AnttiE	Approved / Inspection 14-Jun-02, HA02061839266

3.12	Proposal	24-Apr-02	AnttiE	Display added / RM tool 5287

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| b(5) | GPDS | FTD_SB_ERLC_MS_OUT_OF_MEM |  | R,I,O | MS_OUT_OF_MEMORY counter |
| d(5) | GPDS | FTD_SB_ERLC_LINK_ADAPTATIONS |  | R,I,O | Number of MCS changes during TBF |

Display 07.15: System status (Version: 5.0, Status: Approved)

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Approved / Inspection 24-Jun-03, SWAM ADB TS03062563233

4.1	Proposal	h-field added and layout changed accordingly / ISA RM-RIM tool 9575, additional info

3.22	Approved	Approved / Inspection 15-Oct-02, SWAM ADB AS02101548729

3.21	Proposal	Display added / ISA RM-RIM tool 009-28

[/Version history]

In 2G only products, xx is shown in all counter fields.

+++++++++++++++++++++++++++

+      System status      +

+                         +

+ System status	a(2)  +

+ SMS sig req	b(5)  +

+ Data serv req     c(5)  +

+ GSM serv req	d(5)  +

+ Inter-sys HO	e(5)  +

+ Active PDP 	f(2)  +

+ Active CID	h(12) +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GPDS | FTD_SB_GPDS_ATT_AND_PDP | S | R,I,O | Status of the system: G if GSM, GP if attached to GSM and PDP context created, U if UMTS, UP if attached to UMTS and PDP context created, empty if not attached. |
| b(5) | GPDS | FTD_SB_GPDS_MSL_SMS_SERV_REQ_COUNTER | W:D | R,I,O | Counter for SMS signalling service requests. |
| c(5) | GPDS | FTD_SB_GPDS_MSL_DATA_SERV_REQ_COUNTER | W:D | R,I,O | Counter for data service requests. |
| d(5) | GPDS | FTD_SB_GPDS_MSL_GSM_SERV_REQ_COUNTER | W:D | R,I,O | Counter for GSM service requests. |
| e(5) | GPDS | FTD_SB_GPDS_MSL_INTSYS_HO_COUNTER | W:D | R,I,O | Counter for inter-system handovers. |
| f(2) | GPDS | FTD_SB_GPDS_CONTEXT_COUNT | B:D | R,I,O | Number of active PDP contexts, range 0-10. |
| h(12) | GPDS | FTD_SB_GPDS_ACTIVE_CIDS | S | R,I,O | Active CIDs (6 first) after each other, range 0-10 for each. Example when 4 active PDP contexts: “ 0 1 2 3”. |

Display 07.16: Information of the selected PDP context (Version: 5.0, Status: Approved)

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Approved / Inspection 24-Jun-03, SWAM ADB TS03062563233

4.1	Proposal	m-field added / ISA RM-RIM tool 9575, additional info

3.24	Approved	Fields roteted b->c, c->d, d->c to get data and help displays match / Inspection 11-Nov-02, SWAM ADB AE02110730362

3.22	Approved	Approved / Inspection 15-Oct-02, SWAM ADB AS02101548729

3.21	Proposal	Display added / ISA RM-RIM tool 009-28

[/Version history]

To change PDP context, perform following steps:

- Press the Options button.

- Select Change PDP context

- Press the Select button.

- “INPUT” prompt will be shown on the display, enter the PDP context identifier (CID) in display.

- Confirm with the Ok button.

If change succeeded, the new PDP context information is displayed. If no PDP context is active, xx is shown in all fields. In 2G only products, xx is shown in most of the fields.

+++++++++++++++++++++++++++

+ PDP context information +

+ Change PDP use options  +

+ CID / SAPI     a(2) b(2)+

+ NSAPI / be id  c(2) d(2)+

+ Flow id/Pr lev e(3) f   +

+ H Comp/D comp  g    h   +

+ PDP context address     +

+   I(3).j(3).k(3).l(3)   +

+ Primary CID    m(2)     +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | GPDS | FTD_SB_GPDS_CHANGE_CONTEXT | DW :D | R,I,O | Context identifier (CID), range 0-10. |
| a(2) | GPDS | FTD_SB_GPDS_CID | B:D | R,I,O | CID of the PDP context, range 0-10. |
| b(2) | GPDS | FTD_SB_GPDS_SAPI | B:D | R,I,O | SAPI of the PDP context: 1,3,5,7,9 or 11. |
| c(2) | GPDS | FTD_SB_GPDS_NSAPI | B:D | R,I,O | NSAPI of the PDP context, range 5-15. |
| d(2) | GPDS | FTD_SB_GPDS_MSL_RBID | B:D | R,I,O | The radio bearer ID of the PDP context, range 5-32. |
| e(3) | GPDS | FTD_SB_GPDS_MSL_PFID | B:D | R,I,O | The packet flow ID of the PDP context, range 0-127. |
| f | GPDS | FTD_SB_GPDS_MSL_RPRI | B:D | R,I,O | Radio priority level of the corresponding NSAPI, range 1-4. |
| g | GPDS | FTD_SB_GPDS_MSL_HCOMP | S | R,I,O | Header compression to be used: V - VanJacobsen (RFC1144), D - Degermark (RFC2507), R - RoHC (RFC3095), otherwise 'x' is shown. |
| h | GPDS | FTD_SB_GPDS_MSL_DCOMP | S | R,I,O | Data compression to be used: 2 - V.42bis 4 - V.44, otherwise 'x' is shown. |
| i(3) | GPDS | FTD_SB_GPDS_PDP_ADDRESS_V4_1 | B:D | R,I,O | First byte of the PDP address in IPv4 format. If IPv6 is used 'xxx' is shown. |
| j(3) | GPDS | FTD_SB_GPDS_PDP_ADDRESS_V4_2 | B:D | R,I,O | Second byte of the PDP address in IPv4 format. If IPv6 is used 'xxx' is shown. |
| k(3) | GPDS | FTD_SB_GPDS_PDP_ADDRESS_V4_3 | B:D | R,I,O | Third byte of the PDP address in IPv4 format. If IPv6 is used 'xxx' is shown. |
| l(3) | GPDS | FTD_SB_GPDS_PDP_ADDRESS_V4_4 | B:D | R,I,O | Last byte of the PDP address in IPv4 format. If IPv6 is used 'xxx' is shown. |
| m(2) | GPDS | FTD_SB_GPDS_RELATED PRIM_CID | B:D | R,I,O | Empty if primary context. CID of the related primary PDP context, range 0-10, if secondary context. |

Display 07.17: QoS of the selected PDP context 1 (Version: Charlie spec)

[Version history]

Version	Status	Description / Reference

3.22	Approved	Approved / Inspection 15-Oct-02, SWAM ADB AS02101548729

3.21	Proposal	Display added / ISA RM-RIM tool 009-28

[/Version history]

To change PDP context, perform following steps:

- Press the Options button.

- Select Change PDP context

- Press the Select button.

- “INPUT” prompt will be shown on the display, enter the PDP context identifier (CID) in display.

- Confirm with the Ok button.

If change succeeded, the new QoS information is displayed. If no PDP context is active, xx is shown in all fields.

+++++++++++++++++++++++++++

+  PDP context QoS info 1 +

+ Context ID         a(2) +

+ Trafic class       b    +

+ Erroneous SDU      c    +

+ Delivery order     d    +

+ Max SDU size       e(3) +

+ Bit error rate     f    +

+ SDU error rate     g    +

+ Max bit UL/ DL h(3) I(3)+

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | GPDS | FTD_SB_GPDS_CHANGE_CONTEXT | DW :D | R,I,O | Context identifier (CID), range 0-10. |
| a(2) | GPDS | FTD_SB_GPDS_CID | B:D | R,I,O | CID of the PDP context, range 0-10. |
| b | GPDS | FTD_SB_GPDS_SM_TRAC | B:D | R,I,O | The traffic class, range 1-4. |
| c | GPDS | FTD_SB_GPDS_SM_DOES | B:D | R,I,O | The delivery of the erroneous SDUs, range 1-3. |
| d | GPDS | FTD_SB_GPDS_SM_DELO | B:D | R,I,O | The delivery order, range 1-2. |
| e(3) | GPDS | FTD_SB_GPDS_SM_MSDU | B:D | R,I,O | Maximum SDU size, range 1-153. |
| f | GPDS | FTD_SB_GPDS_SM_RBER | B:D | R,I,O | The residual Bit Error Rate (BER), range 1-9. |
| g | GPDS | FTD_SB_GPDS_SM_SDER | B:D | R,I,O | The SDU error ratio, range 1-7. |
| h(3) | GPDS | FTD_SB_GPDS_SM_MBRU | B:D | R,I,O | Maximum bit rate for uplink, range 1-254. |
| i(3) | GPDS | FTD_SB_GPDS_SM_MBRD | B:D | R,I,O | Maximum bit rate for downlink, range 1-254. |

Display 07.18: QoS of the selected PDP context 2 (Version: Charlie spec)

+++++++++++++++++++++++++++

+  PDP context QoS info 1 +

+ Trafic priority    j    +

+ Transfer delay     k(2) +

+ Guar bit rate UL   l(3) +

+ Guar bit rate DL   m(3) +

+ Neg Rel class      n    +

+ Neg Del class      o    +

+ Neg preced calss   p    +

+ Peak / mean Thru  q r(2)+

+++++++++++++++++++++++++++

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| j | GPDS | FTD_SB_GPDS_SM_TRHP | B:D | R,I,O | The traffic handling priority, range 1-3. |
| k(2) | GPDS | FTD_SB_GPDS_SM_TRDL | B:D | R,I,O | The transfer delay, range 1-62. |
| l(3) | GPDS | FTD_SB_GPDS_SM_GBRU | B:D | R,I,O | Guaranteed bit rate for uplink, range 1-254. |
| m(3) | GPDS | FTD_SB_GPDS_SM_GBRD | B:D | R,I,O | Guaranteed bit rate for downlink, range 1-254. |
| n | GPDS | FTD_SB_GPDS_SM_RELC | B:D | R,I,O | The negotiated reliability class, range 1-5. |
| o | GPDS | FTD_SB_GPDS_SM_DELC | B:D | R,I,O | The negotiated delay class, range 1-4. |
| p | GPDS | FTD_SB_GPDS_SM_PREC | B:D | R,I,O | The negotiated precedence class, range 1-3 |
| q | GPDS | FTD_SB_GPDS_SM_PTPC | B:D | R,I,O | The negotiated peak throughput, range 1-9. |
| r(2) | GPDS | FTD_SB_GPDS_SM_MTPC | B:D | R,I,O | The negotiated mean throughput, range 1-31. |

+++++++++++++++++++++++++++Press the Menu button Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved) Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)Display 07.17: QoS of the selected PDP context (Version: 3.22, Status: Approved)

Display 07.19: PDCP parameters of the selected PDP context (Version: 3.22, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.22	Approved	Approved / Inspection 15-Oct-02, SWAM ADB AS02101548729

3.21	Proposal	Display added / ISA RM-RIM tool 009-28

[/Version history]

To change PDP context, perform following steps:

- Press the Options button.

- Select Change PDP context

- Press the Select button.

- “INPUT” prompt will be shown on the display, enter the PDP context identifier (CID) in display.

- Confirm with the Ok button.

If change succeeded, new PDCP parameters are displayed. If no PDCP radio bearer is active, xx is shown in all fields.

+++++++++++++++++++++++++++

+  PDCP parameters        +

+ Radio bearer ID    a    +

+ PDU presense       b    +

+ Loss SRNS supp     c    +

+ Max window size    d(5) +

+ Dis PDU recept     e(5) +

+ Dis PDU transm     f(5) +

+ S RX PDU/TX PDU    g(5) +

+ T RX PDU/TX PDU i(5)j(5)+

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | GPDS | FTD_SB_GPDS_CHANGE_CONTEXT | DW:D | R,I,O | Context identifier (CID), range 0-10. |
| a(2) | GPDS | FTD_SB_GPDS_PDCP_RBID | B:D | R,I,O | The radio bearer ID of the PDP context, range 5-32. |
| b | GPDS | FTD_SB_GPDS_PDCP_HEADER | B:D | R,I,O | Indicates the presence of the PDCP PDU header, range 0-1. |
| c | GPDS | FTD_SB_GPDS_PDCP_RELOC | B:D | R,I,O | Indicates whether the lossless SRNS relocation is supported, range 0-1. |
| d(5) | GPDS | FTD_SB_GPDS_PDCP_MAX_WS | W:D | R,I,O | Maximum window size, 0-65535. |
| e(5) | GPDS | FTD_SB_GPDS_PDCP_RECV_DISC | W:D | R,I,O | Number of discarded PDUs in reception. |
| f(5) | GPDS | FTD_SB_GPDS_PDCP_SENT_DISC | W:D | R,I,O | Number of discarded PDUs in transmission. |
| g(5) | GPDS | FTD_SB_GPDS_PDCP_SEQ_RECV | W:D | R,I,O | Number of PDUs received with sequence number. |
| h(5) | GPDS | FTD_SB_GPDS_PDCP_SEQ_SENT | W:D | R,I,O | Number of PDUs sent with sequence number. |
| i(5) | GPDS | FTD_SB_GPDS_PDCP_PDU_RECV | W:D | R,I,O | Total number of received PDUs. |
| j(5) | GPDS | FTD_SB_GPDS_PDCP_PDU_SENT | W:D | R,I,O | Total number of sent PDUs. |

Display 07.20: RFC2507 parameters of the selected PDP context (Version: 3.22, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.22	Approved	Approved / Inspection 15-Oct-02, SWAM ADB AS02101548729

3.21	Proposal	Display added / ISA RM-RIM tool 009-28

[/Version history]

To change PDP context, perform following steps:

- Press the Options button.

- Select Change PDP context

- Press the Select button.

- “INPUT” prompt will be shown on the display, enter the PDP context identifier (CID) in display.

- Confirm with the Ok button.

If change succeeded, new RFC2507 parameters are displayed. If RFC2507 is not active, xx is shown in all fields.

+++++++++++++++++++++++++++

+  PDCP parameters        +

+ Radio bearer ID    a(2) +

+ F_MAX_PERIOD       b(5) +

+ F_MAX_TIME         c(3) +

+ MAX_HEADER         d(5) +

+ EXPECT_REORDING    e    +

+ TCP_SPACE          f(3) +

+ NON_TCP_SPACE      g(5) +

+ RX Pac/TX Pac   h(5)i(5)+

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | GPDS | FTD_SB_GPDS_CHANGE_CONTEXT | DW :D | R,I,O | Context identifier (CID), range 0-10. |
| a(2) | GPDS | FTD_SB_GPDS_CID | B:D | R,I,O | CID of the PDP context, range 0-10. |
| b(5) | GPDS | FTD_SB_GPDS_RFC2507_MAX_PERIOD | W:D | R,I,O | RFC2507 parameter F_MAX_PERIOD, range 1-65535. |
| c(3) | GPDS | FTD_SB_GPDS_RFC2507_MAX_TIME | B:D | R,I,O | RFC2507 parameter F_MAX_TIME, range 1-255. |
| d(5) | GPDS | FTD_SB_GPDS_RFC2507_MAX_HEADER | W:D | R,I,O | RFC2507 parameter MAX_HEADER, range 60-65535. |
| e | GPDS | FTD_SB_GPDS_RFC2507_EXP_REORD | B:D | R,I,O | RFC2507 parameter EXPECT_REORDERING, range 0-1. |
| f(3) | GPDS | FTD_SB_GPDS_RFC2507_TCP_SPACE | B:D | R,I,O | RFC2507 parameter TCP_SPACE, range 3-255. |
| g(5) | GPDS | FTD_SB_GPDS_RFC2507_NON_TCP_SPACE | W:D | R,I,O | RFC2507 parameter NON_TCP_SPACE, range 3-65535. |
| h(5) | GPDS | FTD_SB_GPDS_RFC2507_CTX_STATE_RECV | W:D | R,I,O | Number of received context state packets. |
| i(5) | GPDS | FTD_SB_GPDS_RFC2507_CTX_STATE_SENT | W:D | R,I,O | Number of sent context state packets. |

- Group 08: GPRS Measurement displays

Xx is shown in all the fields when the values are not available (e.g. no GPRS support in the cell, no GPRS attach performed).

Display 08.01: Power control parameters (Version: 3.19, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Power control           +

+                         +

+ Alpha          aaa      +

+ TW / TT        bb  cc   +

+ PSI4 / PB      d   eee  +

+ PC channel     ffff     +

+ Ni value       gg       +

+ Gamma0-7    hh hh hh hh +

+             hh hh hh hh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Minor correction to b field description / Inspection 30-Sep-02, SWAM ADB TS02093045811

3.18	Proposal	Server, sub block id, format and mode columns completed / Tapio Saarinen

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | GSS | FTD_SB_ALPHA | B:D | R,I,O | The value of ALPHA, range 0.0-1.0. |
| b(2) | GSS | FTD_SB_T_AVG_W | B:D | R,I,O | The k value for T_AVG_W, range 0-25. |
| c(2) | GSS | FTD_SB_T_AVG_T | B:D | R,I.O | The k value for T_AVG_T, range 0-25. |
| d | GPDS | FTD_MAC_PSI4_BROADCAST | B:D | R,I.O | The value of INT_MEAS_CHANNEL_LIST_AVAIL, range 0-1. If 1, PSI4 is broadcasted. x is shown if PBCCH is not supported. |
| e(3) | GSS | FTD_SB_PB | B:D | R,I.O | The value of Pb, range 0 - -30 [dB]. xxx is shown if PBCCH is not supported. |
| f(4) | GSS | FTD_SB_PC_MEAS_CHAN | S | R,I.O | The value of PC_MEAS_CHAN, either BCCH or PDCH. |
| g(2) | GSS | FTD_SB_N_AVG_I | B:D | R,I.O | The k value for N_AVG_I, range 0-15. |
| h(2) x 8 | GSS | FTD_SB_GAMMA_TN0..7 | B:D | R,I.O | The values of GAMMA_TN0, GAMMA_TN1,…, GAMMA_TN7, range 0-62 [dB] on each. xx is shown in place of unused time slots. (Thus, in packet idle mode, all of them are xx.) |

Display 08.02: The previous channel quality report (Version: 3.20, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Prev channel quality    +

+                         +

+                         +

+ Dlack/RESreq   aaaaaaa  +

+ C value        bb       +

+ RX quality     c        +

+ Var value      dd       +

+ IF0-7       hh hh hh hh +

+             hh hh hh hh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.20	Approved	Approved / Inspection 30-Sep-02, SWAM ADB TS02093045811

3.18	Proposal	Server, sub block id, format and mode columns completed / Tapio Saarinen

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(7) | GPDS | FTD_SB_MAC_QREP_SENT_IN | S | R,I,O | DL_ACK if the previous channel quality report was sent in PACKET DOWNLINK ACK/NACK message. RES_REQ if the previous channel quality report was sent in PACKET RESOURCE REQUEST |
| b(2) | GPDS | FTD_SB_MAC_C_VALUE | B:D | R,I,O | The reported C value, range 0-63. |
| c | GPDS | FTD_SB_MAC_RXQUAL | B:D | R,I,O | The reported RXQUAL value, downlink quality, range 0-7. If the previous channel quality report was included in PACKET RESOURCE REQUEST, x is shown. |
| d(2) | GPDS | FTD_SB_MAC_SIGN_VAR | B:D | R,I.O | The reported SIGN_VAR value, range 0-63. xx is shown if the parameter was not included in the channel quality report (in two-phase access). |
| h(2) x 8 | GPDS | FTD_SB_MAC_QREP_I_TN0..7 | B:D | R,I.O | The reported I_LEVEL_TN0, I_LEVEL_TN1,…,I_LEVEL_TN7 values, range 0-63 on each. xx is shown if the corresponding interference level was not reported. |

Display 08.03: NC and EM parameters (Version: 1, Status: REJECTED)

Data Display:

+++++++++++++++++++++++++++

+ NC and EM parameters    +

+                         +

+ NC mode         a       +

+ N I T           b  C  d +

+ PSI5            eeee    +

+ Ord/Type/Period f  g  h +

+ P/O             iiii    +

+ NCC             jjj     +

+ IFfr            kkk     +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

xx is shown in all fields if PBCCH is not supported and if neither PACKET MEAS ORDER nor PACKET CELL CHANGE ORDER has been received.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a |  |  |  | R,I,O | The value of NETWORK_CONTROL_ORDER, range 0-2. |
| b |  |  |  |  | The value of NC_NON_DRX_PERIOD, range 0-7. |
| C |  |  |  |  | The value of NC_REPORTING_PERIOD_I, range 0-7. |
| d |  |  |  |  | The value of NC_REPORTING_PERIOD_T, range 0-7. |
| eeee |  |  |  |  | PSI5 if the current NC parameters were received in PSI5 message. ORDE if the current NC parameters were received in PACKET MEAS ODER or PACKET CELL CHANGE ORDER. |
| f |  |  |  |  | The value of EXT_MEASUREMENT_ORDER, range 0-1. |
| g |  |  |  |  | The value of EXT_REPORTING_TYPE, range 1-3. |
| h |  |  |  |  | The value of EXT_REPORTING_PERIOD, range 0-7. |
| iiii |  |  |  |  | PSI5 if the current EM parameters were received in PSI5 message. ORDE if the current EM parameters were received in PACKET MEAS ODER or PACKET CELL CHANGE ORDER. |
| jjj |  |  |  |  | The value of NCC_PERMITTED, range 0-255. xxx is shown if EXT_REPORTING_TYPE is not type 2. |
| kkk |  |  |  |  | The carrier number (in decimal) where INT_FREQUENCY refers. xxx is shown if INT_FREQUENCY is not broadcasted (optional). |

Display 08.04: The previous NC report, part 1 (Version: 1, Status: REJECTED)

Data Display:

+++++++++++++++++++++++++++

+ Prev NC report part1    +

+                         +

+ RxL / IFLev   aa   bb   +

+                         +

+ Carrier  BSIC  RxLevel  +

+ 1   ccc    dd      ee   +

+ 2   ccc    dd      ee   +

+ 3   ccc    dd      ee   +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

If the previous NC measurement report included less than 3 measurement results, xx is shown in the corresponding rows.

1.row 1.neighbor information

2.row 2.neighbor information

3.row 3.neighbor information

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa |  |  |  | R,I,O | The value of RXLEV_SERVING_CELL in the previous transmitted NC measurement report (PACKET MEASUREMENT REPORT), range 0-63. |
| bb |  |  |  |  | The value of INTERFERENCE_SERVING_CELL, range 0-63. xx is shown if the interference value was not reported. |
| ccc |  |  |  |  | Carrier number in decimal. |
| dd |  |  |  |  | BSIC value. xx is shown if BSIC was not reported. |
| ee |  |  |  |  | RXLEVEL value, range 0-63 or in dBm. |

Display 08.05: The previous NC report, part 2 (Version: 1, Status: REJECTED)

Data Display:

+++++++++++++++++++++++++++

+ Prev NC report part2    +

+                         +

+                         +

+ Carrier  BSIC  RxLevel  +

+ 4   ccc    dd      ee   +

+ 5   ccc    dd      ee   +

+ 6   ccc    dd      ee   +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

If the previous NC measurement report included less than 6 measurement results, xx is shown in the corresponding rows.

4.row 4.neighbor information

5.row 5.neighbor information

6.row 6.neighbor information

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| Ccc |  |  |  | R,I,O | Carrier number in decimal. |
| Dd |  |  |  |  | BSIC value. xx is shown if BSIC was not reported. |
| Ee |  |  |  |  | RXLEVEL value, range 0-63. |

Display 08.06: The previous EM report, part 1 (Version: 1, Status: REJECTED)

Data Display:

+++++++++++++++++++++++++++

+ Prev EM report part1    +

+                         +

+                         +

+ Carrier  BSIC  RxLevel  +

+ 1   ccc    dd      ee   +

+ 2   ccc    dd      ee   +

+ 3   ccc    dd      ee   +

+ 4   ccc    dd      ee   +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

If the previous EM measurement report included less than 4 measurement results, xx is shown in the corresponding rows.

1.row 1.neighbor information

2.row 2.neighbor information

3.row 3.neighbor information

4.row 4.neighbor information

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| ccc |  |  |  | R,I,O | Carrier number in decimal. |
| dd |  |  |  |  | BSIC value. xx is shown if BSIC was not reported. |
| ee |  |  |  |  | RXLEVEL value, range 0-63. |

Display 08.07: The previous EM report, part 2 (Version: 1, Status: REJECTED)

[Version history]

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

Data Display:

+++++++++++++++++++++++++++

+ Prev EM report part2    +

+                         +

+                         +

+ Carrier  BSIC  RxLevel  +

+ 5   ccc    dd      ee   +

+ 6   ccc    dd      ee   +

+                         +

+ IFLevel0-7 hh hh hh hh  +

+            hh hh hh hh  +

+++++++++++++++++++++++++++

If the previous EM measurement report included less than 6 measurement results, xx is shown in the corresponding rows.

5.row 5.neighbor information

6.row 6.neighbor information

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| ccc |  |  |  | R,I,O | Carrier number in decimal. |
| dd |  |  |  |  | BSIC value. xx is shown if BSIC was not reported. |
| ee |  |  |  |  | RXLEVEL value, range 0-63. |
| hh x 8 |  |  |  |  | The reported I_LEVEL_TN0, I_LEVEL_TN1,…,I_LEVEL_TN7 values, range 0-63 on each. xx is shown if the corresponding interference level was not reported. in dBm if possible |

Display 08.08: Downlink power control parameters (Version: 1, Status: REJECTED)

Data Display:

+++++++++++++++++++++++++++

+ Downlink power control  +

+                         +

+ BTS PWR CTRL    a       +

+ PO value        bb      +

+ PR value (A/B)  c       +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

xx is shown in the fields if there is no TBF active.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a |  |  |  | R,I,O | The value of BTS_PWR_CTRL_MODE, either A or B |
| bb |  |  |  |  | The value of P0, range 0-30 [dB]. |
| c |  |  |  |  | The value of PR mode: A: PR Mode A is used B: PR Mode B is used |

- Group 09: GPRS Test counter displays

Display 09.02: TBF counters (Version: 5.0, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ TBF counters            +

+                         +

+ UL packet idle    aaaaa +

+ UL TBF idle       bbbbb +

+ UL packet tr      ccccc +

+ UL TBF tr         ddddd +

+                         +

+ DL TBF idle       fffff +

+ UL abnormal       hhhhh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Approved / Inspection 24-Jun-03, SWAM ADB TS03062563233

4.1	Proposal	Completed / ISA RM-RIM tool 9575 additional info

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | GPDS | FTD_SB_MAC_UL_TBF_IDLE_ATTEMPT | W:D | R,I,O | Counter for UL TBF attempts in packet idle mode |
| b(5) | GPDS | FTD_SB_RLC_UL_TBF_IDLE_SUCC | W:D | R,I,O | Counter for successful UL TBFs, established in packet idle mode |
| c(5) | GPDS | FTD_SB_MAC_UL_TBF_TRANS_ATTEMPT | W:D | R,I,O | Counter for UL TBF attempts in packet transfer mode |
| d(5) | GPDS | FTD_SB_RLC_UL_TBF_TRANS_SUCC | W:D | R,I,O | Counter for successful UL TBFs, established in packet transfer mode |
| f(5) | GPDS | FTD_SB_MAC_DL_TBF_COUNTER | W:D | R,I,O | Counter for DL TBF establishments in packet idle mode |
| h(5) | GPDS | FTD_SB_RLC_UL_TBF_ABNORM_REL | W:D | R,I,O | Counter for UL TBF abnormal releases, any reason |

Display 09.10: GPRS attach and detach counters (Version: 5.0, Status:Approved)

Data Display:

+++++++++++++++++++++++++++

+ GPRS attach and detach  +

+                         +

+ Attach failure     aa   +

+ Attach attemps     bbb  +

+ Attach succeeded   cccc +

+ NI detach cause    hh   +

+ NI detach counter  iii  +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Completed / Inspection 24-Jun-03, SWAM ADB MH03061636985, TS03062642838, AE03063043175

4.1	Rejected	d, e, f and g fields removed / ISA RM-RIM 9575 additional info

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GPDS | FTD_SB_GMM_ATTACH_FAIL_CAUSE | B:H | R,I,O | Cause of the last attach failure |
| b(3) | GPDS | FTD_SB_GMM_ATTACH_ATTEMPT | W:D | R,I,O | Counter for the attach attempts |
| c(4) | GPDS | FTD_SB_GMM_ATTACH_OK | W:D | R,I,O | Counter for the succeededl attachs |
| h(2) | GPDS | FTD_SB_GMM_NTW_INIT_DET_CAUSE | B:H | R,I,O | Cause of the last network initiated detach |
| i(3) | GPDS | FTD_SB_GMM_NTW_INIT_DETACH | W:D | R,I,O | Counter for network initiated detach |

Display 09.11: Periodic routing area update counters (Version: 5.0, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Periodic routing area   +

+       update counters   +

+                         +

+ RA update cause    aa   +

+ RAU attemps        bbbb +

+ RAU succeeded      cccc +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Completed / Inspection 24-Jun-03, SWAM ADB TS03062642838, AE03063043175

4.1	Rejected	d, e, f and g fields removed / ISA RM-RIM 9575 additional info

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GPDS | FTD_SB_GMM_PRAU_FAIL_CAUSE | B:H | R,I,O | Cause of the last periodic routing area update failure |
| b(4) | GPDS | FTD_SB_GMM_PRAU_ATTEMPT | W:D | R,I,O | Counter for the periodic RAU attempts |
| c(4) | GPDS | FTD_SB_GMM_PRAU_OK | W:D | R,I,O | Counter for the succeeded periodic RAUs |

Display 09.12: Routing area update counters (Version: 5.0, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Routing area update     +

+ counters                +

+                         +

+ RA update cause    aa   +

+ RAU attemps        bbbb +

+ RAU succeeded      cccc +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Completed / Inspection 24-Jun-03, SWAM ADB TS03062642838, AE03063043175

4.1	Rejected	d, e, f and g fields removed / ISA RM-RIM 9575 additional info

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GPDS | FTD_SB_GMM_RAU_FAIL_CAUSE | B:H | R,I,O | Cause of the last routing area update failure |
| b(4) | GPDS | FTD_SB_RAU_ATTEMPT | W:D | R,I,O | Counter for the RAU attempts |
| c(4) | GPDS | FTD_SB_RAU_OK | W:D | R,I,O | Counter for the succeeded RAUs |

Display 09.13: PDP context counters (Version: 5.0, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ PDP context counters    +

+                         +

+ PDP act failure     aa  +

+ PDP act attemps     bbb +

+ PDP succeeded       ccc +

+ PDP MS init deact   gg  +

+ PDP net init deact  jj  +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Completed / Inspection 24-Jun-03, SWAM ADB TS03062642838, AE03063043175

4.1	Rejected	d, e, f, h, i, k and l fields removed / ISA RM-RIM 9575 additional infoVersion	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GPDS | FTD_SB_SM_MO_PDP_FAIL_CAUSE | B:H | R,I,O | Cause of the last MS initiated PDP context activation failure |
| b(3) | GPDS | FTD_SB_SM_MO_PDP_ATTEMPT | W:D | R,I,O | Counter for the MS initiated PDP context activation attempts |
| c(3) | GPDS | FTD_SB_SM_MO_PDP_OK | W:D | R,I,O | Counter for the succeeded MS initiated PDP context activations |
| g(2) | GPDS | FTD_SB_SM_MO_PDP_DEAC_CAUSE | B:H | R,I,O | Cause of the last MS initiated PDP context deactivation |
| j(2) | GPDS | FTD_SB_SM_MT_PDP_DEAC_CAUSE | B:H | R,I,O | Cause of the last Network initiated PDP context deactivation |

Display 09.14: SMS over GPRS counters (Version: 5.0, Status: Approved)

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Completed / Inspection 24-Jun-03, SWAM ADB TS03062642838, AE03063043175

4.1	Rejected	g, h, i, j and k fields removed / ISA RM-RIM 9575 additional info

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

The values in this display concern only SMS transferred via GPRS. Otherwise the SMS counters in GSM displays are valid.

Data Display:

+++++++++++++++++++++++++++

+ SMS over GPRS counters  +

+                         +

+ Send failure        aa  +

+ MO SMS attemps      bbb +

+ MO SMS send OK      ccc +

+ Receive failure     dd  +

+ MT SMS attemps      eee +

+ MT SMS receive OK   fff +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GSS | FTD_SB_SMS_SEND_FAIL_CAUSE_GPRS | B:H | R,I,O | Reason of last sending failure |
| b(3) | GSS | FTD_SB_SMS_MO_ATT_GPRS | W:D | R,I,O | Count of MO short message attempts via GPRS |
| c(3) | GSS | FTD_SB_SMS_MO_GPRS_OK | W:D | R,I,O | Count of succeeded MO short message sendings |
| d(2) | GSS | FTD_SB_SMS_REC_FAIL_CAUSE_GPRS | B:H | R,I,O | Reason of last receiving failure |
| e(3) | GSS | FTD_SB_SMS_MT_ATT_GPRS | W:D | R,I,O | Count of MT short message attempts via GPRS |
| f(3) | GSS | FTD_SB_SMS_MT_GPRS_OK | W:D | R,I,O | Count of succeeded MT short message receptions |

Display 09.17: MS initiated cell re-selection counters (Version: 5.0, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ MS init cell reselection+

+    counters             +

+                         +

+ CCCH  => CCCH      aaaa +

+ CCCH  => PCCCH     cccc +

+ PCCCH => CCCH      eeee +

+ PCCCH => PCCCH     gggg +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

5.0/4.2	Approved	Completed / ISA RM-RIM 9575, Inspection 24-Jun-03 SWAM ADB TS03062563233

Version	Status	Date	Author	Description

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | GPDS | FTD_SB_MAC_RESEL_C_C | W:D | R,I,O | Counter for cell re-selection attempts from CCCH to CCCH in NC0 or NC1 mode |
| c(4) | GPDS | FTD_SB_MAC_RESEL_C_P | W:D | R,I,O | Counter for cell re-selection attempts from CCCH to PCCCH in NC0 or NC1 mode |
| e(4) | GPDS | FTD_SB_MAC_RESEL_P_C | W:D | R,I.O | Counter for cell re-selection attempts from PCCCH to CCCH in NC0 or NC1 mode |
| g(4) | GPDS | FTD_SB_MAC_RESEL_P_P | W:D | R,I,O | Counter for cell re-selection attempts from PCCCH to PCCCH in NC0 or NC1 mode |

Display 09.18: Network initiated cell re-selection counters (Version: 5.0, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+    Network init cell    +

+    reselction counters  +

+                         +

+ CCCH  =>CCCH  aaaa bbbb +

+ CCCH  =>PCCCH cccc dddd +

+ PCCCH =>CCCH  eeee ffff +

+ PCCCH =>PCCCH gggg hhhh +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

Version	Status	Description / Reference

5.0/4.2	Approved	Completed / ISA RM-RIM 9575, Inspection 24-Jun-03 SWAM ADB TS03062563233

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | GPDS | FTD_SB_MAC_PCCO_C_C_ATT | W:D | R,I,O | Counter for network commanded cell re-selection attempts from CCCH to CCCH |
| b(4) | GPDS | FTD_SB_MAC_PCCO_C_C_OK | W:D | R,I,O | Counter for succeeded network commanded cell re-selections from CCCH to CCCH |
| c(4) | GPDS | FTD_SB_MAC_PCCO_C_P_ATT | W:D | R,I,O | Counter for network commanded cell re-selection attempts from CCCH to PCCCH |
| d(4) | GPDS | FTD_SB_MAC_PCCO_C_P_OK | W:D | R,I,O | Counter for succeeded network commanded cell re-selections from CCCH to PCCCH |
| e(4) | GPDS | FTD_SB_MAC_PCCO_P_C_ATT | W:D | R,I,O | Counter for network commanded cell re-selection attempts from PCCCH to CCCH |
| f(4) | GPDS | FTD_SB_MAC_PCCO_P_C_OK | W:D | R,I,O | Counter for succeeded network commanded cell re-selections from PCCCH to CCCH |
| g(4) | GPDS | FTD_SB_MAC_PCCO_P_P_ATT | W:D | R,I,O | Counter for network commanded cell re-selection attempts from PCCCH to PCCCH |
| h(4) | GPDS | FTD_SB_MAC_PCCO_P_P_OK | W:D | R,I,O | Counter for succeeded network commanded cell re-selections from PCCCH to PCCCH |

- Group 10: GSM DSP displays

Display 10.01: DSP Data Display 1 (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ DSP data display 1      +

+                         +

+ 1             aaaa      +

+ 2             bbbb      +

+ 3             cccc      +

+ 4             dddd      +

+ 5             eeee      +

+ 6             ffff      +

+ 7   8         gggg hhhh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSM_DSP | FTD_SB_DSP_DATA_1 |  | R | DSP specific data in hex format |
| bbbb | GSM_DSP | FTD_SB_DSP_DATA_2 |  | R | DSP specific data in hex format |
| cccc | GSM_DSP | FTD_SB_DSP_DATA_3 |  | R | DSP specific data in hex format |
| dddd | GSM_DSP | FTD_SB_DSP_DATA_4 |  | R | DSP specific data in hex format |
| eeee | GSM_DSP | FTD_SB_DSP_DATA_5 |  | R | DSP specific data in hex format |
| ffff | GSM_DSP | FTD_SB_DSP_DATA_6 |  | R | DSP specific data in hex format |
| gggg | GSM_DSP | FTD_SB_DSP_DATA_7 |  | R | DSP specific data in hex format |
| hhhh | GSM_DSP | FTD_SB_DSP_DATA_8 |  | R | DSP specific data in hex format |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

Display 10.02: DSP Data Display 2 (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ DSP data display 2       +

+                         +

+ 9             aaaa      +

+ 10            bbbb      +

+ 11            cccc      +

+ 12            dddd      +

+ 13            eeee      +

+ 14            ffff      +

+ 15   16       gggg hhhh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSM_DSP | FTD_SB_DSP_DATA_9 |  | R | DSP specific data in hex format |
| bbbb | GSM_DSP | FTD_SB_DSP_DATA_10 |  | R | DSP specific data in hex format |
| cccc | GSM_DSP | FTD_SB_DSP_DATA_11 |  | R | DSP specific data in hex format |
| dddd | GSM_DSP | FTD_SB_DSP_DATA_12 |  | R | DSP specific data in hex format |
| eeee | GSM_DSP | FTD_SB_DSP_DATA_13 |  | R | DSP specific data in hex format |
| ffff | GSM_DSP | FTD_SB_DSP_DATA_14 |  | R | DSP specific data in hex format |
| gggg | GSM_DSP | FTD_SB_DSP_DATA_15 |  | R | DSP specific data in hex format |
| hhhh | GSM_DSP | FTD_SB_DSP_DATA_16 |  | R | DSP specific data in hex format |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

Display 10.03: DSP Data Display 3 (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ DSP data display 3      +

+                         +

+ 17            aaaa      +

+ 18            bbbb      +

+ 19            cccc      +

+ 20            dddd      +

+ 21            eeee      +

+ 22            ffff      +

+ 23   24       gggg hhhh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | GSM_DSP | FTD_SB_DSP_DATA_17 |  | R | DSP specific data in hex format |
| bbbb | GSM_DSP | FTD_SB_DSP_DATA_18 |  | R | DSP specific data in hex format |
| cccc | GSM_DSP | FTD_SB_DSP_DATA_19 |  | R | DSP specific data in hex format |
| dddd | GSM_DSP | FTD_SB_DSP_DATA_20 |  | R | DSP specific data in hex format |
| eeee | GSM_DSP | FTD_SB_DSP_DATA_21 |  | R | DSP specific data in hex format |
| ffff | GSM_DSP | FTD_SB_DSP_DATA_22 |  | R | DSP specific data in hex format |
| gggg | GSM_DSP | FTD_SB_DSP_DATA_23 |  | R | DSP specific data in hex format |
| hhhh | GSM_DSP | FTD_SB_DSP_DATA_24 |  | R | DSP specific data in hex format |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

Display 10.04: FER measuments for sub ch0: REAL, FULL and SUB values (Version: 3.24, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ FER meas for sub ch0    +

+                         +

+ FER real             aa +

+ Correct FACCH        bb +

+ Erroneous TCH        cc +

+ Erroneous FACCH      dd +

+ Erroneous Full       ff +

+ Used frames          gg +

+ Erroneous frames     hh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.24	Approved	Approved / Inspection 11-Nov-02, SWAM ADB AE02111437929

3.23	Proposal	Display added / ISA RM-RIM tool 009-129

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GSM_DSP | FTD_SB_DSP_DATA_25 | B:D | R,I,O | FER REAL, a counter for correct frames including TCH, FACCH, SACCH |
| b(2) | GSM_DSP | FTD_SB_DSP_DATA_26 | B:D | R,I,O | FER REAL, a counter for correct FACCH frames |
| c(2) | GSM_DSP | FTD_SB_DSP_DATA_27 | B:D | R,I,O | FER REAL, a counter for erroneous TCH frames |
| d(2) | GSM_DSP | FTD_SB_DSP_DATA_28 | B:D | R,I,O | FER REAL, a counter for erroneous FACCH frames |
| f(2) | GSM_DSP | FTD_SB_DSP_DATA_29 | B:D | R,I,O | FER FULL, a counter for erroneous frames including TCH, SACCH, FACCH and SID frames |
| g(2) | GSM_DSP | FTD_SB_DSP_DATA_30 | B:D | R,I,O | FER SUB, a counter for used frames |
| h(2) | GSM_DSP | FTD_SB_DSP_DATA_31 | B:D | R,I,O | FER SUB, a counter for erroneous frames |

[/FTD_FIELDS]

These values are updated for every 480ms (One SACCH multiframe)

Display 10.05: FER measuments for sub ch1: REAL, FULL and SUB values (Version: 3.25, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ FER meas for sub ch1    +

+                         +

+ FER real             aa +

+ Correct FACCH        bb +

+ Erroneous TCH        cc +

+ Erroneous FACCH      dd +

+ Erroneous Full       ff +

+ Used frames          gg +

+ Erroneous frames     hh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.25	Approved	Approved / Inspection 11-Nov-02, SWAM ADB AE021114479493.24	Proposal	Display added / ISA RM-RIM tool 009-129 & Inspection 11-Nov-02, SWAM ADB AE02111357365

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GSM_DSP | FTD_SB_DSP_DATA_32 | B:D | R,I,O | FER REAL, a counter for correct frames including TCH, FACCH, SACCH |
| b(2) | GSM_DSP | FTD_SB_DSP_DATA_33 | B:D | R,I,O | FER REAL, a counter for correct FACCH frames |
| c(2) | GSM_DSP | FTD_SB_DSP_DATA_34 | B:D | R,I,O | FER REAL, a counter for erroneous TCH frames |
| d(2) | GSM_DSP | FTD_SB_DSP_DATA_35 | B:D | R,I,O | FER REAL, a counter for erroneous FACCH frames |
| f(2) | GSM_DSP | FTD_SB_DSP_DATA_36 | B:D | R,I,O | FER FULL, a counter for erroneous frames including TCH, SACCH, FACCH and SID frames |
| g(2) | GSM_DSP | FTD_SB_DSP_DATA_37 | B:D | R,I,O | FER SUB, a counter for used frames |
| h(2) | GSM_DSP | FTD_SB_DSP_DATA_38 | B:D | R,I,O | FER SUB, a counter for erroneous frames |

[/FTD_FIELDS]

These values are updated for every 480ms (One SACCH multiframe)

Display 10.06: FER measuments for sub ch2: REAL, FULL and SUB values (Version: 3.25, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ FER meas for sub ch2    +

+                         +

+ FER real             aa +

+ Correct FACCH        bb +

+ Erroneous TCH        cc +

+ Erroneous FACCH      dd +

+ Erroneous Full       ff +

+ Used frames          gg +

+ Erroneous frames     hh +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.25	Approved	Approved / Inspection 11-Nov-02, SWAM ADB AE02111447949

3.24	Proposal	Display added / ISA RM-RIM tool 009-129 & Inspection 11-Nov-02, SWAM ADB AE02111357365

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | GSM_DSP | FTD_SB_DSP_DATA_39 | B:D | R,I,O | FER REAL, a counter for correct frames including TCH, FACCH, SACCH |
| b(2) | GSM_DSP | FTD_SB_DSP_DATA_40 | B:D | R,I,O | FER REAL, a counter for correct FACCH frames |
| c(2) | GSM_DSP | FTD_SB_DSP_DATA_41 | B:D | R,I,O | FER REAL, a counter for erroneous TCH frames |
| d(2) | GSM_DSP | FTD_SB_DSP_DATA_42 | B:D | R,I,O | FER REAL, a counter for erroneous FACCH frames |
| f(2) | GSM_DSP | FTD_SB_DSP_DATA_43 | B:D | R,I,O | FER FULL, a counter for erroneous frames including TCH, SACCH, FACCH and SID frames |
| g(2) | GSM_DSP | FTD_SB_DSP_DATA_44 | B:D | R,I,O | FER SUB, a counter for used frames |
| h(2) | GSM_DSP | FTD_SB_DSP_DATA_45 | B:D | R,I,O | FER SUB, a counter for erroneous frames |

[/FTD_FIELDS]

These values are updated for every 480ms (One SACCH multiframe)

- Group 11: CDSP GSM AMR displays

Display 11.01: AMR1:Information of the Active Mode Set (Version: 3.16, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Channel type       aaaa +

+ Active mode set    bbbb +

+ Treshold 1         ccc  +

+ Hysteresis 1       dd   +

+ ICMI / ICM     e   f    +

+ Treshold 2/3   ggg jjj  +

+ Hysteresis 2/3 hh  kk   +

+ RATSCCH            ii   +

+ Noise suppression  l    +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	Server for e-field corrected, some fields removed, some explanation changes, formats still missing, approved / Inspection 09-Aug-02, SWAM ADB OA02071161215

3.15	Proposal	11-Jul-02	AnttiE	Many changes / RM-RIM tool 6239

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | GSS | FTD_SB_TYPE_OF_CURR_CH |  | R | Channel type |
| b(4) | GSM_DSP | FTD_SB_AMR1_MODESET |  | R | Active codec mode set , mode numbers in ACS are shown as digits 0 = 4.75 1 = 5.15 2 = 5.9 3 = 6.7 4 = 7.4 5 = 7.95 6 = 10.2 7 = 12.2 (1-4 digit hex without leading zeros) |
| c(3) | GSM_DSP | FTD_SB_AMR1_TH1 |  | R | first threshold, 999 if not in use |
| d(2) | GSM_DSP | FTD_SB_AMR1_HYS1 |  | R | first hysteresis, 99 if not in use |
| e | GSM_DSP | FTD_SB_AMR1_ICMI |  | R | ICMI (Initial codec mode indicator) 0=not signalled. 1=signalled. |
| f | GSM_DSP | FTD_SB_AMR1_ICM |  | R | Initial codec mode (ICM) If ICMI=0 mode selected by implicit rule If ICMI=1 signalled start mode used (shown as absolute mode 0-7) |
| g(3) | GSM_DSP | FTD_SB_AMR1_TH2 |  | R | Second threshold, 999 if not in use |
| h(2) | GSM_DSP | FTD_SB_AMR1_HYS2 |  | R | Second hysteresis, 99 if not in use |
| i(2) | GSM_DSP | FTD_SB_AMR1_RATSCCH |  | R | Count RATSCCH messages, truncated (wrap around overflow) to 8 bits (print two digit hex value) Counter is reset at call setup and HO. |
| j(3) | GSM_DSP | FTD_SB_AMR1_TH3 |  | R | third threshold, 999 if not in use |
| k(2) | GSM_DSP | FTD_SB_AMR1_HYS3 |  | R | third hysteresis, 99 if not in use |
| l | GSM_DSP | FTD_SB_AMR1_NSCB |  | R | Noise Suppression Control bit: 0=noise suppression can be used; 1=noise suppression is turned off |

[/FTD_FIELDS]

Data display contains the information from MS multirate information signalling. From 8 possible speech codec modes at most 4 belongs to active codec mode set. Active codecs are shown in display using mode numbers which are shown as digits (0..7). Modeset display has that many digits as there are modes in active set (1–4).  Highest mode is shown left so that also mode 0, if included in the ACS, is shown properly.

In addition to active codec mode set display shows thresholds and hysteresis values for mode changes according link quality. With 4 modes in active mode set three threshold/hysteresis values are needed. Number is always 1 less than number of codecs in mode set. If number of active modes is less than 4 unused threshold and hysteresis values are filled with 9's.

Even though the information needed for display is known by L3 on call setup  RATSCCH signalling may override all values. That is why display information is always fetched from DSP.

Display 11.02: AMR2: Information of the mode control (Version: 3.16, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Mode up/downlink  a   b +

+ Rx level/quality  ccc d +

+ Channel type      eeee  +

+ Mode command   ffff     +

+ Mode command   gggggggg +

+ Mode ind       hhhh     +

+ Mode ind       iiiiiiii +

+ Mode request   jjjj     +

+ Mode request   kkkkkkkk +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	Minor editorial changes / Inspection 09-Aug-02, SWAM ADB OA02071161215

3.15	Proposal	11-Jul-02	AnttiE	Sub block id for c field corrected and some changes in descriptions / RM-RIM tool 6239

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | GSM_DSP | FTD_SB_AMR_UL_MODE |  | R | Current absolute uplink mode within modeset (0-7) |
| b | GSM_DSP | FTD_SB_AMR_DL_MODE |  | R | Current absolute downlink mode within modeset (0-7) |
| c(3) | GSS | FTD_SB_RX_LEVEL |  | R | Rx level in dBm, minus sign not shown if <=-100 |
| d | GSS | FTD_SB_RX_QUALITY |  | R | Rx quality (sub), range is 0 - 7 |
| e(4) | GSS | FTD_SB_TYPE_OF_CURR_CH |  | R | Channel type |
| f(4) | GSM_DSP | FTD_SB_AMR2_MODE_CMD_HI |  | R | mode command history(Hex) |
| g(8) | GSM_DSP | FTD_SB_AMR2_MODE_CMD_LO |  | R | mode command history (Hex) |
| h(4) | GSM_DSP | FTD_SB_AMR2_MODE_IND_DL_HI |  | R | mode indication downlink history (Hex) |
| i(8) | GSM_DSP | FTD_SB_AMR2_MODE_IND_DL_LO |  | R | mode indication downlink history (Hex) |
| j(4) | GSM_DSP | FTD_SB_AMR2_MODE_REQ_HI |  | R | mode request history (Hex) |
| k(8) | GSM_DSP | FTD_SB_AMR2_MODE_REQ_LO |  | R | mode request history (Hex) |

[/FTD_FIELDS]

First line shows in compact form current channel type and basic quality information including up and downlink AMR modes. All information in the first row is available also from Display 01.01

Three history lines are used to print mode commands, mode indication to downlink direction and mode requests. Mode indication to uplink is not that interesting as it reflects mode commands after short delay. Mode requests might differ from mode indications downlink as BTS is not required to obey mode request. It should be noted that mode information in history lines in FTD is shown using absolute modes  (0–7) even though in actual inband transmission relative value within mode set is used (0–3).

With update rate 480 ms each digit in display corresponds mode related commands / indications in history.

DSP history algorithm can be modified to maintain history as different averages on the basis of the future needs without changing FTD display specification. See also executive  Display 11.04: AMR4: Control AMR FTD data pre processing (Version: 1, Status: Approved)

Display 11.03: AMR3: Information of the link quality estimation (Version: 3.16, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Mode up/down 	a   b +

+ Rx level/quality  ccc d +

+ Channel type      eeee  +

+ Link quality est  FFff  +

+ Frame erasure%    GGggg +

+ Fast fading       HHhh  +

+ Slow fading       IIii  +

+ Fading comp       Jj    +

+ Mode regs kkkk llllllll +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	Minor changes in display format (i and j fields) and explanations (a and b fields), formats still missing, approved / Inspection 09-Aug-02, SWAM ADB OA02071161215

3.15	Proposal	11-Jul-02	AnttiE	Sub block ids for a, b and c fields corrected and some some changes in field lengths and descriptions / RM-RIM tool 6239

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | GSM_DSP | FTD_SB_AMR_UL_MODE |  | R | Current absolute uplink mode (0-7) |
| b | GSM_DSP | FTD_SB_AMR_DL_MODE |  | R | Current absolute downlink mode (0-7) |
| c(3) | GSS | FTD_SB_RX_LEVEL |  | R | Rx level in dBm, minus sign not shown if <=-100 |
| d | GSS | FTD_SB_RX_QUALITY |  | R | Rx quality (sub), range is 0 - 7 |
| e(4) | GSS | FTD_SB_TYPE_OF_CURR_CH |  | R | Channel type |
| FFff | GSM_DSP | FTD_SB_AMR3_LQE |  | R | Current link quality estimate (dB) (floating point number) |
| GGggg | GSM_DSP | FTD_SB_AMR3_FRAME_ERASURE |  | R | frame erasure % (floating point number) |
| HHhh | GSM_DSP | FTD_SB_AMR3_FAST_FAD_FILTER |  | R | Output of the fast fading filter (floating point number) |
| IIii | GSM_DSP | FTD_SB_AMR3_SLOW_FAD_FILTER |  | R | slow fading filter output (floating point number) |
| Jj | GSM_DSP | FTD_SB_AMR3_FAD_COMP |  | R | fading compensation factor |
| k(4) | GSM_DSP | FTD_SB_AMR2_MODE_REQ_HI |  | R | mode request history (Hex) |
| l(8) | GSM_DSP | FTD_SB_AMR2_MODE_REQ_LO |  | R | mode request history (Hex) |

[/FTD_FIELDS]

First line is same as used in 'Display 11.02: AMR2: Information of the mode control (Version: 3.16, Status: Approved)'.

With update rate 480 ms each digit in display corresponds mode related commands / indication in history.

FFff, GGggg, HHhh, IIii and Jj represent floating point values. Decimal part is marked with lower case letters in this specification and decimal point is not printed

Display 11.04: AMR4: Control AMR FTD data pre processing (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ AMR FTD control         +

+                         +

+ XXXX                    +

+                         +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | GSM_DSP | FTD_SB_AMR4_CTRL_WORD | DW:H | R | Control word for DSP for controlling pre processing of AMR mode control related field test displays. |
| XXXX | GSM_DSP | FTD_SB_AMR4_CTRL_DISP |  | R | Control word for DSP for controlling pre processing of AMR mode control related field test displays. |

[/FTD_FIELDS]

When display is selected, control word can be edited. Keyboard is in "text"-mode, so hex numbers 0-F can be written. After editing of the control word is completed, request is sent to DSP.

Display 11.05: AMR5: Control AMR algorithms (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ AMR algorithm control   +

+                         +

+ XXXX                    +

+                         +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | GSM_DSP | FTD_SB_AMR5_CTRL_WORD | DW:H | R | Control word for DSP for controlling AMR algorithms. |
| XXXX | GSM_DSP | FTD_SB_AMR5_CTRL_ALGO |  | R | Control word for DSP for controlling AMR algorithms. |

[/FTD_FIELDS]

When display is selected, control word can be edited. Keyboard is in "text"-mode, so hex numbers 0-F can be written. After editing of the control word is completed, request is sent to DSP.

- Group 12: CDSP SW Platform

Display 12.01: CDSP version and date (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ CDSP version and date   +

+                         +

+ Version    aaaaaaaaaaaa +

+ Version    bbbbbbbbbbbb +

+ Version    cccccccccccc +

+ Date       dddddd       +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Display corrected & status updated / ADSP_CORE has got resource id & inspection 14-Jun-02, SWAM ADB MS02061047379

1	REJECTED	24-Apr-02	IHu	Original status: REJECTED

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(12) | ADSP_CORE | FTD_SB_ADSP_VERSION_1 | S | R | CDSP version string |
| b(12) | ADSP_CORE | FTD_SB_ADSP_VERSION_2 | S | R | CDSP version string |
| c(12) | ADSP_CORE | FTD_SB_ADSP_VERSION_3 | S | R | CDSP version string |
| d(6) | ADSP_CORE | FTD_SB_ADSP_VERSION_DATE | S | R | CDSP version date. Format: ddmmyy |

Display 12.02: Reset display (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

With this display all counters and error codes in this group can be reset.

Note: Does not reset PPCs

Data Display:

+++++++++++++++++++++++++++

+ Reset counters          +

+                         +

+                         +

+                         +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | MON | FTD_SB_ADSP_CLEAR_CNT | N/A | R | Clears all counters and error codes in this group. Note! This is not resetting any PPC counters. |

Display 12.03: CDSP error and reset counters and MCU reset time (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Sub block ids for fields a, b and c changed because the fields are PPCs / Inspection 14-Jun-02, SWAM ADB AE02061146579

1	Approved	24-Apr-2002	Ihu	Changed status to approved

[/Version history]

This display has counters for MCU originated CDSP resets, CDSP originated CDSP resets (fatal CDSP errors) and non-fatal CDSP errors. The display also holds the time of the last MCU originated CDSP reset.

Data Display:

+++++++++++++++++++++++++++

+ CDSP error and reset    +

+   and MCU reset time    +

+                         +

+ MCU resets         aaaa +

+ Fatal CDSP         bbbb +

+ Non fatal CDSP     cccc +

+ MCU reset time          +

+             ddddddddddd +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | MON | FTD_SB_PPC_MCU_GEN_ADSP_RESET | W:D | R | Number of MCU originated CDSP resets. |
| b(4) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_TOTAL | W:D | R | Number of CDSP originated CDSP resets (fatal CDSP errors). |
| c(4) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERRORS_TOTAL | W:D | R | Number of non-fatal CDSP errors. |
| d(11) | MON | FTD_SB_ADSP_MCU_RESET_DATE | S | R | Date and time of the last MCU originated CDSP reset. Format: ddmmyy hhmm |

[/FTD_FIELDS]

Note! These are not PPC counters.

Display 12.04: Counters for specific CDSP non-fatal errors (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-2002	IHu	Changed status to Approved.

[/Version history]

This display holds information about specific non-fatal errors in the CDSP.

Data Display:

+++++++++++++++++++++++++++

+ Specific non-fatal CDSP +

+                         +

+ CDSP unrec    aaaa      +

+ Signal error  bbbb      +

+ Res  Res      cccc dddd +

+ Res  Res      eeee ffff +

+ Res  Res      gggg hhhh +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | ADSP_CORE | FTD_SB_ADSP_SPECIFIC_ERR_CNT_1 | W:D | R | Counter for unrecognised messages received by the CDSP. |
| b(4) | ADSP_CORE | FTD_SB_ADSP_SPECIFIC_ERR_CNT_2 | W:D | R | Counter for errors when a signal is meant for its receiving process, but in different state of the process. |
| c(4) | ADSP_CORE | FTD_SB_ADSP_SPECIFIC_ERR_CNT_3 | W:D | R | Reserved. |
| d(4) | ADSP_CORE | FTD_SB_ADSP_SPECIFIC_ERR_CNT_4 | W:D | R | Reserved. |
| e(4) | ADSP_CORER | FTD_SB_ADSP_SPECIFIC_ERR_CNT_5 | W:D | R | Reserved. |
| f(4) | ADSP_CORE | FTD_SB_ADSP_SPECIFIC_ERR_CNT_6 | W:D | R | Reserved. |
| g(4) | ADSP_CORE | FTD_SB_ADSP_SPECIFIC_ERR_CNT_7 | W:D | R | Reserved. |
| h(4) | ADSP_CORE | FTD_SB_ADSP_SPECIFIC_ERR_CNT_8 | W:D | R | Reserved. |

[/FTD_FIELDS]

Note! These counters are not PPC counters.

Display 12.05: CDSP fatal error codes (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Sub block ids changed because fields are PPCs, formats added, approved / Inspection 14-Jun-02, SWAM ADB AE02061146579

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

This display holds four last fatal error codes. The display works in a FIFO principle. When a new error occurs all the errors are moved forward and if there are errors more than four, the oldest error code is removed. There are separate queues for fatal and non-fatal error codes. Also the ID of the last executed process is displayed after the error code. Codes and process IDs are in hexadecimal numbers.´

Data Display:

+++++++++++++++++++++++++++

+ CDSP fatal error codes  +

+                         +

+ Last 4 fatal    process +

+                         +

+ aaaaaaaa        bbb     +

+ cccccccc        ddd     +

+ eeeeeeee        fff     +

+ gggggggg        hhh     +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_ERROR_CODES _1 | DW:H | R | Last fatal error code. |
| b(3) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_ERROR_TASK_ID_1 | W:H | R | Last process running when the error occurred. |
| c(8) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_ERROR_CODES _2 | DW:H | R | Last – 1 fatal error code. |
| d(3) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_ERROR_TASK_ID_2 | W:H | R | Last process running when the error occurred. |
| e(8) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_ERROR_CODES _3 | DW:H | R | Last – 2 fatal error code. |
| f(3) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_ERROR_TASK_ID_3 | W:H | R | Last process running when the error occurred. |
| g(8) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_ERROR_CODES _4 | DW:H | R | Last – 3 fatal error code. |
| h(3) | MON | FTD_SB_PPC_ADSP_ORI_RESETS_ERROR_TASK_ID_4 | W:H | R | Last process running when the error occurred. |

3.12.Display 12.06: CDSP non-fatal error codes (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Sub block ids changed because fields are PPCs, formats added, approved / Inspection 14-Jun-02, SWAM ADB AE02061146579

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

This display holds four last non-fatal error codes. The display works in a FIFO principle. When a new error occurs all the errors are moved forward and if there are errors more than four, the oldest error code is removed. There are separate queues for fatal and non-fatal error codes. Also the ID of the last executed process is displayed after the error code. Codes and process IDs are in hexadecimal numbers.

Data Display:

+++++++++++++++++++++++++++

+ CDSP non-fatal errors   +

+                         +

+ Last 4 non-fat  process +

+                         +

+ aaaaaaaa        bbb     +

+ cccccccc        ddd     +

+ eeeeeeee        fff     +

+ gggggggg        hhh     +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERRORS_CODE_1 | DW :H | R | Last non-fatal error code. |
| b(3) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERROR_TASK_ID _1 | W :H | R | Last process running when the error occurred. |
| c(8) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERRORS_CODE_2 | DW :H | R | Last – 1 non-fatal error code. |
| d(3) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERROR_TASK_ID _2 | W:H | R | Last process running when the error occurred. |
| e(8) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERRORS_CODE_3 | DW :H | R | Last – 2 non-fatal error code. |
| f(3) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERROR_TASK_ID _3 | W:H | R | Last process running when the error occurred. |
| g(8) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERRORS_CODE_4 | DW:H | R | Last – 3 non-fatal error code. |
| h(3) | MON | FTD_SB_PPC_ADSP_NON_FATAL_ERROR_TASK_ID _4 | W:H | R | Last process running when the error occurred. |

3.12.Display 12.07: CDSP fatal error code timestamps (Version: 3.31, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.31	Approved	Missing formats added

Version	Status	Date	Author	Description

1	Approved	27-Jun-02	AnttiE	Status updated / Inspection 14-Jun-02, SWAM ADB AE02061146579

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

This display holds timestamps for fatal error codes in display 12.05.

Data Display:

+++++++++++++++++++++++++++

+ CDSP fatal timestamps   +

+                         +

+ date           time     +

+                         +

+ aaaaaa         bbbb     +

+ cccccc         dddd     +

+ eeeeee         ffff     +

+ gggggg         hhhh     +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | MON | FTD_SB_ADSP_LAST_FATAL_ERR_CODE_DATE_1 | S | R | Date of the last fatal error. Format: ddmmyy |
| b(4) | MON | FTD_SB_ADSP_LAST_FATAL_ERR_CODE_TIME_1 | S | R | Clock time of the last fatal error. Format:hhmm |
| c(6) | MON | FTD_SB_ADSP_LAST_FATAL_ERR_CODE_DATE_2 | S | R | Date of the last – 1 fatal error. Format: ddmmyy |
| d(4) | MON | FTD_SB_ADSP_LAST_FATAL_ERR_CODE_TIME_2 | S | R | Clock time of the last – 1 fatal error. Format: hhmm |
| e(6) | MON | FTD_SB_ADSP_LAST_FATAL_ERR_CODE_DATE_3 | S | R | Date of the last – 2 fatal error. Format: ddmmyy |
| f(4) | MON | FTD_SB_ADSP_LAST_FATAL_ERR_CODE_TIME_3 | S | R | Clock time of the last – 2 fatal error. Format: hhmm |
| g(6) | MON | FTD_SB_ADSP_LAST_FATAL_ERR_CODE_DATE_4 | S | R | Date of the last – 3 fatal error. Format: ddmmyy |
| h(4) | MON | FTD_SB_ADSP_LAST_FATAL_ERR_CODE_TIME_4 | S | R | Clock time of the last - 3 fatal error. Format: hhmm |

Display 12.08: CDSP non-fatal error code timestamps (Version: 3.31, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.31	Approved	Missing formats added

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

This display holds timestamps for non-fatal error codes in display 12.06.

Data Display:

+++++++++++++++++++++++++++

+ CDSP non-fatal timest.  +

+                         +

+ date           time     +

+                         +

+ aaaaaa         bbbb     +

+ cccccc         dddd     +

+ eeeeee         ffff     +

+ gggggg         hhhh     +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | MON | FTD_SB_ADSP_LAST_NON_FATAL_ERR_CODE_DATE_1 | S | R | Date of the last non-fatal error. Format: ddmmyy |
| b(4) | MON | FTD_SB_ADSP_LAST_NON_FATAL_ERR_CODE_TIME_1 | S | R | Clock time of the last non-fatal error. Format: hhmm |
| c(6) | MON | FTD_SB_ADSP_LAST_NON_FATAL_ERR_CODE_DATE_2 | S | R | Date of the last – 1 non-fatal error. Format: ddmmyy |
| d(4) | MON | FTD_SB_ADSP_LAST_NON_FATAL_ERR_CODE_TIME_2 | S | R | Clock time of the last – 1 non-fatal error. Format: hhmm |
| e(6) | MON | FTD_SB_ADSP_LAST_NON_FATAL_ERR_CODE_DATE_3 | S | R | Date of the last – 2 non-fatal error. Format: ddmmyy |
| f(4) | MON | FTD_SB_ADSP_LAST_NON_FATAL_ERR_CODE_TIME_3 | S | R | Clock time of the last – 2 non-fatal error. Format: hhmm |
| g(6) | MON | FTD_SB_ADSP_LAST_NON_FATAL_ERR_CODE_DATE_4 | S | R | Date of the last – 3 non-fatal error. Format: ddmmyy |
| h(4) | MON | FTD_SB_ADSP_LAST_NON_FATAL_ERR_CODE_TIME_4 | S | R | Clock time of the last - 3 non-fatal error. Format: hhmm |

Display 12.09: Counters for failed application starts (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	21-Aug-02	Antti E	Approved / Inspection 09-Aug-02, SWAM ADB MS02071156310

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

This display holds counters for failed application starts.

Data Display:

+++++++++++++++++++++++++++

+ Failed applicat. starts +

+                         +

+ Audio         aaaa      +

+ Voice         bbbb      +

+ Midi          cccc      +

+ Video         dddd      +

+                         +

+ Res  Res      eeee ffff +

+ Res  Res      gggg hhhh +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| Aaaa | ADSP_CORE | FTD_SB_ADSP_APP_START_FAIL_CNT_1 | W:D | R | Counter for failed audio application start requests. |
| bbbb | ADSP_CORE | FTD_SB_ADSP_APP_START_FAIL_CNT_2 | W:D | R | Counter for failed voice application start requests. |
| Cccc | ADSP_CORE | FTD_SB_ADSP_APP_START_FAIL_CNT_3 | W:D | R | Counter for failed midi application start requests. |
| Dddd | ADSP_CORE | FTD_SB_ADSP_APP_START_FAIL_CNT_4 | W:D | R | Counter for failed video application start requests. |
| Eeee | ADSP_CORE | FTD_SB_ADSP_APP_START_FAIL_CNT_5 | W:D | R | Reserved. |
| Ffff | ADSP_CORE | FTD_SB_ADSP_APP_START_FAIL_CNT_6 | W:D | R | Reserved. |
| Gggg | ADSP_CORE | FTD_SB_ADSP_APP_START_FAIL_CNT_7 | W:D | R | Reserved. |
| Hhhh | ADSP_CORE | FTD_SB_ADSP_APP_START_FAIL_CNT_8 | W:D | R | Reserved. |

Display 12.10: Common CDSP test data display (Version: 3.22, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.22	Approved	Changed to be an executive display / Inspection 15-Oct-02, SWAM ADB AE02101637189

Version	Status	Date	Author	Description

1	Approved	21-Aug-02	Antti E	Approved / Inspection 09-Aug-02, SWAM ADB MS02071156310

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

This display prompts for a number, which is then sent to the CDSP. The CDSP responds with four 12 character string, which are displayed by the FTD.

Data Display:

+++++++++++++++++++++++++++

+ Common CDSP test data   +

+                         +

+                         +

+ aaaaaaaaaaaa            +

+ bbbbbbbbbbbb            +

+ cccccccccccc            +

+ dddddddddddd            +

+                         +

+                         +

+++++++++++++++++++++++++++

To select the test data to be show, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 1210 (CDSP SW platform; Common CDSP test data display) to the query prompt.

- “INPUT” prompt will be shown on the display, ented data number.

- Confirm with the Ok button.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | ADSP_CORE | FTD_SB_ADSP_TEST_INDEX | DW:D | R | Select data to be shown by giving the index of the data. |
| a(12) | ADSP_CORE | FTD_SB_ADSP_TEST_DATA_1 | S | R | CDSP test data. |
| b(12) | ADSP_CORE | FTD_SB_ADSP_TEST_DATA_2 | S | R | CDSP test data. |
| c(12) | ADSP_CORE | FTD_SB_ADSP_TEST_DATA_3 | S | R | CDSP test data. |
| d(12) | ADSP_CORE | FTD_SB_ADSP_TEST_DATA_4 | S | R | CDSP test data. |

- Group 41: WCDMA CDSP displays

The displays in this group show CDSP specific information.

Display 41.01: RACH MSG TX profile (Version: 3.19, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.19	Approved	Approved / Inspection 30-Sep-02, SWAM ADB JT02092962030

3.18	Proposal	M-field encoding changed / Jukka Ta Toivanen

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

RACH MSG TX profile shows detailed information about latest RACH message transmission procedure.

Data Display:

+++++++++++++++++++++++++++

+ RACH MSG TX profile     +

+ Initial TxtxPower   aaa +

+ Po bbb Pp_m ccc SFN ddd +

+ A_slot ee   SubChan fff +

+ Lenght  g   Sign_m hhhh +

+ Sign_rnd iiii Pre   lll +

+ D_CH_G  k  C_CH_G j     +

+ Message m  data   n     +

+ Message tx power  ooo   +

+++++++++++++++++++++++++++

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | WCDMA_DSP_CS | FTD_WDSP_SB_INIT_TX_PWR | W:D | R,I,O | Initial transmission power, unit dBm. Value range –99 … +99. |
| b(3) | WCDMA_DSP_CS | FTD_WDSP_SB_TX_PO_PARAM | W:D | R,I,O | Po parameter, unit dBm. Value range –99 … +99. |
| c(3) | WCDMA_DSP_CS | FTD_WDSP_SB_TX_PP_M_PARAM | W:D | R,I,O | Pp_m parameter, unit dBm. Value range –99 … +99. |
| d(3) | WCDMA_DSP_CS | FTD_WDSP_SB_TX_START_TIMING | W:H | R,I,O | Base sfn for transmission start timing in hexadecimal format, value range 0x0 -0xFFF |
| e(2) | WCDMA_DSP_CS | FTD_WDSP_SB_TX_FIRST_ACCESS_SLOT | B:D | R,I,O | First used access slot in decimal format, value range 0 … 59 |
| f(3) | WCDMA_DSP_CS | FTD_WDSP_SB_TX_SUBCH_MASK | W:H | R,I,O | Subchannel mask in hexadecimal format, value range 0x0 … 0xFFF |
| g | WCDMA_DSP_CS | FTD_WDSP_SB_TX_MSG_LEN | B:D | R,I,O | Message length, two values: 1: 10 ms 2: 20 ms |
| h(4) | WCDMA_DSP_CS | FTD_WDSP_SB_TX_SIGSEL_MASK | W:H | R,I,O | Available signatures selection mask in hexadecimal format, value range 0x0 … 0xFFFF |
| I(4) | WCDMA_DSP_CS | FTD_WDSP_SB_TX_SIGSEL_RND | W:H | R,I,O | Signature selection random seed in hexadecimal format, value range 0x0 …0xFFFF |
| j | WCDMA_DSP_CS | FTD_WDSP_SB_TX_CTRL_CH_GAIN | B:H | R,I,O | Control channel gain in hexadecimal format, value range 0x0 … 0xF |
| k | WCDMA_DSP_CS | FTD_WDSP_SB_TX_DATA_CH_GAIN | B:H | R,I,O | Data channel gain in hexadecimal format, value range 0x0 … 0xF |
| l(3) | WCDMA_DSP_CS | FTD_WDSP_SB_TX_PREAMP_COUNT | W:H | R,I,O | Preamble transmission count in hexadecimal format, value range 0x0 …0xFFF |
| m | PN_WCDMA_DSP_CS | FTD_WDSP_SB_TX_MSG_RES | B:D | R,I,O | Message transmission result, three values: . 0: Message not transmitted 1: Message transmitted 2: Message transmission denied |
| n | PN_WCDMA_DSP_CS | FTD_WDSP_SB_TX_DATA_CH_SF | B:D | R,I,O | Data channel spreading factor, four values: 0: SF256 1: SF128 2: SF64 3: SF32 |
| o(3) | PN_WCDMA_DSP_CS | FTD_WDSP_SB_TX_MSG_PWR | W:D | R,I,O | Message transmission power, unit dBm. Value range –99 … +99. |

[/FTD_FIELDS]

Display 41.02: Dedicated uplink channel power control status (Version: 3.24, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.24	Approved	Approved / Inspection 11-Nov-02, SWAM ADB JT02111043773

3.23	Proposal	Coding for l- and m-fields changed / Jukka Ta. Toivanen

3.19	Approved	Approved / Inspection 30-Sep-02, SWAM ADB JT02092962030

3.18	Proposal	A, b and c field format changed. E-field encoding changed. N-field size and value range corrected. O- and p-field descriptions changed / Jukka Ta Toivanen

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display gives a quick overview of the current uplink dedicated channel power control status. From this display the operating point of the uplink power control can be seen

Data Display:

+++++++++++++++++++++++++++

+ Dedicated tx power info +

+ Tx min/max      aaa bbb +

+ Tx current      ccc     +

+ Algo e  step f  SSDT  g +

+ Tx loop      h  DPCCH i +

+ Comp mode    j  sync  k +

+ PhCh min l  PhCh max  m +

+ PhCh average      nnnnn +

+ Ul+ ooooo     Ul- ppppp +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | WCDMA_DSP_CS | FTD_SB_WDSP_TX_MIN_PWR | W:D | R,I,O | Minimum TX power during the display update period, unit dBm. Value range –99 … +99. |
| b(3) | WCDMA_DSP_CS | FTD_SB_WDSP_TX_MAX_PWR | W:D | R,I,O | Maximum TX power during the display update period, unit dBm. Value range –99 … +99. |
| c(3) | WCDMA_DSP_CS | FTD_SB_WDSP_TX_CUR_PWR | W:D | R,I,O | Current TX power at display update time, unit dBm. Value range –99 … +99. |
| e | WCDMA_DSP_CS | FTD_SB_WDSP_TX_PWR_CTRL_ALG | B:D | R,I,O | Current power control algorithm type, 1 = power control algorithm 1 2 = power control algorithm 2 |
| f | WCDMA_DSP_CS | FTD_SB_WDSP_TX_PWR_STEP | B:D | R,I,O | Current power control step size, unit dB, value range 1 .. 2. |
| g | WCDMA_DSP_CS | FTD_SB_WDSP_TX_SSDT | B:D | R,I,O | Current SSDT state values: 0: not active 1: active |
| h | WCDMA_DSP_CS | FTD_SB_WDSP_TX_DIV_MODE | B:D | R,I,O | TX closed loop diversity state, values: 0: not active 1: Closed loop mode 1 2: Closed loop mode 2 |
| I | WCDMA_DSP_CS | FTD_SB_WDSP_TX_DPCCH_FFORM | B:D | R,I,O | DPCCH frame format, value range 0 …5. |
| j | WCDMA_DSP_CS | FTD_SB_WDSP_TX_CM_MODE | B:D | R,I,O | Compressed mode used during display update period, values: 0: No compressed mode 1: Compressed mode active |
| k | WCDMA_DSP_CS | FTD_SB_WDSP_TX_OOS | B:D | R,I,O | Out of sync state visited during display update period, values: 0: No out of sync 1: Out of sync active |
| l | WCDMA_DSP_CS | FTD_SB_WDSP_TX_MIN_PHCH_BIT_RATE | B:H | R,I,O | Minimum PhCh bit rate used in the uplink frames during display update period. Value range is 0 …12, shown as hexadecimal number. Value coding is as follows: Range 0: 0 Range 1-6: 2^(L - 1) * 150 Range 8-12: (L - 6) * 9600 |
| m | WCDMA_DSP_CS | FTD_SB_WDSP_TX_MAX_PHCH_BIT_RATE | B:H | R,I,O | Maximum PhCh bit rate used in the uplink frames during display update period. Value range is 0 …12, shown as hexadecimal number. Value coding is as follows: Range 0: 0 Range 1-6: 2^(m - 1) * 150 Range 8-12: (m - 6) * 9600 |
| n(5) | WCDMA_DSP_CS | FTD_SB_WDSP_TX_AVR_PHCH_BIT_RATE | W:D | R,I,O | Average PhCH frame bit rate during display update period, value range 0 …57600 |
| o(5) | WCDMA_DSP_CS | FTD_SB_WDSP_TX_NUM_OF_UL_INC_PWR_COMMANDS | DW:D | R,I,O | Amount of derived "increase power" uplink power control commands during display update period. Value range 0 .. 99999. |
| p(5) | WCDMA_DSP_CS | FTD_SB_WDSP_TX_NUM_OF_UL_DEC_PWR_COMMANDS | DW:D | R,I,O | Amount of derived "decrease power" uplink power control commands during display update period. Value range 0 … 99999. |

[/FTD_FIELDS]

Display 41.03: Dedicated downlink channel power control status (Version: 3.19, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.19	Approved	Approved / Inspection 30-Sep-02, SWAM ADB JT02092962030

3.18	Proposal	Descriptions of all fields corrected / Jukka Ta Toivanen

Version	Status	Date	Author	Description

3.17	Approved	29-Aug-02	AnttiE	Sub block ids for d- and e-fields corrected / Inspection 14-Jun-02, SWAM ADB JL02080150458

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display gives a quick overview of the current downlik dedicated channel power control status. From this display the operating point of the downlink  power control can be seen

Data Display:

+++++++++++++++++++++++++++

+ Dedicated tx power info +

+                         +

+ SIR minimum        aaaa +

+ SIR maximum        bbbb +

+ SIR current        cccc +

+ Downlink increase ddddd +

+ Downlink decrease eeeee +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | WCDMA_DSP_CS | FTD_SB_WDSP_TARGET_SIR_MIN | W:D | R,I,O | Minimum target SIR value during display update period. Unit is dBm / 10. Value range is –320 … 310. |
| b(4) | WCDMA_DSP_CS | FTD_SB_WDSP_TARGET_SIR_MAX | W:D | R,I,O | Maximum target SIR value during display update period. Unit is dBm / 10 Value range is –320 … 310. |
| c(4) | WCDMA_DSP_CS | FTD_SB_WDSP_TARGET_SIR_CURRENT | W:D | R,I,O | Current target SIR at display update time. Unit is dBm / 10. Value range is –320 … 310. |
| d(5) | WCDMA_DSP_CS | FTD_SB_WDSP_NUM_OF_DL_INC_PWR_COMMANDS | DW:D | R,I,O | Amount of sent "increase power" downlink power control commands during display update period. Value range is 0 … 99999 |
| e(5) | WCDMA_DSP_CS | FTD_SB_WDSP_NUM_OF_DL_DEC_PWR_COMMANDS | DW:D | R,I,O | Amount of sent "decrease power" downlink power control commands during display update period. Value range is 0 … 99999 |

[/FTD_FIELDS]

Display 41.04: WCDMA AFC status (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

The purpose of the display is to monitor WCDMA AFC behavior (including aging algorithm).

Data Display:

+++++++++++++++++++++++++++

+ WCDMA AFC status        +

+                         +

+ AFC DAC init       aaaa +

+ AFC DAC current    bbbb +

+ AFC DAC minimum    cccc +

+ AFC DAC maximum    dddd +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | WCDMA_DSP_CS | FTD_SB_WDSP_AFC_INIT_DAC | W:D | R | Initial AFC DAC setting. Value range 0 to 2047. |
| b(4) | WCDMA_DSP_CS | FTD_SB_WDSP_AFC_CURR_DAC | W:D | R | Current AFC DAC setting. Value range 0 to 2047. |
| c(4) | WCDMA_DSP_CS | FTD_SB_WDSP_AFC_MIN_CURR_DAC | W:D | R | Minimum AFC DAC setting during display update period. Value range 0 to 2047. |
| d(4) | WCDMA_DSP_CS | FTD_SB_WDSP_AFC_MAX_CURR_DAC | W:D | R | Maximum AFC DAC setting during display update period. Value range 0 to 2047. |

[/FTD_FIELDS]

Display 41.05: CDSP load status (Version: 3.19, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.19	Approved	Left to group 41, approved / Inspection 30-Sep-02, SWAM ADB AE02100349561

3.18	Proposal	Probably in wrong group, but needs to be implemented anyway / Jukka Ta Toivanen

Version	Status	Date	Author	Description

3.13	Rejected	27-Jun-02	AnttiE	Display number changed. Rejected because display may be in a wrong group. / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display gives information about the CDSP processor load.

Data Display:

+++++++++++++++++++++++++++

+ CDSP load status        +

+ Act mode         aaaaa% +

+ very light sleep bbbbb% +

+ light sleep      ccccc% +

+ deep sleep       ddddd% +

+ Act mode         eeeeee +

+ very light sleep ffffff +

+ light sleep      gggggg +

+ deep sleep       hhhhhh +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | WCDMA_DSP_CS | FTD_SB_WDSP_LOAD_ACT_PORTION | W:D | R | Amount of time spent in active mode during display update period. Value is relative portion, range 0 ... 10000. |
| b(5) | WCDMA_DSP_CS | FTD_SB_WDSP_LOAD_VLS_PORTION | W:D | R | Amount of time spent in very light sleep mode during display update period. Value is relative portion, range 0 ... 10000. |
| c(5) | WCDMA_DSP_CS | FTD_SB_WDSP_LOAD_LS_PORTION | W:D | R | Amount of time spent in light sleep mode during display update period. Value is relative portion, range 0 ... 10000. |
| d(5) | WCDMA_DSP_CS | FTD_SB_WDSP_LOAD_DS_PORTION | W:D | R | Amount of time spent in deep sleep mode during display update period. Value is relative portion, range 0 ... 10000. |
| e(6) | WCDMA_DSP_CS | FTD_SB_WDSP_LOAD_ACT_CYCLES | DW:H | R | Amount of time spent in active mode during display update period. Value is sleep clock cycles (one cycle duration is approximately 30.5 s). |
| f(6) | WCDMA_DSP_CS | FTD_SB_WDSP_LOAD_VLS_CYCLES | DW:H | R | Amount of time spent in very light sleep mode during display update period. Value is sleep clock cycles (one cycle duration is approximately 30.5 s). |
| g(6) | WCDMA_DSP_CS | FTD_SB_WDSP_LOAD_LS_CYCLES | DW:H | R | Amount of time spent in light sleep mode during display update period. Value is sleep clock cycles (one cycle duration is approximately 30.5 s). |
| h(6) | WCDMA_DSP_CS | FTD_SB_WDSP_LOAD_DS_CYCLES | DW:H | R | Amount of time spent in deep sleep mode during display update period. Value is sleep clock cycles (one cycle duration is approximately 30.5 s). |

[/FTD_FIELDS]

Display 41.06: Downlink receiver gain control status (Version: 3.28, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.28	Approved	Format of g- and h-fields changed / Inspection 20-Dec-02, SWAM ADB JT02121948306

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Minor correction to help display, status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display gives information about WCDMA downlink receiver gain control operating point.

Data Display:

+++++++++++++++++++++++++++

+ Downlink rx gain status +

+                         +

+ Rx ana/digi      aa fff +

+ BB amp   bb  Gain pre c +

+ LNA state d  LNA bias e +

+ RSSI before FIR    gggg +

+ RSSI after FIR     hhhh +

+ Amp estim analog   iiii +

+ Amp estim digital  jjjj +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_ANA_AGC_DB_COUNT | B:D | R | Analog AGC's RX_dB counter. Value range 0…27. |
| b(2) | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_BB_AMP | B:D | R | Control word of BB amplifier. Value range 0…18. |
| c | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_PRE_AMP | B:D | R | Gain of preamplifier. 0 = minimum gain 1 = maximum gain |
| d | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_LNA | B:D | R | LNA state. 3 = LNA has maximum gain 1 = LNA has middle gain 0 = LNA has minimum gain |
| e | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_LNA_BIAS | B:H | R | LNA bias state. 1 = half bias current 0 = normal bias current |
| f(3) | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_DIGI_AGC_DB_COUNT | W:D | R | Digital AGC's RX_dB counter. Value range 0 … 511. |
| g(4) | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_RSSI_ANA_DBM | W:D | R | RSSI before FIR, range –240 … +20 (-120 dBm … +10 dBm) , unit dBm, format q1_16 |
| h(4) | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_RSSI_DIGI_DBM | W:D | R | RSSI after FIR, range –240 … +20 (-120 dBm … +10 dBm) unit dBm, format q1_16 |
| I(4) | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_ANA_AMP_EST | W:H | R | Amplitude estimate of analog AGC. Value range 0x0000…0xFFFF |
| j(4) | WCDMA_DSP_CS | FTD_SB_WDSP_RX_GAIN_DIGI_AMP_EST | W:H | R | Amplitude estimate of digital AGC. Value range 0x0000…0xFFFF |

[/FTD_FIELDS]

Display 41.07: WCDMA rake receiver status (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display gives information about WCDMA downlink rake receiver operating point

Data Display:

+++++++++++++++++++++++++++

+ WCDMA rake receiver     +

+                         +

+ Active BS             a +

+ Active PHCH           b +

+ Fingers               c +

+ Delay              dddd +

+ Master fingers       ee +

+ Finger alloc ffffffffff +

+ Finger algor gggggggggg +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | WCDMA_DSP_CS | FTD_SB_WDSP_RAKE_NUM_OF_ACTIVE_BS | B:D | R | Number of active BSs in rake. Value range 0 … 6. |
| b | WCDMA_DSP_CS | FTD_SB_WDSP_RAKE_NUM_OF_ACTIVE_PHCH | B:D | R | Number of active PHCHs in rake. Value range 0 … 3. |
| c | WCDMA_DSP_CS | FTD_SB_WDSP_RAKE_NUM_OF_USED_FINGERS | B:D | R | Number of allocated fingers i.e multipaths (filtered average over 50 frames). Value range 0 … 8. |
| d(4) | WCDMA_DSP_CS | FTD_SB_WDSP_RAKE_DELAY_SPREAD | W:D | R | Time difference of 1st and last multipath used for combining, value range 0 … 5120, unit 0.1 chips. |
| e(2) | WCDMA_DSP_CS | FTD_SB_WDSP_RAKE_ALLOCATED_MASTERS | B:H | R | Currently allocated master fingers. Bit #0 (mask 0x01) = Master finger 0 Bit #1 (mask 0x02) = Master finger 1 … Bit #7 (mask 0x80)= Master finger 7 |
| f(10) | WCDMA_DSP_CS | FTD_SB_WDSP_RAKE_ALLOC_COUNT | DW:D | R | Number of finger allocations done during display update period on current PhCHs. |
| g(10) | WCDMA_DSP_CS | FTD_SB_WDSP_RAKE_ALLOC_CALL_COUNT | DW:D | R | Number of finger allocation algorithm calls done during display update period on current PhCHs. |

[/FTD_FIELDS]

Display 41.08: WCDMA decoder status (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display gives a quick overview of the current decoder internal values regarding reception quality and sync status

Data Display:

+++++++++++++++++++++++++++

+ WCDMA decoder status    +

+                         +

+ TFCI forced        aaaa +

+ TFCI BER           bbbb +

+ BER cccc      BLER dddd +

+ Sync lost          eeee +

+ Sync status           f +

+ Spreading factor    ggg +

+ Slot format          hh +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | WCDMA_DSP_CS | FTD_SB_WDSP_DEC_FORCED_TFCI | W:H | R | Number of forced TFCI values in display update period. |
| b(4) | WCDMA_DSP_CS | FTD_SB_WDSP_DEC_TFCI_BER | W:H | R | TFCI BER averaged over display update period. |
| c(4) | WCDMA_DSP_CS | FTD_SB_WDSP_DEC_BER | W:H | R | BER used by the sync status algorithm (based on the TPC or the PILOT bits, depending on the situation). Averaged over display update period. |
| d(4) | WCDMA_DSP_CS | FTD_SB_WDSP_DEC_BLER | W:H | R | BLER in q12_16 format. This is the combined BLER for all the trch with BLER being measured, used by outer loop power control (averaged over display update period) |
| e(4) | WCDMA_DSP_CS | FTD_SB_WDSP_DEC_SYNC_LOST | W:H | R | Number of times the phone lost sync since the activation of the display. |
| f | WCDMA_DSP_CS | FTD_SB_WDSP_DEC_SYNC_STATUS | B:D | R | SYNC sync status: 0 out of sync 1 in sync |
| g(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DEC_SPREADING_FACTOR | B:D | R | Spreading factor |
| h(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DEC_SLOT_FORMAT | B:D | R | Slot format used in the physical channel |

[/FTD_FIELDS]

Display 41.09: Common WCDMA DSP CS scratchpad display (interactive) (Version: 3.19, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.19	Approved	Approved / Inspection 30-Sep-02, SWAM ADB JT02092962030

3.18	Proposal	String of 48 bytes splitted to four 12 byte strings / Implementation problem, Marko Haajo

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB JT02071462724

3.15	Proposal	11-Jul-02	AnttiE	Corrections proposed

3.13	Rejected	27-Jun-02	AnttiE	Display number changed / Inspection 14-Jun-02

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display provides a generic display template that can be used to display any data on field test display. This display is needed in order to be able to support for the various needs detected in field testing. With this display quick test-specific temporary displays can be easily added to support specific problems detected in field testing.The display prompts for an index and then shows the data associated with the given index. Use the following steps to activate this display. The display is not activated if the arrow () buttons are used.

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display by giving the display number 4109 to the query prompt.

- Test input prompt will be shown on the display. Enter scratchpad index.

- Confirm with the Ok button.

- Data associated with the given index is shown if the activation was successful.

Data Display:

+++++++++++++++++++++++++++

+ Interactive CDSP        +

+ scratchpad display      +

+                         +

+ aaaaaaaaaaaa            +

+ bbbbbbbbbbbb            +

+ cccccccccccc            +

+ dddddddddddd            +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | WCDMA_DSP_CS | FTD_SB_WDSP_SCRATCHPAD_INDEX | DW:D | R | Scratchpad index, see project specific documentation. |
| a(12) | WCDMA_DSP_CS | FTD_SB_WDSP_SCRATCHPAD_DATA1 | S | R | FTD data 0 … 12 character string |
| b(12) | WCDMA_DSP_CS | FTD_SB_WDSP_SCRATCHPAD_DATA2 | S | R | FTD data 0 … 12 character string |
| c(12) | WCDMA_DSP_CS | FTD_SB_WDSP_SCRATCHPAD_DATA3 | S | R | FTD data 0 … 12 character string |
| d(12) | WCDMA_DSP_CS | FTD_SB_WDSP_SCRATCHPAD_DATA4 | S | R | FTD data 0 … 12 character string |

[/FTD_FIELDS]

Display 41.10: FDD neighbour cell summary (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This displays summarises the number of cells in the active, monitored, detected and undetected  sets on the 3 frequencies. Active and detected sets only contain cells on the home frequency

Data Display:

+++++++++++++++++++++++++++

+ FDD neighbour cell info +

+ Active cells         aa +

+ Intra cells          bb +

+ Inter 1 freq         cc +

+ Inter 2 freq         dd +

+ Detected cells       ee +

+ Intra cells undetect ff +

+ Inter 1 freq undet   gg +

+ Inter 2 freq undet   hh +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_SUM_INTRA_ACTIVE | W:D | R,I,O | Number of cells in the active set |
| b(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_SUM_INTRA_MON | W:D | R,I,O | Number of cells in intra-freq monitired set |
| c(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_SUM_INTER1_MON | W:D | R,I,O | Number of cells in Inter 1 freq monitored set |
| d(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_SUM_INTER2_MON | W:D | R,I,O | Number of cells in Inter 2 freq monitored set |
| e(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_SUM_INTRA_DET | W:D | R,I,O | Number of cells in detected set |
| f(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_SUM_INTRA_UNDET | W:D | R,I,O | Number of cells in Intra freq neighbour list that are undetected |
| g(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_SUM_INTER1_UNDET | W:D | R,I,O | Number of cells in Inter1 freq neighbour list that are undetected |
| h(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_SUM_INTER2_UNDET | W:D | R,I,O | Number of cells in Inter2 freq neighbour list that are undetected |

[/FTD_FIELDS]

Display 41.11: FDD ranking summary (Version: 3.24, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.24	Approved	Approved / Inspection 11-Nov-02, SWAM ADB JT02111043773

3.23	Proposal	Coding for I-, j-, k- and l-fields changed / Jukka Ta. Toivanen

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display shows the 4 highest ranked cells. The ranking criteria (RSCP, EcNo) will be automatically selected based on the parameters received from the network

Data Display:

+++++++++++++++++++++++++++

+ FDD ranking summary     +

+ Freq1   BS1   System    +

+ aaaaa   eee        i    +

+ Freq2   BS2   System    +

+ bbbbb   fff        j    +

+ Freq3   BS3   System    +

+ ccccc   ggg        k    +

+ Freq4   BS4   System    +

+ ddddd   hhh        l    +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_FREQ_1 | W:D | R,I,O | Cell 1 frequency code, center frequency is value / 5. |
| b(5) | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_FREQ_2 | W:D | R,I,O | Cell 2 frequency code, center frequency is value / 5. |
| c(5) | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_FREQ_3 | W:D | R,I,O | Cell 3 frequency code, center frequency is value / 5. |
| d(5) | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_FREQ_4 | W:D | R,I,O | Cell 4 frequency code, center frequency is value / 5. |
| e(3) | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_BSID_1 | W:D | R,I,O | Cell 1 BS ID |
| f(3) | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_BSID_2 | W:D | R,I,O | Cell 2 BS ID |
| g(3) | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_BSID_3 | W:D | R,I,O | Cell 3 BS ID |
| h(3) | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_BSID_4 | W:D | R,I,O | Cell 4 BS ID |
| i | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_SYSTEM_1 | S | R,I,O | FDD home cell = "W", FDD neighbour cell = "w" GSM neigbour cell = "g" Data not available = "-" In case data is not available also corresponding frequency and BS ID field values are meaningless. |
| j | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_SYSTEM_2 | S | R,I,O | FDD home cell = "W", FDD neighbour cell = "w" GSM neigbour cell = "g" Data not available = "-" In case data is not available also corresponding frequency and BS ID field values are meaningless. |
| k | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_SYSTEM_3 | S | R,I,O | FDD home cell = "W", FDD neighbour cell = "w" GSM neigbour cell = "g" Data not available = "-" In case data is not available also corresponding frequency and BS ID field values are meaningless. |
| l | WCDMA_DSP_CS | FTD_SB_WDSP_RANK_SYSTEM_4 | S | R,I,O | FDD home cell = "W", FDD neighbour cell = "w" GSM neigbour cell = "g" Data not available = "-" In case data is not available also corresponding frequency and BS ID field values are meaningless. |

[/FTD_FIELDS]

Display 41.12: FDD Frequency summary (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display shows the RSSI measured on the 3 FDD frequencies being used by the UE

Data Display:

+++++++++++++++++++++++++++

+ FDD frequency summary   +

+                         +

+ Freq       INTRA RSSI   +

+ aaaaa      dddd         +

+ Freq       INTRA RSSI   +

+ bbbbb      eeee         +

+ Freq       INTRA RSSI   +

+ ccccc      ffff         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | WCDMA_DSP_CS | FTD_SB_WDSP_FREQ_INTRA | W:D | R,I,O | Frequency code of home cell, center frequency is value / 5. |
| b(5) | WCDMA_DSP_CS | FTD_SB_WDSP_FREQ _INTER1 | W:D | R,I,O | Frequency code of measured frequency 1, center frequency is value / 5. |
| c(5) | WCDMA_DSP_CS | FTD_SB_WDSP_FREQ _INTER2 | W:D | R,I,O | Frequency code of measured frequency 2, center frequency is value / 5. |
| d(4) | WCDMA_DSP_CS | FTD_SB_WDSP_RSSI_INTRA | W:D | R,I,O | INTRA RSSI * -10 |
| e(4) | WCDMA_DSP_CS | FTD_SB_WDSP_RSSI _INTER1 | W:D | R,I,O | INTER1 RSSI * -10 |
| f(4) | WCDMA_DSP_CS | FTD_SB_WDSP_RSSI _INTER2 | W:D | R,I,O | INTER2 RSSI * - 10 |

[/FTD_FIELDS]

Display 41.13: FDD intra frequency neighbour summary (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This displays shows the status of eight best ranked neighbour cells on home cell frequency

Data Display:

+++++++++++++++++++++++++++

+ FDD intra freq neigh    +

+ Stat ID  Ec Stat ID  Ec +

+    a bbb cc    d eee ff +

+ Stat ID Ec  Stat ID  Ec +

+    g hhh ii    j kkk ll +

+ Stat ID Ec  Stat ID  Ec +

+    m nnn oo    p qqq rr +

+ Stat ID Ec  Stat ID  Ec +

+    s ttt uu    v xx  yy +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_STATUS_1 | S | R,I,O | BS status “a” = Active set, STTD not active on PCCPCH. “m” = Monitored set, STTD not active on PCCPCH. “d” = Detected set, STTD not active on PCCPCH. “u” = Undetected, STTD not active on PCCPCH. “n” = Not listed or detected, STTD not active on PCCPCH. “A” = Active set, STTD active on PCCPCH. “M” = Monitored set, STTD active on PCCPCH. “D” = Detected set, STTD active on PCCPCH. “U” = Undetected, STTD active on PCCPCH. “N” = Not listed or detected, STTD active on PCCPCH. |
| b(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_BSID_1 | W:D | R,I,O | BS ID on INTRA |
| c(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_ECNO_1 | W:D | R,I,O | Cell Ec/No * -1 |
| d | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_STATUS_2 | S | R,I,O | BS status |
| e(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_BSID_2 | W:D | R,I,O | BS ID on INTRA |
| f(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_ECNO_2 | W:D | R,I,O | Cell Ec/No * -1 |
| g | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_STATUS_3 | S | R,I,O | BS status |
| h(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_BSID_3 | W:D | R,I,O | BS ID on INTRA |
| I(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_ECNO_3 | W:D | R,I,O | Cell Ec/No * -1 |
| j | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_STATUS_4 | S | R,I,O | BS status |
| k(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_BSID_4 | W:D | R,I,O | BS ID on INTRA |
| l(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_ECNO_4 | W:D | R,I,O | Cell Ec/No * -1 |
| m | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_STATUS_5 | S | R,I,O | BS status |
| n(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_BSID_5 | W:D | R,I,O | BS ID on INTRA |
| o(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_ECNO_5 | W:D | R,I,O | Cell Ec/No * -1 |
| p | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_STATUS_6 | S | R,I,O | BS status |
| q(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_BSID_6 | W:D | R,I,O | BS ID on INTRA |
| r(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_ECNO_6 | W:D | R,I,O | Cell Ec/No * -1 |
| s | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_STATUS_7 | S | R,I,O | BS status |
| t(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_BSID_7 | W:D | R,I,O | BS ID on INTRA |
| u(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_ECNO_7 | W:D | R,I,O | Cell Ec/No * -1 |
| v | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_STATUS_8 | S | R,I,O | BS status |
| x(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_BSID_8 | W:D | R,I,O | BS ID on INTRA |
| y(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTRA_ECNO_8 | W:D | R,I,O | Cell Ec/No * -1 |

[/FTD_FIELDS]

Displays 41.14 – 41.15: FDD inter frequencies 1 and 2 neighbour summary (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

These two displays show the status of eight best ranked neighbour cells on inter frequencies 1 and 2. Display 41.14 shows inter frequency 1 summary and 41.15 inter frequency 2 summary.

Data Display:

+++++++++++++++++++++++++++

+ FDD inter freq neigh    +

+ Stat ID  Ec Stat ID  Ec +

+    a bbb cc    d eee ff +

+ Stat ID Ec  Stat ID  Ec +

+    g hhh ii    j kkk ll +

+ Stat ID Ec  Stat ID  Ec +

+    m nnn oo    p qqq rr +

+ Stat ID Ec  Stat ID  Ec +

+    s ttt uu    v xxx yy +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_STATUS_1 | S | R,I,O | BS status “a” = Active set, STTD not active on PCCPCH. “m” = Monitored set, STTD not active on PCCPCH. “d” = Detected set, STTD not active on PCCPCH. “u” = Undetected, STTD not active on PCCPCH. “n” = Not listed or detected, STTD not active on PCCPCH. “A” = Active set, STTD active on PCCPCH. “M” = Monitored set, STTD active on PCCPCH. “D” = Detected set, STTD active on PCCPCH. “U” = Undetected, STTD active on PCCPCH. “N” = Not listed or detected, STTD active on PCCPCH. |
| b(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_BSID_1 | W:D | R,I,O | BS ID on INTERx |
| c(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_ECNO_1 | W:D | R,I,O | Cell Ec/No * -1 |
| d | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_STATUS_2 | S | R,I,O | BS status |
| e(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_BSID_2 | W:D | R,I,O | BS ID on INTERx |
| f(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_ECNO_2 | W:D | R,I,O | Cell Ec/No * -1 |
| g | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_STATUS_3 | S | R,I,O | BS status |
| h(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_BSID_3 | W:D | R,I,O | BS ID on INTERx |
| I(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_ECNO_3 | W:D | R,I,O | Cell Ec/No * -1 |
| j | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_STATUS_4 | S | R,I,O | BS status |
| k(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_BSID_4 | W:D | R,I,O | BS ID on INTERx |
| l(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_ECNO_4 | W:D | R,I,O | Cell Ec/No * -1 |
| m | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_STATUS_5 | S | R,I,O | BS status |
| n(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_BSID_5 | W:D | R,I,O | BS ID on INTERx |
| o(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_ECNO_5 | W:D | R,I,O | Cell Ec/No * -1 |
| p | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_STATUS_6 | S | R,I,O | BS status |
| q(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_BSID_6 | W:D | R,I,O | BS ID on INTERx |
| r(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_ECNO_6 | W:D | R,I,O | Cell Ec/No * -1 |
| s | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_STATUS_7 | S | R,I,O | BS status |
| t(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_BSID_7 | W:D | R,I,O | BS ID on INTERx |
| u(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_ECNO_7 | W:D | R,I,O | Cell Ec/No * -1 |
| v | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_STATUS_8 | S | R,I,O | BS status |
| x(3) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_BSID_8 | W:D | R,I,O | BS ID on INTERx |
| y(2) | WCDMA_DSP_CS | FTD_SB_WDSP_DETECT_INTERx_ECNO_8 | W:D | R,I,O | Cell Ec/No * -1 |

[/FTD_FIELDS]

where “x” denotes the number of the inter frequence., x=1 in display 41.14 and x=2 in display 41.15.

Display 41.16: FDD mode GSM cell detection summary  (Version: 3.13, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Status updated and display number changed / Inspection 14-Jun-02, SWAM ADB JT02061255238

3.12	Proposal	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

Displays the top 4 GSM cells showing the frequency code, measured RSSI and if the BSIC has been verified.

Data Display:

+++++++++++++++++++++++++++

+ FDD mode GSM cell detec +

+ GSM1   RSSI    Verified +

+ aaaa   eeee    i        +

+ GSM2   RSSI    Verified +

+ bbbb   ffff    j        +

+ GSM3   RSSI    Verified +

+ cccc   gggg    k        +

+ GSM4   RSSI    Verified +

+ dddd   hhhh    l        +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_FREQ_1 | W:D | R,I,O | GSM Cell 1 frequency code |
| b(4) | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_FREQ_2 | W:D | R,I,O | GSM Cell 2 frequency code |
| c(4) | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_FREQ_3 | W:D | R,I,O | GSM Cell 3 frequency code |
| d(4) | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_FREQ_4 | W:D | R,I,O | GSM Cell 4 frequency code |
| e(4) | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_RSSI_1 | W:D | R,I,O | Cell 1 RSSI *-10 |
| f(4) | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_RSSI_2 | W:D | R,I,O | Cell 2 RSSI *-10 |
| g(4) | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_RSSI_3 | W:D | R,I,O | Cell 3 RSSI *-10 |
| h(4) | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_RSSI_4 | W:D | R,I,O | Cell 4 RSSI *-10 |
| I | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_STATUS_1 | S | R,I,O | Cell 1 verified status “V”=verified, “N” = Not verified |
| j | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_STATUS_2 | S | R,I,O | Cell 2 verified status “V”=verified, “N” = Not verified |
| k | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_STATUS_3 | S | R,I,O | Cell 3 verified status “V”=verified, “N” = Not verified |
| l | WCDMA_DSP_CS | FTD_SB_WDSP_GSM_STATUS_4 | S | R,I,O | Cell 4 verified status “V”=verified, “N” = Not verified |

[/FTD_FIELDS]

Display 41.17: FDD detailed cell info (interactive):  (Version: 3.16, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

3.17	Approved	29-Aug-02	AnttiE	Sub block id for i-field corrected / Inspection 14-Jun-02, SWAM ADB JL02080150458

3.16	Approved	21-Aug-02	AnttiE	A-field description corrected, approved / Inspection 09-Aug-02, SWAM ADB JT02071462724

3.15	Proposal	11-Jul-02	AnttiE	Corrections proposed

3.13	Rejected	27-Jun-02	AnttiE	Display number changed / Inspection 14-Jun-02

3.12	Rejected	07-Jun-02	AnttiE	Display added / RM tool 4278

[/Version history]

This display promts for a cell frequence code and BSID and shows info about the selected cell.

Use the following steps to activate this display in interactive mode. The display is not activated if the arrow () buttons are used.

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display by giving the display number 4117 to the query prompt.

- Test input prompt will be shown on the display. Enter frequency code and BSID as specified in the table below.

- Confirm with the Ok button.

- Info about the cell is shown if the activation was successful.

Data Display:

+++++++++++++++++++++++++++

+ FDD detailed cell info  +

+                         +

+ Frequency code    aaaaa +

+ RSSI  bbbb   BsID   ccc +

+ R_Order dd   BsStatus e +

+ Syncro   f   TxDiv    g +

+ Frame timing      hhhhh +

+ SCPICH   l   EcNO   jjj +

+ RSCP  kkkk              +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_INPUT | DW:D | r,o.i | Format xxxxxyyy, where xxxxx:: frequency code (decimal) yyy: BSID (decimal) |
| a(5) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_FREQ | W:D | R,I,O | Frequency code |
| b(4) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_RSSI | W:D | R,I,O | RSSI * -10 |
| c(3) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_BSID | W:D | R,I,O | BS ID |
| d(2) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_RANK | W:D | R,I,O | cell ranking order |
| e | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_STATUS | S | R,I,O | BS status “A” = Active set “M” = Monitored set “D” = Detected set “U” = Undetected “N” = Not listed or detected |
| f | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_SYNC | S | R,I,O | Cell syncronization status: “N” = Not syncronized, “S” = Syncronized “D” = SFN Decoded |
| g | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_STTD | S | R,I,O | Cell tx diversity status: “-” = STTD not used on PCCPCH “S” = STTD used on PCCPCH. |
| h(5) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_TIMING | W:D | R,I,O | Cell frame timing in relation to WCDMA system clock. |
| I | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_SCPICH | S | R,I,O | SCPICH measurement status: “-” = SCPICH not used “S” = SCPICH used |
| j(3) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_H | W:D | R,I,O | EcNo * -10 |
| k(4) | WCDMA_DSP_CS | FTD_SB_WDSP_CELL_DETAIL_G | W:D | R,I,O | RSCP * -10 |

- Groups 42-45: WCDMA Layer 1 Field Test Displays

To be defined.

- Group 46: WCDMA RAN System displays

Display Display 46.01: RRC Global Status (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Global RRC status

Data Display:

+++++++++++++++++++++++++++

+ RRC Global status       +

+                         +

+ Global state  aaaaaaaaa +

+ Active Domain CS:     b +

+ Active Domain PS:     c +

+ Drop cause dddddddddddd +

+ Ciphering CS          e +

+ Ciphering PS          f +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(9) | PN_WRAN | FTD_SB_RRC_GLOBAL_STATE | S | R,O | RRC Global State OOZ, idle-pch, cell-dch, cell-fach, cell-pch, ura-pch |
| b(1) | PN_WRAN | FTD_SB_RRC_GLOBAL_ACTIVE_DOMAIN_CS | B:D | R,O | RRC Active Domain - CS 1/0 |
| c(1) | PN_WRAN | FTD_SB_RRC_GLOBAL_ACTIVE_DOMAIN_PS | B:D | R,O | RRC Active Domain - PS 1/0 |
| d(12) | PN_WRAN | FTD_SB_RRC_CALL_RELEASE_DROP_CAUSE | S | R,O | Last Call Drop/Release Cause |
| e(1) | PN_WRAN | FTD_SB_RRC_CS_DOMAIN_CIPHERING | B:D | R,O | Ciphering - CS Domain on/off - 1/0 |
| f(1) | PN_WRAN | FTD_SB_RRC_PS_DOMAIN_CIPHERING | B:D | R,O | Ciphering - PS Domain on/off - 1/0 |

[/FTD_FIELDS]

Display 46.02: PEER message MSC (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows an MSC of last 7 PEER messages Message order runs vertically. The oldest message is blanked out.

ASUp - Active Set Update (C - Complete, F - Failure)

ADD - Assistance Data Delivery CCO - Cell Change Order From UTRAN (F - Failure)

CU - Cell Update (Cnf - Confirm)

CtCk - Counter Check (R - Response)

HOFU - Handover From UTRAN Command (F - Failure)

HOTU - Handover To UTRAN Command (C - Complete)

IRHI - Inter RAT Handover Info

MeCn - Measurement Control (F - Failure)

PAGEx - Paging Type x

PCRc - Physical Channel Reconfiguration (C - Complete, F - Failure)

PSCA - Physical Shared Channel Allocation

PCRq - PUSCH Capacity Request

RBRc - Radio Bearer Reconfiguration (C - Complete, F - Failure)

RBRl - Radio Bearer Release (C - Complete, F - Failure)

RBSt - Radio Bearer Setup (C - Complete, F - Failure)

RCRej - RRC Connection Reject

RCRl - RRC Connection Release (C - Complete)

RCReq - RRC Connection Request

RCSt - RRC Connection Setup (C - Complete)

RFI - RRC Failure Info

RS - RRC Status

SecM - Security Mode Command (C - Complete, F - Failure)

SgCR - Signalling Connection Release (I - Indication)

SICI - System Information Change Indication

TCRc - Transport Channel Reconfiguration (C - Complete, F - Failure)

TFCC - Transport Format Combination Control (F - Failure)

UECEq - UE Capability Enquiry

UECI - UE Capability Information (C - Confirm)

UPCC - Uplink Physical Channel Control

UraU - URA Update (C - Confirm)

UMI - UTRAN Mobility Information (C - Confirm, F - Failure)

Data Display:

+++++++++++++++++++++++++++

+ PEER message MSC        +

+ PEER message ID   aaaaa +

+ PEER message ID   bbbbb +

+ PEER message ID   ccccc +

+ PEER message ID   ddddd +

+ PEER message ID   eeeee +

+ PEER message ID   fffff +

+ PEER message ID   ggggg +

+ PEER message ID   hhhhh +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | PN_WRAN | FTD_SB_RRC_PEER_MSG_1_ID | S | R,O | PEER Message ID |
| e(5) | PN_WRAN | FTD_SB_RRC_PEER_MSG_5_ID | S | R,O | PEER Message ID |
| b(5) | PN_WRAN | FTD_SB_RRC_PEER_MSG_2_ID | S | R,O | PEER Message ID |
| f(5) | PN_WRAN | FTD_SB_RRC_PEER_MSG_6_ID | S | R,O | PEER Message ID |
| c(5) | PN_WRAN | FTD_SB_RRC_PEER_MSG_3_ID | S | R,O | PEER Message ID |
| g(5) | PN_WRAN | FTD_SB_RRC_PEER_MSG_7_ID | S | R,O | PEER Message ID |
| d(5) | PN_WRAN | FTD_SB_RRC_PEER_MSG_4_ID | S | R,O | PEER Message ID |
| h(5) | PN_WRAN | FTD_SB_RRC_PEER_MSG_8_ID | S | R,O | PEER Message ID |

[/FTD_FIELDS]

Display 46.03: RNTI Values (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows current RNTI values

Data Display:

+++++++++++++++++++++++++++

+ RNTI values             +

+                         +

+ USRNC identity      aaa +

+ USRNTI            bbbbb +

+ C-RNTI             cccc +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_U_RNTI_SRNC_IDENTITY | W:H | R,O | U-RNTI SRNC Identity 0..FFF |
| b(5) | PN_WRAN | FTD_SB_RRC_U_RNTI_SRNTI | DW:H | R,O | U-RNTI SRNTI 0..FFFFF |
| c(4) | PN_WRAN | FTD_SB_RRC_C_RNTI | W:H | R,O | C-RNTI 0..FFFF |

[/FTD_FIELDS]

Display 46.04: Ciphering Capability (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen is used to set and display the Ciphering capability - possible inputs are shown on the help text

Data Display:

+++++++++++++++++++++++++++

+ Ciphering capability    +

+                         +

+ UEA0 ciphering:         +

+ aaaaaaaa                +

+                         +

+ UEA1 ciphering:         +

+ bbbbbbbb                +

+                         +

+                         +

+++++++++++++++++++++++++++

[[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_rrc_ftd_flag_uea_ciphering_capability | DW:D | R,O | callback function subblock identity |
| a(8) | PN_WRAN | FTD_SB_RRC_UEA0_CIPHER_FLAG | S | R,O | Ciphering capability flag to show if UEA0 is ENABLED or DISABLED |
| b(8) | PN_WRAN | FTD_SB_RRC_UEA1_CIPHER_FLAG | S | R,O | Ciphering capability flag to show if UEA1 is ENABLED or DISABLED |

[/FTD_FIELDS]

Display 46.05: Cell Selection - Screen 2 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows the current PLMN information

Data Display:

+++++++++++++++++++++++++++

+ Cell selection - 2      +

+                         +

+ PLMN number      aaaaaa +

+ Search type      bbbbbb +

+ Trigger type     cccccc +

+ PLMN frequency    ddddd +

+ PLMN scramble code  eee +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | PN_WRAN | FTD_SB_RRC_PLMN_NUMBER | DW:H | R,O | PLMN Number - 0..FFFFFF |
| b(6) | PN_WRAN | FTD_SB_RRC_PLMN_SEARCH_TYPE | S | R,O | Type of search used to find PLMN nonini, init, candid, allcel, emerge, PLMN |
| c(6) | PN_WRAN | FTD_SB_RRC_PLMN_SEARCH_TRIGGER | S | R,O | Type of trigger used to find PLMN - L1trig, statch, intRat |
| d(5) | PN_WRAN | FTD_SB_RRC_PLMN_FREQ | W:D | R,O | PLMN frequeny |
| e(3) | PN_WRAN | FTD_SB_RRC_PLMN_SCR_CODE | W:D | R,O | PLMN scramble code |

[/FTD_FIELDS]

Display 46.06: FDD BTS Carrier Lock Mode (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen is used in BTS carrier lock mode to retrieve FDD Frequency and Scrambling code. The frequency and scrambling code is entered as one decimal value. Both frequency and scrambling code are five digits. e.g. entered value on FTD is '1060000040', where '10600' is the BTS test frequency and '00040' is the test scrambling code. To clear the BTS test mode user enters '0000000000'.

Data Display:

+++++++++++++++++++++++++++

+ FDD BTS carrier lock    +

+ mode                    +

+                         +

+ FDD frequency:          +

+ aaaaa                   +

+                         +

+ FDD scrambling code:    +

+ bbbbb                   +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_rrc_ftd_wcdma_bts_freq_lock_mode | DW:D | R,O | callback function subblock identity |
| a(5) | PN_WRAN | FTD_SB_RRC_BTS_CARRIER_LOCK_FREQ | W:D | R,O | FDD Frequency to use in BTS carrier lock mode |
| b(5) | PN_WRAN | FTD_SB_RRC_BTS_CARRIER_LOCK_SCR_CODE | W:D | R,O | FDD scrambling code to use in BTS carrier lock mode |

[/FTD_FIELDS]

Display 46.07: Counter Reset (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen is used to reset counters

Data Display:

+++++++++++++++++++++++++++

+ Counter reset           +

+                         +

+ Reset all counters 1    +

+ Quit               0    +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_rrc_ftd_reset_counters | DW:D | R,O | callback function subblock identity |

[/FTD_FIELDS]

Display 46.08: Call Failure Reasons (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Counts for each call failure reason

Data Display:

+++++++++++++++++++++++++++

+ Call failure            +

+ reason                  +

+                         +

+ RLink  aaa Handover bbb +

+ RLC    ccc Uplayer  ddd +

+ Normal eee RLC/SIB7 fff +

+ Config ggg V/N300   hhh +

+ CRT    iii V/N300   jjj +

+ OoS    kkk Unspec   lll +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_RADIO_LINK_FAILURE_COUNT | W:D | R,O | Number of Radio link Failures |
| b(3) | PN_WRAN | FTD_SB_RRC_HO_FROM_UTRAN_COUNT | W:D | R,O | Number of Handover to UTRAN |
| c(3) | PN_WRAN | FTD_SB_RRC_RLC_LINK_ERROR_COUNT | W:D | R,O | Number of RLC link Error |
| d(3) | PN_WRAN | FTD_SB_RRC_RELEASE_REQ_COUNT | W:D | R,O | Number of upper layer triggered release |
| e(3) | PN_WRAN | FTD_SB_RRC_NORMAL_REL_COUNT | W:D | R,O | Number of normal release |
| f(3) | PN_WRAN | FTD_SB_RRC_SIB7_RECEPTION_FAIL_COUNT | W:D | R,O | Number of RLC link Error |
| g(3) | PN_WRAN | FTD_SB_RRC_CONFIG_FAILURE_COUNT | W:D | R,O | Number of configuration failures |
| h(3) | PN_WRAN | FTD_SB_RRC_V300_GR_N300_COUNT | W:D | R,O | Number of V300 > N300 failures |
| i(3) | PN_WRAN | FTD_SB_RRC_T314_T315_TIMEOUT_COUNT | W:D | R,O | Number of Connection Reestablishment Timers Timeouts failures-T314/T315 |
| j(3) | PN_WRAN | FTD_SB_RRC_V302_GR_N302_COUNT | W:D | R,O | Number of V302 > N302 failures |
| k(3) | PN_WRAN | FTD_SB_RRC_T316_T317_T307_COUNT | W:D | R,O | Number of Out of Service Timer Timeout failures-T316/T317/T307 |
| l(3) | PN_WRAN | FTD_SB_RRC_UNSPECIFIC_FAILURE_COUNT | W:D | R,O | Number of unspecific failures |

[/FTD_FIELDS]

Display 46.09: L1 State (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows L1 State

Data Display:

+++++++++++++++++++++++++++

+ L1 state                +

+                         +

+ PCCPCH state   a        +

+ SCCPCH state   b        +

+ RACH state     c        +

+ DPCH state     d        +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(1) | PN_WRAN | FTD_SB_RRC_PCCPCH_STATE | B:D | R,O | PCCPCH State |
| b(1) | PN_WRAN | FTD_SB_RRC_SCCPCH_STATE | B:D | R,O | SCCPCH State |
| c(1) | PN_WRAN | FTD_SB_RRC_RACH_STATE | B:D | R,O | RACH State |
| d(1) | PN_WRAN | FTD_SB_RRC_DPCH_STATE | B:D | R,O | DPCH State |

[/FTD_FIELDS]

Display 46.10: Cell Reselection - Screen 1 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows reselection OK and Fail counters

Data Display:

+++++++++++++++++++++++++++

+ Cell reselection - 1    +

+ Idle        OK / Failed +

+          aaaaa    bbbbb +

+ Connected   OK / Failed +

+          ccccc    ddddd +

+ UMTS => GSM OK / FAILED +

+          eeeee    fffff +

+ GSM => UMTS OK / FAILED +

+          ggggg    hhhhh +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | PN_WRAN | FTD_SB_RRC_IDLE_RESELECTION_OK_COUNT | W:D | R,O | Number of successful Idle mode reselections |
| b(5) | PN_WRAN | FTD_SB_RRC_IDLE_RESELECTION_FAIL_COUNT | W:D | R,O | Number of unsuccessful Idle mode reselections |
| c(5) | PN_WRAN | FTD_SB_RRC_CON_RESELECTION_OK_COUNT | W:D | R,O | Number of successful Connected mode reselections |
| d(5) | PN_WRAN | FTD_SB_RRC_CON_RESELECTION_FAIL_COUNT | W:D | R,O | Number of unsuccessful Connected mode reselections |
| e(5) | PN_WRAN | FTD_SB_RRC_UMTS_TO_GSM_OK_COUNT | W:D | R,O | Number of successful UMTS to GSM reselections |
| f(5) | PN_WRAN | FTD_SB_RRC_UMTS_TO_GSM_FAIL_COUNT | W:D | R,O | Number of unsuccessful UMTS to GSM reselections |
| g(5) | PN_WRAN | FTD_SB_RRC_GSM_TO_UMTS_OK_COUNT | W:D | R,O | Number of successful GSM to UMTS reselections |
| h(5) | PN_WRAN | FTD_SB_RRC_GSM_TO_UMTS_FAIL_COUNT | W:D | R,O | Number of unsuccessful GSM to UMTS reselections |

[/FTD_FIELDS]

Display 46.11: Radio Access Bearer Information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Radio Access Bearers currently setup

Data Display:

+++++++++++++++++++++++++++

+ RAB information         +

+ RAB ID        Domain    +

+ aaa           b         +

+ ccc           d         +

+ eee           f         +

+ ggg           h         +

+ iii           j         +

+ kkk           l         +

+ SRB’s ID   mm nn oo pp  +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_RAB_ID1 | B:D | R,O | Radio Access Bearer ID |
| b(1) | PN_WRAN | FTD_SB_RRC_RAB_ID1_DOMAIN | S | R,O | Domain - PS, CS |
| c(3) | PN_WRAN | FTD_SB_RRC_RAB_ID2 | B:D | R,O | Radio Access Bearer ID |
| d(1) | PN_WRAN | FTD_SB_RRC_RAB_ID2_DOMAIN | S | R,O | Domain - PS, CS |
| e(3) | PN_WRAN | FTD_SB_RRC_RAB_ID3 | B:D | R,O | Radio Access Bearer ID |
| f(1) | PN_WRAN | FTD_SB_RRC_RAB_ID3_DOMAIN | S | R,O | Domain - PS, CS |
| g(3) | PN_WRAN | FTD_SB_RRC_RAB_ID4 | B:D | R,O | Radio Access Bearer ID |
| h(1) | PN_WRAN | FTD_SB_RRC_RAB_ID4_DOMAIN | S | R,O | Domain - PS, CS |
| i(3) | PN_WRAN | FTD_SB_RRC_RAB_ID5 | B:D | R,O | Radio Access Bearer ID |
| j(1) | PN_WRAN | FTD_SB_RRC_RAB_ID5_DOMAIN | S | R,O | Domain - PS, CS |
| k(3) | PN_WRAN | FTD_SB_RRC_RAB_ID6 | B:D | R,O | Radio Access Bearer ID |
| l(1) | PN_WRAN | FTD_SB_RRC_RAB_ID6_DOMAIN | S | R,O | Domain - PS, CS |
| m(2) | PN_WRAN | FTD_SB_RRC_SRB_ID1 | B:D | R,O | Signalling Radio Bearer ID |
| n(2) | PN_WRAN | FTD_SB_RRC_SRB_ID2 | B:D | R,O | Signalling Radio Bearer ID |
| o(2) | PN_WRAN | FTD_SB_RRC_SRB_ID3 | B:D | R,O | Signalling Radio Bearer ID |
| p(2) | PN_WRAN | FTD_SB_RRC_SRB_ID4 | B:D | R,O | Signalling Radio Bearer ID |

[/FTD_FIELDS]

Display 46.12: Radio Bearer Information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows all Radio Bearers associated with a RAB id. The RAB id and the associated domain are entered as a single decimal value. The RAB id is the first three digits and the domain is input as the last digit, 1 for PS and 0 for CS.

Data Display:

+++++++++++++++++++++++++++

+ RB information - 1      +

+                         +

+ RAB ID and domain(P/C): +

+ aaab                    +

+                         +

+ Radio Bearer ID         +

+ cc dd ee ff gg hh ii jj +

+ Re-establishment timer  +

+ kkkkk                   +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_rrc_ftd_rab_id_set | DW:D | R,O | callback function subblock identity |
| a(3) | PN_WRAN | FTD_SB_RRC_RAB_ID | B:D | R,O | Radio Access Bearer ID |
| b(1) | PN_WRAN | FTD_SB_RRC_RAB_ID_DOMAIN | S | R,O | Domain - PS, CS |
| c(2) | PN_WRAN | FTD_SB_RRC_RB_ID1 | B:D | R,O | Radio Bearer ID |
| d(2) | PN_WRAN | FTD_SB_RRC_RB_ID2 | B:D | R,O | Radio Bearer ID |
| e(2) | PN_WRAN | FTD_SB_RRC_RB_ID3 | B:D | R,O | Radio Bearer ID |
| f(2) | PN_WRAN | FTD_SB_RRC_RB_ID4 | B:D | R,O | Radio Bearer ID |
| g(2) | PN_WRAN | FTD_SB_RRC_RB_ID5 | B:D | R,O | Radio Bearer ID |
| h(2) | PN_WRAN | FTD_SB_RRC_RB_ID6 | B:D | R,O | Radio Bearer ID |
| i(2) | PN_WRAN | FTD_SB_RRC_RB_ID7 | B:D | R,O | Radio Bearer ID |
| j(2) | PN_WRAN | FTD_SB_RRC_RB_ID8 | B:D | R,O | Radio Bearer ID |
| k(5) | PN_WRAN | FTD_SB_RRC_RAB_RE_EST_TIMER | S | R,O | Radio Access Bearer Re-establishment Timer (T314/T315) |

[/FTD_FIELDS]

Display 46.13: Radio Bearer Information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen takes an input Radio Bearer id and shows associated information

Data Display:

+++++++++++++++++++++++++++

+ RB information - 2      +

+                         +

+ RB ID: aa               +

+                         +

+ Domain            bbbbb +

+ Re-establishment  ccccc +

+ RB status         ddddd +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_rrc_ftd_rb_id_set | DW:D | R,O | callback function subblock identity |
| a(2) | PN_WRAN | FTD_SB_RRC_RB_ID | B:D | R,O | Radio Bearer ID |
| b(5) | PN_WRAN | FTD_SB_RRC_RB_DOMAIN | S | R,O | Domain - PS, CS, SRB |
| c(5) | PN_WRAN | FTD_SB_RRC_RB_RE_EST_TIMER | S | R,O | Radio Bearer Re-establishment Timer (T314/T315) |
| d(5) | PN_WRAN | FTD_SB_RRC_RB_STATUS | S | R,O | RB Status - Start, Stop |

[/FTD_FIELDS]

Display 46.14: DPCH Change Count (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

Display number of successful HHOs and channel reconfigurations

Data Display:

+++++++++++++++++++++++++++

+ DPCH change count       +

+                         +

+ HHO count           aaa +

+ DPCH reconf count   bbb +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_SUCCESSFUL_HHOS | B:H | R | Number of successful HHOs |
| b(3) | PN_WRAN | FTD_SB_SUCCESSFUL_DPCH_RECONFIGS | B:H | R | Number of successful DPCH reconfigs |

[/FTD_FIELDS]

Display 46.15: Tracker Feature Enable/Disable (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

Sets Forced InterSystem Mode - IS resel fail, ISHO blind/fallback and CCO blind/fallback

Data Display:

+++++++++++++++++++++++++++

+ Tracker feature on/off  +

+                         +

+ Tracker feature         +

+ enable/disable          +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_WPH_SET_IS_MODE | DW:D | R | callback function subblock identity |

[/FTD_FIELDS]

Display 46.27: Voice RLC-U Information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Voice RLC-U information. This includes the number of RB's used, whether or not DTX is supported and the Frame Rate

Data Display:

+++++++++++++++++++++++++++

+ Voice RLC-U info        +

+                         +

+ Radio Bearer  a         +

+ DTX           b         +

+ Frame rate    cc.dd     +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(1) | PN_WRAN | FTD_SB_W_MAC_VOICE_NUMBER_OF_RADIO_BEARERS_USED | B:D | R | This parameter reflects the number of radio bearers used. This will have a range of 1-3 and is represented as a single digit decimal value |
| b(1) | PN_WRAN | FTD_SB_W_MAC_VOICE_DTX_SUPPORTED | B:D | R | This Boolean flag shows whether DTX is supported or not. This is represented as a single digit decimal value as follows: 0 - DTX Not Supported 1 - DTX Supported |
| c(2) | PN_WRAN | FTD_SB_W_MAC_VOICE_FRAME_RATE_WHOLE_PART | W:D | R | This field shows the voice frame rate representing the whole part of the 3-digit frame rate which is represented as a single digit decimal value. It may represent the whole part of one of the following valid frame rate values: 4.57, 5.15, 5.90, 6.70, 7.40, 7.95, 10.2, 12.2 |
| d(2) | PN_WRAN | FTD_SB_W_MAC_VOICE_FRAME_RATE_FRAC_PART | W:D | R | This field shows the voice frame rate representing the fractional part of the 3-digit frame rate which is represented as a 2 digit decimal value. It may represent the fractional part of one of the following valid frame rate values: 4.57, 5.15, 5.90, 6.70, 7.40, 7.95, 10.2, 12.2 |

[/FTD_FIELDS]

Display 46.28: DTX Enable/Disable input (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shall form the input to Enable or Disable DTX

Data Display:

+++++++++++++++++++++++++++

+ DTX enable/disable      +

+ input                   +

+                         +

+ DTX enable/disable      +

+ (1/0): a                +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_DTX_ENABLE_DISABLE | DW:D | R | callback function subblock identity |
| a(1) | PN_WRAN | FTD_SB_W_MAC_DTX_ENABLE_DISABLE_INPUT | B:D | R | This screen require a single digit decimal input to either enable or disable DTX. This may be done by entering the digit 1 or 0, having the following effect: 0 - Disable DTX 1 - Enable DTX |

[/FTD_FIELDS]

Display 46.60: 3G to 2G Reselections (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This is used to display information helpful in analysing 3G to 2G reselections

Data Display:

+++++++++++++++++++++++++++

+ 3G to 2G reselections   +

+                         +

+ Scrambling aaa ECNO bbb +

+ Inter-RAT new cell  ccc +

+ ARFCN     dddd verify k +

+ RSSI eee  Mode State  j +

+ GSM sui OK/FAIL fff ggg +

+ GSM res OK/FAIL hhh iii +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_PRI_SCR_CODE_TYPE | W:D | R | Primary Scrambling Code |
| b(3) | PN_WRAN | FTD_SB_ECNO_VALUE | B:D | R | ECNO Value |
| c(3) | PN_WRAN | FTD_SB_NEW_RAT_CELL_IND_COUNT | B:D | R | Count of inter-RAT new cell inds |
| d(4) | PN_WRAN | FTD_SB_GSM_ARFCN0 | W:D | R | Strongest GSM ARFCN |
| e(3) | PN_WRAN | FTD_SB_STATUS_RSSI0 | W:D | R | Status RSSI level |
| k(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS | B:D | R | 0 indicates that the ARFCN is not verified, 1 indicates that the ARFCN is verified |
| f(3) | PN_WRAN | FTD_SB_SUCCESS_GSM_SUITABILITY_CHECK_COUNT | B:D | R | Count of Successfull GSM Suitability Checks |
| g(3) | PN_WRAN | FTD_SB_FAILED_GSM_SUITABILITY_CHECK_COUNT | B:D | R | Count of failed GSM Suitability Checks |
| h(3) | PN_WRAN | FTD_SB_SUCCESS_GSM_INTER_RESELECTION_COUNT | B:D | R | Count of successful GSM inter-RAT reselections |
| i(3) | PN_WRAN | FTD_SB_FAILED_GSM_INTER_RESELECTION_COUNT | B:D | R | Count of failed GSM inter-RAT reselections |
| j(1) | PN_WRAN | FTD_SB_MODE_STATE | W:D | R | 0 indicates 3G selected cell, 1 indicates 2G selected state |

[/FTD_FIELDS]

Display 46.61: 3G to 2G ISHO #1 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This is used to display information helpful in analysing 3G to 2G ISHO

Data Display:

+++++++++++++++++++++++++++

+ 3G to 2G ISHO - 1       +

+                         +

+ Scrambling aaa ECNO bbb +

+ Comp c ARFCN dddd ver k +

+ RSSI eee Mode state   j +

+ Handover Ok         fff +

+ Handover Fail       ggg +

+ Handover OK fall    hhh +

+ Handover Fail fall  iii +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_PRI_SCR_CODE_TYPE | W:D | R | Primary Scrambling Code |
| b(3) | PN_WRAN | FTD_SB_ECNO_VALUE | B:D | R | ECNO Value |
| c(1) | PN_WRAN | FTD_SB_CM_STATUS | B:D | R | Status of compressed mode patterns for RSSI, Initial BSIC verification & BSIC refresh |
| d(4) | PN_WRAN | FTD_SB_GSM_ARFCN0 | W:D | R | Strongest GSM ARFCN |
| e(3) | PN_WRAN | FTD_SB_STATUS_RSSI0 | W:D | R | Status RSSI level |
| k(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS | B:D | R | 0 indicates that the ARFCN is not verified, 1 indicates that the ARFCN is verified |
| f(3) | PN_WRAN | FTD_SB_SUCCESS_HO_ATTEMPS | B:D | R | Count of succesful handover attemps |
| g(3) | PN_WRAN | FTD_SB_FAILED_HO_ATTEMPS | B:D | R | Count of failed handover attemps |
| h(3) | PN_WRAN | FTD_SB_SUCCESS_FALLBACK_ATTEMPS | B:D | R | Count of succesful fallback attemps |
| i(3) | PN_WRAN | FTD_SB_FAILED_FALLBACK_ATTEMPS | B:D | R | Count of failed fallback attemps |
| j(1) | PN_WRAN | FTD_SB_MODE_STATE | W:D | R | 0 indicates 3G selected cell, 1 indicates 2G selected state |

[/FTD_FIELDS]

Display 46.62: 3G to 2G ISHO #2 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

List of strongest GSM cells containing ARFCN, verification status, RSSI level

Data Display:

+++++++++++++++++++++++++++

+ 3G to 2G ISHO - 2       +

+ AFCN0 RSSI0 Verified    +

+ aaaa  bbb   i           +

+ AFCN1 RSSI1 Verified    +

+ cccc  ddd   j           +

+ AFCN2 RSSI2 Verified    +

+ eeee  fff   k           +

+ AFCN3 RSSI3 Verified    +

+ gggg  hhh   l           +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | PN_WRAN | FTD_SB_GSM_ARFCN0 | W:D | R | Strongest GSM ARFCN |
| b(3) | PN_WRAN | FTD_SB_STATUS_RSSI0 | W:D | R | Status RSSI level |
| i(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS | B:D | R | 0 indicates that the ARFCN is not verified, 1 indicates that the ARFCN is verified |
| c(4) | PN_WRAN | FTD_SB_GSM_ARFCN1 | W:D | R | Strongest GSM ARFCN 1 |
| d(3) | PN_WRAN | FTD_SB_STATUS_RSSI1 | W:D | R | Status RSSI level 1 |
| j(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS1 | B:D | R | 0 indicates the ARFCN is not verified, 1 indicates the ARFCN is verified |
| e(4) | PN_WRAN | FTD_SB_GSM_ARFCN2 | W:D | R | Strongest GSM ARFCN 2 |
| f(3) | PN_WRAN | FTD_SB_STATUS_RSSI2 | W:D | R | Status RSSI level 2 |
| k(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS2 | B:D | R | 0 indicates the ARFCN is not verified, 1 indicates the ARFCN is verified |
| g(4) | PN_WRAN | FTD_SB_GSM_ARFCN3 | W:D | R | Strongest GSM ARFCN 3 |
| h(3) | PN_WRAN | FTD_SB_STATUS_RSSI3 | W:D | R | Status RSSI level 3 |
| l(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS3 | B:D | R | 0 indicates the ARFCN is not verified, 1 indicates the ARFCN is verified |

[/FTD_FIELDS]

Display 46.63: 3G to 2G ISHO #3 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

List of strongest GSM cells containing ARFCN, verification status, RSSI level

Data Display:

+++++++++++++++++++++++++++

+ 3G to 2G ISHO - 3       +

+ AFCN4 RSSI4 Verified    +

+ aaaa  bbb   i           +

+ AFCN5 RSSI5 Verified    +

+ cccc  ddd   j           +

+ AFCN6 RSSI6 Verified    +

+ eeee  fff   k           +

+ AFCN7 RSSI7 Verified    +

+ gggg  hhh   l           +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | PN_WRAN | FTD_SB_GSM_ARFCN4 | W:D | R | Strongest GSM ARFCN 4 |
| b(3) | PN_WRAN | FTD_SB_STATUS_RSSI4 | W:D | R | Status RSSI level 4 |
| i(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS4 | B:D | R | 0 indicates the ARFCN is not verified, 1 indicates the ARFCN is verified |
| c(4) | PN_WRAN | FTD_SB_GSM_ARFCN5 | W:D | R | Strongest GSM ARFCN 5 |
| d(3) | PN_WRAN | FTD_SB_STATUS_RSSI5 | W:D | R | Status RSSI level 5 |
| j(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS5 | B:D | R | 0 indicates the ARFCN is not verified, 1 indicates the ARFCN is verified |
| e(4) | PN_WRAN | FTD_SB_GSM_ARFCN6 | W:D | R | Strongest GSM ARFCN 6 |
| f(3) | PN_WRAN | FTD_SB_STATUS_RSSI6 | W:D | R | Status RSSI level 6 |
| k(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS6 | B:D | R | 0 indicates the ARFCN is not verified, 1 indicates the ARFCN is verified |
| g(4) | PN_WRAN | FTD_SB_GSM_ARFCN7 | W:D | R | Strongest GSM ARFCN 7 |
| h(3) | PN_WRAN | FTD_SB_STATUS_RSSI7 | W:D | R | Status RSSI level 7 |
| l(1) | PN_WRAN | FTD_SB_VERIFICATION_STATUS7 | B:D | R | 0 indicates the ARFCN is not verified, 1 indicates the ARFCN is verified |

- Group 47: WCDMA RRC displays

Display 47.01: RRC Global State Change Counters (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Global RRC state Change Countes

Data Display:

+++++++++++++++++++++++++++

+ RRC global state change +

+ counters                +

+                         +

+ DCH  => FACH      aaaaa +

+ FACH => DCH       bbbbb +

+ FACH => CELL_PCH  ccccc +

+ FACH => URA_PCH   ddddd +

+ DCH  => CELL_PCH  eeeee +

+ DCH  => URA_PCH   fffff +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | PN_WRAN | FTD_SB_RRC_DCH_TO_FACH_COUNT | W:D | R,O | Number of state changes from DCH to FACH |
| b(5) | PN_WRAN | FTD_SB_RRC_FACH_TO_DCH_COUNT | W:D | R,O | Number of state changes from FACH to DCH |
| c(5) | PN_WRAN | FTD_SB_RRC_FACH_TO_CELLPCH_COUNT | W:D | R,O | Number of state changes from FACH to CELL_PCH |
| d(5) | PN_WRAN | FTD_SB_RRC_FACH_TO_URAPCH_COUNT | W:D | R,O | Number of state changes from FACH to URA_PCH |
| e(5) | PN_WRAN | FTD_SB_RRC_DCH_TO_CELLPCH_COUNT | W:D | R,O | Number of state changes from DCH to CELL_PCH |
| f(5) | PN_WRAN | FTD_SB_RRC_DCH_TO_URAPCH_COUNT | W:D | R,O | Number of state changes from DCH to URA_PCH |

[/FTD_FIELDS]

Display 47.02: Counters for Timeouts - Screen 1 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows the number timeouts for RRC timers

Data Display:

+++++++++++++++++++++++++++

+ RRC count timeouts - 1  +

+                         +

+ T300  T312  T301  T302  +

+  aaa   bbb   ccc   ddd  +

+                         +

+ T304  T305  T307  T308  +

+  eee   fff   ggg   hhh  +

+                         +

+ T309   iii              +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_T300_IDLE_TIMEOUT_COUNT | W:D | R,O | T300 timeout counter |
| b(3) | PN_WRAN | FTD_SB_RRC_T312_IDLE_TIMEOUT_COUNT | W:D | R,O | T312 timeout counter |
| c(3) | PN_WRAN | FTD_SB_RRC_T301_TIMEOUT_COUNT | W:D | R,O | T301 timeout counter |
| d(3) | PN_WRAN | FTD_SB_RRC_T302_TIMEOUT_COUNT | W:D | R,O | T302 timeout counter |
| e(3) | PN_WRAN | FTD_SB_RRC_T304_TIMEOUT_COUNT | W:D | R,O | T304 timeout counter |
| f(3) | PN_WRAN | FTD_SB_RRC_T305_TIMEOUT_COUNT | W:D | R,O | T305 timeout counter |
| g(3) | PN_WRAN | FTD_SB_RRC_T307_TIMEOUT_COUNT | W:D | R,O | T307 timeout counter |
| h(3) | PN_WRAN | FTD_SB_RRC_T308_TIMEOUT_COUNT | W:D | R,O | T308 timeout counter |
| i(3) | PN_WRAN | FTD_SB_RRC_T309_TIMEOUT_COUNT | W:D | R,O | T309 timeout counter |

[/FTD_FIELDS]

Display 47.03: Counters for Timeouts - Screen 2 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows the number timeouts for RRC timers

Data Display:

+++++++++++++++++++++++++++

+ RRC count timeouts - 2  +

+                         +

+ T310  T311  T312  T313  +

+  aaa   bbb   ccc   ddd  +

+                         +

+ T314  T315  T316  T317  +

+  eee   fff   ggg   hhh  +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_T310_TIMEOUT_COUNT | W:D | R,O | T310 timeout counter |
| b(3) | PN_WRAN | FTD_SB_RRC_T311_TIMEOUT_COUNT | W:D | R,O | T311 timeout counter |
| c(3) | PN_WRAN | FTD_SB_RRC_T312_TIMEOUT_COUNT | W:D | R,O | T312 timeout counter |
| d(3) | PN_WRAN | FTD_SB_RRC_T313_TIMEOUT_COUNT | W:D | R,O | T313 timeout counter |
| e(3) | PN_WRAN | FTD_SB_RRC_T314_TIMEOUT_COUNT | W:D | R,O | T314 timeout counter |
| f(3) | PN_WRAN | FTD_SB_RRC_T315_TIMEOUT_COUNT | W:D | R,O | T315 timeout counter |
| g(3) | PN_WRAN | FTD_SB_RRC_T316_TIMEOUT_COUNT | W:D | R,O | T316 timeout counter |
| h(3) | PN_WRAN | FTD_SB_RRC_T317_TIMEOUT_COUNT | W:D | R,O | T317 timeout counter |

[/FTD_FIELDS]

Display 47.04: Peer Message Count- Screen1 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Count for Peer Messages

Data Display:

+++++++++++++++++++++++++++

+ Peer message count - 1  +

+                         +

+  MIB aaa       PMIB bbb +

+ SIB1 ccc       SIB2 ddd +

+ SIB3 eee      PSIB3 fff +

+ SIB4 ggg      PSIB4 hhh +

+ SIB5 iii       SIB6 jjj +

+ SIB7 kkk       SIB8 lll +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_MIB_COUNT | W:D | R,O | MIB Message Count |
| b(3) | PN_WRAN | FTD_SB_RRC_PMIB_COUNT | W:D | R,O | MIB Message Count |
| c(3) | PN_WRAN | FTD_SB_RRC_SIB1_COUNT | W:D | R,O | SIB1 Message Count |
| d(3) | PN_WRAN | FTD_SB_RRC_SIB2_COUNT | W:D | R,O | SIB2 Message Count |
| e(3) | PN_WRAN | FTD_SB_RRC_SIB3_COUNT | W:D | R,O | SIB3 Message Count |
| f(3) | PN_WRAN | FTD_SB_RRC_PSIB3_COUNT | W:D | R,O | SIB3 Message Count |
| g(3) | PN_WRAN | FTD_SB_RRC_SIB4_COUNT | W:D | R,O | SIB4 Message Count |
| h(3) | PN_WRAN | FTD_SB_RRC_PSIB4_COUNT | W:D | R,O | SIB4 Message Count |
| i(3) | PN_WRAN | FTD_SB_RRC_SIB5_COUNT | W:D | R,O | SIB5 Message Count |
| j(3) | PN_WRAN | FTD_SB_RRC_SIB6_COUNT | W:D | R,O | SIB6 Message Count |
| k(3) | PN_WRAN | FTD_SB_RRC_SIB7_COUNT | W:D | R,O | SIB7 Message Count |
| l(3) | PN_WRAN | FTD_SB_RRC_SIB8_COUNT | W:D | R,O | SIB8 Message Count |

[/FTD_FIELDS]

Display 47.05: Peer Message Count- Screen2 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Peer Message Count

Data Display:

+++++++++++++++++++++++++++

+ Peer message count - 2  +

+                         +

+ SIB9  aaa     SIB15 ggg +

+ SIB10 bbb     SIB16 hhh +

+ SIB11 ccc     SIB17 iii +

+ SIB12 ddd     SIB18 jjj +

+ SIB13 eee               +

+ SIB14 fff               +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_SIB9_COUNT | W:D | R,O | SIB9 Message Count |
| b(3) | PN_WRAN | FTD_SB_RRC_SIB10_COUNT | W:D | R,O | SIB10 Message Count |
| c(3) | PN_WRAN | FTD_SB_RRC_SIB11_COUNT | W:D | R,O | SIB11 Message Count |
| d(3) | PN_WRAN | FTD_SB_RRC_SIB12_COUNT | W:D | R,O | SIB12 Message Count |
| e(3) | PN_WRAN | FTD_SB_RRC_SIB13_COUNT | W:D | R,O | SIB13 Message Count |
| f(3) | PN_WRAN | FTD_SB_RRC_SIB14_COUNT | W:D | R,O | SIB14 Message Count |
| g(3) | PN_WRAN | FTD_SB_RRC_SIB15_COUNT | W:D | R,O | SIB15 Message Count |
| h(3) | PN_WRAN | FTD_SB_RRC_SIB16_COUNT | W:D | R,O | SIB16 Message Count |
| i(3) | PN_WRAN | FTD_SB_RRC_SIB17_COUNT | W:D | R,O | SIB17 Message Count |
| j(3) | PN_WRAN | FTD_SB_RRC_SIB18_COUNT | W:D | R,O | SIB18 Message Count |

[/FTD_FIELDS]

Display 47.06: Peer Message Count-Screen3 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Peer Message Count

Data Display:

+++++++++++++++++++++++++++

+ Peer message count - 3  +

+ Connect request     aaa +

+ Connect reject      bbb +

+ Connect setup       ccc +

+ Connect setup compl ddd +

+ Connect release     eee +

+ Connect rel compl   fff +

+ Paging type1        ggg +

+ Paging type2        hhh +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_CONRQ_COUNT | W:D | R,O | RRC Connection Request Message Count |
| b(3) | PN_WRAN | FTD_SB_RRC_CONREJ_COUNT | W:D | R,O | RRC Connection Reject Message Count |
| c(3) | PN_WRAN | FTD_SB_RRC_CONSET_COUNT | W:D | R,O | RRC Connection Setup Message Count |
| d(3) | PN_WRAN | FTD_SB_RRC_CONSC_COUNT | W:D | R,O | RRC Connection Setup Complete Message Count |
| e(3) | PN_WRAN | FTD_SB_RRC_CONREL_COUNT | W:D | R,O | RRC Connection Release Message Count |
| f(3) | PN_WRAN | FTD_SB_RRC_CNRLC_COUNT | W:D | R,O | RRC Connection Release Complete Message Count |
| g(3) | PN_WRAN | FTD_SB_RRC_PAGE1_COUNT | W:D | R,O | Paging Type1 Message Count |
| h(3) | PN_WRAN | FTD_SB_RRC_PAGE2_COUNT | W:D | R,O | Paging Type2 Message Count |

[/FTD_FIELDS]

Display 47.07: Peer Message Count-Screen4 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Peer Message Count

Data Display:

+++++++++++++++++++++++++++

+ Peer message count – 4  +

+ RB setup        ok/fail +

+   aaa           bbb ccc +

+ RB reconf       ok/fail +

+   ddd           eee fff +

+ RB release      ok/fail +

+   ggg           hhh iii +

+ Transport CH    ok/fail +

+   jjj           kkk lll +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_RBSET_COUNT | W:D | R,O | Radio Bearer Setup Message Count |
| b(3) | PN_WRAN | FTD_SB_RRC_RBSTC_COUNT | W:D | R,O | Radio Bearer Setup Complete Message Count |
| c(3) | PN_WRAN | FTD_SB_RRC_RBSTF_COUNT | W:D | R,O | Radio Bearer Setup Failure Message Count |
| d(3) | PN_WRAN | FTD_SB_RRC_RBRC_COUNT | W:D | R,O | Radio Bearer Reconfiguration Message Count |
| e(3) | PN_WRAN | FTD_SB_RRC_RBRCC_COUNT | W:D | R,O | Radio Bearer Reconfiguration Complete Message Count |
| f(3) | PN_WRAN | FTD_SB_RRC_RBRCF_COUNT | W:D | R,O | Radio Bearer Reconfiguration Failure Message Count |
| g(3) | PN_WRAN | FTD_SB_RRC_RBRL_COUNT | W:D | R,O | Radio Bearer Release Message Count |
| h(3) | PN_WRAN | FTD_SB_RRC_RBRLC_COUNT | W:D | R,O | Radio Bearer Release Complete Message Count |
| i(3) | PN_WRAN | FTD_SB_RRC_RBRLF_COUNT | W:D | R,O | Radio Bearer Release Failure Message Count |
| j(3) | PN_WRAN | FTD_SB_RRC_TCR_COUNT | W:D | R,O | Transport Channel Reconfiguration Message Count |
| k(3) | PN_WRAN | FTD_SB_RRC_TCHRC_COUNT | W:D | R,O | Transport Channel Reconfiguration Complete Message Count |
| l(3) | PN_WRAN | FTD_SB_RRC_TCHRF_COUNT | W:D | R,O | Transport Channel Reconfiguration Failure Message Count |

[/FTD_FIELDS]

Display 47.08: Peer Message Count-Screen5 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Peer Message Count

Data Display:

+++++++++++++++++++++++++++

+ Peer message count – 5  +

+ Physical CH     ok/fail +

+   aaa           bbb ccc +

+ Act set update  ok/fail +

+   ddd           eee fff +

+ HO to UTRAN     ok/fail +

+   ggg           hhh iii +

+ HO from UTRAN   ok/fail +

+   jjj           kkk lll +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_PCRC_COUNT | W:D | R,O | Physical Channel Reconfiguration Message Count |
| b(3) | PN_WRAN | FTD_SB_RRC_PCHRC_COUNT | W:D | R,O | Physical Channel Reconfiguration Complete Message Count |
| c(3) | PN_WRAN | FTD_SB_RRC_PCHRF_COUNT | W:D | R,O | Physical Channel Reconfiguration Failure Message Count |
| d(3) | PN_WRAN | FTD_SB_RRC_ASUP_COUNT | W:D | R,O | Active Set Update Message Count |
| e(3) | PN_WRAN | FTD_SB_RRC_ASUPC_COUNT | W:D | R,O | Active Set Update Complete Message Count |
| f(3) | PN_WRAN | FTD_SB_RRC_ASUPF_COUNT | W:D | R,O | Active Set Update Failure Message Count |
| g(3) | PN_WRAN | FTD_SB_RRC_HOTUT_COUNT | W:D | R,O | Handover to UTRAN Message Count |
| h(3) | PN_WRAN | FTD_SB_RRC_HOTUC_COUNT | W:D | R,O | Handover to UTRAN Complete Message Count |
| i(3) | PN_WRAN | FTD_SB_RRC_HFUG_COUNT | W:D | R,O | Handover from UTRAN Message Count |
| j(3) | PN_WRAN | FTD_SB_RRC_HOFUF_COUNT | W:D | R,O | Handover from UTRAN Failure Message Count |

[/FTD_FIELDS]

Display 47.09: Peer Message Count-Screen6 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Peer Message count

Data Display:

+++++++++++++++++++++++++++

+ Peer message count – 6  +

+                         +

+ CU aaa   CU confirm bbb +

+ CountChk ccc  IDT   ddd +

+ DL_tr    eee  UL_tr fff +

+ MContr ggg   MCFail hhh +

+ MRep   iii   SecMod jjj +

+ SecModComp   kkk        +

+ SecModFail   lll        +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_CELUP_COUNT | W:D | R,O | Cell Update Message Count |
| b(3) | PN_WRAN | FTD_SB_RRC_CUCNF_COUNT | W:D | R,O | Cell Update Confirm Message Count |
| c(3) | PN_WRAN | FTD_SB_RRC_CTCK_COUNT | W:D | R,O | Counter Check Message Count |
| d(3) | PN_WRAN | FTD_SB_RRC_IDT_COUNT | W:D | R,O | Initial Direct Transfer Message Count |
| e(3) | PN_WRAN | FTD_SB_RRC_DDT_COUNT | W:D | R,O | Downlink Direct Transfer Message Count |
| f(3) | PN_WRAN | FTD_SB_RRC_UDT_COUNT | W:D | R,O | Uplink Direct Transfer Message Count |
| g(3) | PN_WRAN | FTD_SB_RRC_MECON_COUNT | W:D | R,O | Measurement Control Message Count |
| h(3) | PN_WRAN | FTD_SB_RRC_MECOF_COUNT | W:D | R,O | Measurement Control Failure Message Count |
| i(3) | PN_WRAN | FTD_SB_RRC_MEREP_COUNT | W:D | R,O | Measurement Report Message Count |
| j(3) | PN_WRAN | FTD_SB_RRC_SECM_COUNT | W:D | R,O | Security Mode Command Message Count |
| k(3) | PN_WRAN | FTD_SB_RRC_SECMC_COUNT | W:D | R,O | Security Mode Complete Message Count |
| l(3) | PN_WRAN | FTD_SB_RRC_SECMF_COUNT | W:D | R,O | Security Mode Failure Message Count |

[/FTD_FIELDS]

Display 47.10: Peer Message Count-Screen7 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows Peer Message Count

Data Display:

+++++++++++++++++++++++++++

+ Peer message count – 7  +

+                         +

+ SG_CR mes/ind   aaa bbb +

+ UE_CE  URA      ccc ddd +

+ RRC  URA_conf   eee fff +

+ UTRAN UTRAN_cf  ggg hhh +

+ UTRAN_fail          iii +

+ CCO_from_UTRAN      jjj +

+ CCO_from_UTRAN_fail kkk +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RRC_SGCR_COUNT | W:D | R,O | Signalling Connection Release Message Count |
| b(3) | PN_WRAN | FTD_SB_RRC_SGCRI_COUNT | W:D | R,O | Signalling Connection Release Indication Message Count |
| c(3) | PN_WRAN | FTD_SB_RRC_UECEQ_COUNT | W:D | R,O | UE Capability Enquiry Message Count |
| d(3) | PN_WRAN | FTD_SB_RRC_URAUP_COUNT | W:D | R,O | URA Update Message Count |
| e(3) | PN_WRAN | FTD_SB_RRC_RRCST_COUNT | W:D | R,O | RRC Status Message Count |
| f(3) | PN_WRAN | FTD_SB_RRC_URAUC_COUNT | W:D | R,O | URA Update Confirm Message Count |
| g(3) | PN_WRAN | FTD_SB_RRC_UTMI_COUNT | W:D | R,O | UTRAN Mobility Information Message Count |
| h(3) | PN_WRAN | FTD_SB_RRC_UMIC_COUNT | W:D | R,O | UTRAN Mobility Information Confirm Message Count |
| i(3) | PN_WRAN | FTD_SB_RRC_UMIF_COUNT | W:D | R,O | UTRAN Mobility Information Failure Message Count |
| j(3) | PN_WRAN | FTD_SB_RRC_CCOFU_COUNT | W:D | R,O | CellChangeOrderFromUTRAN message Count |
| k(3) | PN_WRAN | FTD_SB_RRC_CCOFUF_COUNT | W:D | R,O | CellChangeOrderFromUTRANFailure message Count |

- Group 48: WCDMA RLC displays

Display 48.01: Set AM Bearer ID (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

Set the RLC AM bearer ID for which the AM info will be displayed

Data Display:

+++++++++++++++++++++++++++

+ Set AM bearer ID        +

+                         +

+ Radio bearer: aa        +

+ RLC SAP:      bbbb      +

+ RLC mode:     cc        +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_RLC_FTD_AM_BEARER_SELECT | DW:D | R,O | callback function subblock identity |
| a(2) | PN_WRAN | FTD_SB_RLC_FTD_SELECTED_AM_RADIO_BEARER | B:D | R,O | AM radio bearer for which data is to be displayed. It is uint8 data. It can be 1 - 32. |
| b(4) | PN_WRAN | FTD_SB_RLC_FTD_SAP_USER | S | R,O | RLC SAP user for selected radio bearer. It is string data. It can be "RRC" or "PDCP". |
| c(2) | PN_WRAN | FTD_SB_RLC_FTD_MODE | S | R,O | RLC mode for selected radio bearer. It is string data. It can be "AM" or "UM". |

[/FTD_FIELDS]

Display 48.02: RLC AM State information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen displays state information for the AM bearer selected using display 48.01

Data Display:

+++++++++++++++++++++++++++

+ RLC AM state info       +

+                         +

+ AM uplink status    aaa +

+ AM downlink status  bbb +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RLC_FTD_AM_UL_FLOW_CTRL | S | R,O | AM Uplink flow control status. It is string data. It can be "ON" or "OFF". |
| b(3) | PN_WRAN | FTD_SB_RLC_FTD_AM_DL_FLOW_CTRL | S | R,O | AM Downlink flow control status. It is string data. It can be "ON" or "OFF". |

[/FTD_FIELDS]

Display 48.03: RLC AM Counters 1 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen displays counters for the AM bearer selected using display 48.01

Data Display:

+++++++++++++++++++++++++++

+ RLC AM counters 1       +

+                         +

+ PDU Tx        aaaaaa    +

+ PDU ReTx      bbbbbb    +

+ PDU Rx        cccccc    +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_DATA_PDU_TXD | DW:D | R | Number of the transmitted AMD PDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |
| b(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_DATA_PDU_RETXD | DW:D | R | Number of the re-transmitted AMD PDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |
| c(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_DATA_PDU_RXD | DW:D | R | Number of the received AMD PDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |

[/FTD_FIELDS]

Display 48.04: RLC AM Counters 2 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen displays counters for the AM bearer selected using display 48.01

Data Display:

+++++++++++++++++++++++++++

+ RLC AM counters 2       +

+                         +

+ Ctrl PDU Tx   aaaaaa    +

+ Ctrl PDU Rx   bbbbbb    +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_CTRL_PDU_TXD | DW:D | R | Number of the transmitted Control PDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |
| b(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_CTRL_PDU_RXD | DW:D | R | Number of the received Control PDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |

[/FTD_FIELDS]

Display 48.05: RLC AM Counters 3 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen displays counters for the AM bearer selected using display 48.01

Data Display:

+++++++++++++++++++++++++++

+ RLC AM counters 3       +

+                         +

+ SDU Requested  aaaaaa   +

+ SDU Discarded  bbbbbb   +

+ SDU Acked      cccccc   +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_TX_SDU_REQUESTED | DW:D | R | Number of the requested AM Tx SDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |
| b(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_TX_SDU_DISCARDED | DW:D | R | Number of the discarded AM Tx SDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |
| c(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_TX_SDU_ACKED | DW:D | R | Number of the acknowledged AM Tx SDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |

[/FTD_FIELDS]

Display 48.06: RLC AM Counters 4 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen displays counters for the AM bearer selected using display 48.01

Data Display:

+++++++++++++++++++++++++++

+ RLC AM counters 4       +

+                         +

+ Reject PDU Data  aaaaaa +

+ Reject PDU Ctrl  bbbbbb +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_REJECTED_DATA_PDUS | DW:D | R | Number of the rejected AMD PDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |
| b(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_REJECTED_CTRL_PDUS | DW:D | R | Number of the rejected AM Status PDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |

[/FTD_FIELDS]

Display 48.07: RLC AM Counters 5 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen displays counters for the AM bearer selected using display 48.01

Data Display:

+++++++++++++++++++++++++++

+ RLC AM counters 5       +

+                         +

+ Delivered SDU    aaaaaa +

+ Resets           bbbbbb +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_DELIVERED_SDUS | DW:D | R | Number of the delivered AM SDUs. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |
| b(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_NUM_OF_RESETS | DW:D | R | Number of times of the AM resets. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |

[/FTD_FIELDS]

Display 48.08: RLC AM Counters 6 (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen displays counters for the AM bearer selected using display 48.01

Data Display:

+++++++++++++++++++++++++++

+ RLC AM counters 6       +

+                         +

+ AM re-establish  aaaaaa +

+ AM reconfigs     bbbbbb +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_NUM_OF_REESTABLISHMENTS | DW:D | R | Number of times of the AM re-establishments. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |
| b(6) | PN_WRAN | FTD_SB_RLC_FTD_AM_NUM_OF_RECONFIGS | DW:D | R | Number of times of the AM resets. It is uint32 data displayed as 6 decimal digits. It can be 0 - 4294967295 but displayed 0 - 999999. |

[/FTD_FIELDS]

Display 48.09: RLC AM Tx Buffer Occupancy (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen displays the Tx buffer occupancy (in %) of the AM bearer selected using display 48.01

Data Display:

+++++++++++++++++++++++++++

+ RLC AM Tx Buffer        +

+ Occupancy               +

+                         +

+ AM Tx current     aaa % +

+ AM Tx peak        bbb % +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | PN_WRAN | FTD_SB_RLC_FTD_AM_TX_BFR_OCC | B:D | R | Current buffer occupancy of the AM Tx buffer. It is uint8 data displayed as 3 decimal digits. It can be 0 - 100%. |
| b(3) | PN_WRAN | FTD_SB_RLC_FTD_AM_TX_BFR_OCC_PEAK | B:D | R | Peak buffer occupancy of the AM Tx buffer. It is uint8 data displayed as 3 decimal digits. It can be 0 - 100%. |

[/FTD_FIELDS]

- Group 49: WCDMA MAC displays

Display 49.01: Basic Radio Bearer Information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows the number of active UL and DL Radio Bearers and the RB ID Bit-Fields

Data Display:

+++++++++++++++++++++++++++

+ Basic RB information    +

+                         +

+ Uplink         bb       +

+ Sub ID1        aaaaaaaa +

+ Sub ID2        cccccccc +

+ Downlink       ee       +

+ Sub ID1        dddddddd +

+ Sub ID2        ffffffff +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | PN_WRAN | FTD_SB_W_MAC_UL_RB_SUB_ID_1 | DW:H | R | This field shows the Active Up-Link Radio Bearer Id bit-field,nSub_Id_1, as an 8 digit hexadecimal value. This will have a range ofn0-FFFFFFFF. When this hex value is converted into a bit-field, a 1 willnrepresent an active RB for this SUB ID and 0 an inactive RB for this Sub ID.nThe RB shall be indicated by its position in the bit-field. e.g the Least Significant Bit (LSB) will represent RB 1, the Most Significant Bit (MSB) will represent RB 32. |
| b(2) | PN_WRAN | FTD_SB_W_MAC_NUM_ACTIVE_UL_RB | B:D | R | This field shows the number of active up-link radio bearers having a range of 0-32 represented as an integer |
| c(8) | PN_WRAN | FTD_SB_W_MAC_UL_RB_SUB_ID_2 | DW:H | R | This field shows the Active Up-Link Radio Bearer Id bit-field, Sub_Id_2, as an 8 digit hexadecimal value. This will have a range of 0-FFFFFFFF. When this hex value is converted into a bit-field, a 1 will represent an active RB for this SUB ID and 0 an inactive RB for this Sub ID. The RB shall be indicated by its position in the bit-field. e.g the Least Significant Bit (LSB) will represent RB 1, the Most Significant Bit (MSB) will represent RB 32. |
| d(8) | PN_WRAN | FTD_SB_W_MAC_DL_RB_SUB_ID_1 | DW:H | R | This field shows the Active Down-Link Radio Bearer Id bit-field, Sub_Id_1, as an 8 digit hexadecimal value. This will have a range of 0-FFFFFFFF. When this hex value is converted into a bit-field, a 1 will represent an active RB for this SUB ID and 0 an inactive RB for this Sub ID. The RB shall be indicated by its position in the bit-field. e.g the Least Significant Bit (LSB) will represent RB 1, the Most Significant Bit (MSB) will represent RB 32. |
| e(2) | PN_WRAN | FTD_SB_W_MAC_NUM_ACTIVE_DL_RB | W:D | R | This field shows the number of active down-link radio bearersnhaving a range of 0-32, represented as an integer |
| f(8) | PN_WRAN | FTD_SB_W_MAC_DL_RB_SUB_ID_2 | DW:H | R | This field shows the Active Down-Link Radio Bearer Id bit-field, Sub_Id_2, as an 8 digit hexadecimal value. This will have a range of 0-FFFFFFFF. When this hex value is converted into a bit-field, a 1 will represent an active RB for this SUB ID and 0 an inactive RB for this Sub ID. The RB shall be indicated by its position in the bit-field. e.g the Least Significant Bit (LSB) will represent RB 1, the Most Significant Bit (MSB) will represent RB 32. |

[/FTD_FIELDS]

Display 49.02: Basic DCH Information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows the number of UL and DL DCH's and their corresponding DCH ID Bit-Field

Data Display:

+++++++++++++++++++++++++++

+ Basic DCH information   +

+                         +

+ Uplink DCH           aa +

+ DCH ID         bbbbbbbb +

+ Downlink DCH         cc +

+ DCH ID         dddddddd +

+ Conf DCH             ee +

+ Free DCH ID    ffffffff +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | PN_WRAN | FTD_SB_W_MAC_ACTIVE_UL_DCH | W:D | R | This field shows the number of active up-link DCH's having a range of 0-32, represented as an integer |
| b(8) | PN_WRAN | FTD_SB_W_MAC_ACTIVE_UL_DCH_ID_BITFIELD | DW:H | R | This field shows the Active Up-Link DCH Id bit-field, as an 8 digit hexadecimal value. This will have a range of 0-FFFFFFFF. When this hex value is converted into a bit-field, a 1 will represent an active DCH and a 0 an inactive DCH.nThe DCH shall be indicated by its position in the bit-field. e.g the Least Significant Bit (LSB) will represent DCH 1, the Most Significant Bit (MSB) will represent DCH 32. |
| c(2) | PN_WRAN | FTD_SB_W_MAC_ACTIVE_DL_DCH | W:D | R | This field shows the number of active down-link DCH's having a range of 0-32, represented as an integer |
| d(8) | PN_WRAN | FTD_SB_W_MAC_ACTIVE_DL_DCH_ID_BITFIELD | DW:H | R | This field shows the Active Down-Link DCH Id bit-field, as an 8 digit hexadecimal value. This will have a range of 0-FFFFFFFF. When this hex value is converted into a bit-field, a 1 will represent an active DCH and a 0 an inacative DCH. The DCH shall be indicated by its position in the bit-field. e.g the Least Significant Bit (LSB) will represent DCH 1, the Most Significant Bit (MSB) will represent DCH 32. |
| e(2) | PN_WRAN | FTD_SB_W_MAC_FREE_DCH | W:D | R | This field shows the number of configured DCH's that do not have RB's attached. This field has a range of 0-32, represented as an integer. |
| f(8) | PN_WRAN | FTD_SB_W_MAC_FREE_DCH_ID_BITFIELD | DW:H | R | This field shows the unconnected DCH Id bit-field as an 8 digit hexadecimal value. These are the DCH's that are configured but no UL or DL RB is connected to them. When this hex value is converted into a bit-field, a 1 will represent an active DCH and a 0 an inacative DCH. The DCH shall be indicated by its position in the bit-field. e.g the Least Significant Bit (LSB) will represent DCH 1, the Most Significant Bit (MSB) will represent DCH 32. |

[/FTD_FIELDS]

Display 49.04: Radio Bearer ID and SUB ID input (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shall form the input for the Radio Bearer ID and sub ID. This will be formed of a 2 digit decimal value of the RB ID (XX, range 1-32) followed by a single digit for the Sub ID (Y being either 1 or 2), input as XXY

Data Display:

+++++++++++++++++++++++++++

+ RB ID and SUB ID input  +

+                         +

+ Enter RB ID and SUB ID  +

+ as XXY: aaa             +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_RB_AND_SUBID_SET | DW:D | R | callback function subblock identity |
| a(3) | PN_WRAN | FTD_SB_W_MAC_RB_SUB_ID_INPUT | B:D | R | This 3 digit decimal input requests the 2 digit RB-ID, having a range of 0-32, followed by the single digit Sub-ID, which can be either 1 or 2 |

[/FTD_FIELDS]

Display 49.05: Radio Bearer Information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shall display the Radio Bearer Information including both UL and DL information

Data Display:

+++++++++++++++++++++++++++

+ RB information          +

+                         +

+ ActStatus aa RLC mode b +

+ RLC connect c  Cipher d +

+ Priority e    TCHtype f +

+ TCH_ID  gg    CTstat  h +

+ CTvalue i     Size    j +

+ DEDBR   kk    TCHtype l +

+ ID mm status n  value o +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | PN_WRAN | FTD_SB_W_MAC_RB_ACTIVE_STATUS | S | R | Represents the RB's Active status using 2 characters. Thesencharacters represent the following status:nUL - Up-LinknDL - Down-LinknUD - Up-Link and Down-LinknNO - Representing none of the above i.e. no active status |
| b(1) | PN_WRAN | FTD_SB_W_MAC_RB_RLC_MODE | S | R | The RLC mode is represented by a single character whichnreflects the following valid information on the display about the RB RLCnmode:nM - Main,nV - VoicenC - CSDnL - Loopback |
| c(1) | PN_WRAN | FTD_SB_W_MAC_RB_RLC_CONNECTED_STATUS | S | R | This field represents the RLC Connected status which isnrepresented as a single character and is used to represent the followingninformationnC - ConnectednU - Unconnected |
| d(1) | PN_WRAN | FTD_SB_W_MAC_RB_CIPHERING_STATUS | S | R | This field informs us if ciphering for this radio bearer isnEnabled or Disabled. This is represented as a single character as followsnE - Ciphering EnablednD - Ciphering Disabled |
| e(1) | PN_WRAN | FTD_SB_W_MAC_RB_UL_PRIORITY | B:D | R | This parameter represents the RB UL information priority as andecimal value which has a range of 1-8 |
| f(1) | PN_WRAN | FTD_SB_W_MAC_RB_UL_MAPPED_TCH_TYPE | S | R | The UL Mapped Traffic Channel Type is represented as a singlencharacter as followsnR - RACHnD - DCH |
| g(2) | PN_WRAN | FTD_SB_W_MAC_RB_UL_MAPPED_TCH_ID | W:D | R | This field represents the UL Mapped TCH ID as a 2 digitndecimal value. If the UL Mapped Traffic Channel Type is DCH then thisnhas a range of 1-32 |
| h(1) | PN_WRAN | FTD_SB_W_MAC_RB_UL_CT_STATUS | B:D | R | The UL CT Status flag for the RB is represented as a singlendigit decimal values which shows the following information:n0 - Not Presentn1 - Present |
| i(1) | PN_WRAN | FTD_SB_W_MAC_RB_UL_CT_VALUE | B:H | R | The UL CT Value has a range of 0-15. This is represented on thendisplay as a single digit Hexadecimal Value having a range 0-F |
| j(1) | PN_WRAN | FTD_SB_W_MAC_RB_UL_SIZES_STATUS | S | R | The Sizes Status field is represented as a single characternshowing the following information:nA - All SizesnE - ExplicitnC - Configured |
| k(2) | PN_WRAN | FTD_SB_W_MAC_RB_UL_DEDBR_INSTANCE | B:D | R | This field represents the DEDBR Instance and has the rangen0-63. This is represented as a two digit decimal value on the display |
| l(1) | PN_WRAN | FTD_SB_W_MAC_RB_DL_MAPPED_TCH_TYPE | S | R | The DL Mapped Traffic Channel Type or FACH is representednby a single character which shows the following information:nF - FACHnD - DCH |
| m(2) | PN_WRAN | FTD_SB_W_MAC_RB_DL_MAPPED_TCH_ID | W:D | R | If the DL Mapped traffic Channel type is DCH then thisnfield represents a 32 bit mapped traffic channel ID, having a range of 1-32.nThis is represented as a 2 digit decimal value |
| n(1) | PN_WRAN | FTD_SB_W_MAC_RB_DL_CT_STATUS | B:D | R | The DL CT Status flag for the RB is represented as a singlendigit decimal values which shows the following information:n0 - Not Presentn1 - Present |
| o(1) | PN_WRAN | FTD_SB_W_MAC_RB_DL_CT_VALUE | B:H | R | The DL CT Value has a range of 0-15. This is represented on thendisplay as a single digit Hexadecimal Value having a range 0-F |

[/FTD_FIELDS]

Display 49.06: Input DCH ID (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen forms the input screen for the DCH ID in order to display the information related to the DCH on other display screens. The input is a 2-digit decimal value with a range of 1-32

Data Display:

+++++++++++++++++++++++++++

+ Input DCH ID            +

+                         +

+ Enter DCH ID(xx): aa   +

+                         +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_DCH_ID_SET | DW:D | R | callback function subblock identity |
| a(2) | PN_WRAN | FTD_SB_W_MAC_DCH_ID_INPUT | B:D | R | This field represents a 2 digit deciaml value representing the DCH ID which has a range of 1-32 |

[/FTD_FIELDS]

Display 49.07: DCH Information (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This screen shows the UL and DL DCH information for a DCH ID

Data Display:

+++++++++++++++++++++++++++

+ DCH information         +

+ UL act status   SUB ID1 +

+ a              bbbbbbbb +

+ UL TTI          SUB ID2 +

+ cc             dddddddd +

+ DL act status   SUB ID1 +

+ e              ffffffff +

+ DL TTI          SUB ID2 +

+ gg             hhhhhhhh +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(1) | PN_WRAN | FTD_SB_W_MAC_DCH_UL_ACTIVE_STATUS | S | R | This field represents the DCH UL Active status using a single character showing the following information: A - ActiveI - Inactive |
| b(8) | PN_WRAN | FTD_SB_W_MAC_DCH_UL_RB_SUBID_1 | DW:H | R | This field is represented as an 8-digit hexadecimal value which represents the 32-bit bit-field of the UL RB sub id 1 |
| c(2) | PN_WRAN | FTD_SB_W_MAC_DCH_UL_TTI | B:D | R | This field represents the UL TTI field as a two digit decimal value. The valid values for this field are 10, 20, 40 and 80 ms |
| d(8) | PN_WRAN | FTD_SB_W_MAC_DCH_UL_RB_SUBID_2 | DW:H | R | This field is represented as an 8-digit hexadecimal value which represents the 32-bit bit-field of the UL RB sub id 2 |
| e(1) | PN_WRAN | FTD_SB_W_MAC_DCH_DL_ACTIVE_STATUS | S | R | This field represents the DCH DL Active status using a single character showing the following information: A - ActiveI - Inactive |
| f(8) | PN_WRAN | FTD_SB_W_MAC_DCH_DL_RB_SUBID_1 | DW:H | R | This field is represented as an 8-digit hexadecimal value which represents the 32-bit bit-field of the DL RB sub id 1 |
| g(2) | PN_WRAN | FTD_SB_W_MAC_DCH_DL_TTI | B:D | R | This field represents the DL TTI field as a two digit decimal value. The valid values for this field are 10, 20, 40 and 80 ms |
| h(8) | PN_WRAN | FTD_SB_W_MAC_DCH_DL_RB_SUBID_2 | DW:H | R | This field is represented as an 8-digit hexadecimal value which represents the 32-bit bit-field of the DL RB sub id 2 |

- Group 50: WCDMA WPH displays

Display 50.01: WPH Connection counts (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

Count number of successful & unsuccessful connections. DPCH field also counts number of fallbacks.

Data Display:

+++++++++++++++++++++++++++

+ WPH connection counts   +

+                         +

+ PCCPCH OK/NOK aaaa bbbb +

+ PRACH  OK/NOK cccc dddd +

+ SCCPCH OK/NOK eeee ffff +

+ DPCH   OK/NOK gggg  iii +

+ DPCH fallback       hhh +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | PN_WRAN | FTD_SB_SUCCESSFUL_PCCPCH_CONNECTS | B:H | R | Number of successful PCCPCH connections |
| b(4) | PN_WRAN | FTD_SB_FAILED_PCCPCH_CONNECTS | B:H | R | Number of failed PCCPCH connections |
| c(4) | PN_WRAN | FTD_SB_SUCCESSFUL_PRACH_CONNECTS | B:H | R | Number of successful PRACH connections |
| d(4) | PN_WRAN | FTD_SB_FAILED_PRACH_CONNECTS | B:H | R | Number of failed PRACH connections |
| e(4) | PN_WRAN | FTD_SB_SUCCESSFUL_SCCPCH_CONNECTS | B:H | R | Number of successful SCCPCH connections |
| f(4) | PN_WRAN | FTD_SB_FAILED_SCCPCH_CONNECTS | B:H | R | Number of failed SCCPCH connections |
| g(4) | PN_WRAN | FTD_SB_SUCCESSFUL_DPCH_CONNECTS | B:H | R | Number of successful DPCH connections |
| h(3) | PN_WRAN | FTD_SB_FALLBACK_DPCH_CONNECTS | B:H | R | Number of fallback DPCH connections |
| i(3) | PN_WRAN | FTD_SB_FAILED_FALLBACK_DPCH_CONNECTS | B:H | R | Number of failed DPCH connections |

[/FTD_FIELDS]

Display 50.10: WPH L1 Received Error Codes (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This display presents error information (specifically, error code values) received from L1 by WPH

Data Display:

+++++++++++++++++++++++++++

+ WPH L1 received errors  +

+                         +

+ Process ID         aaaa +

+ L1 error type      bbbb +

+ L1 error subtype   cccc +

+ L1 main code   dddddddd +

+ L1 extra code  eeeeeeee +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | PN_WRAN | FTD_SB_WPH_L1_OS_TASK_ID | W:H | R | Identification number of L1 o/s process |
| b(4) | PN_WRAN | FTD_SB_WPH_L1_ERROR_TYPE | W:H | R | L1 error type |
| c(4) | PN_WRAN | FTD_SB_WPH_L1_ERROR_SUBTYPE | W:H | R | L1 error subtype |
| d(8) | PN_WRAN | FTD_SB_WPH_L1_MAIN_ERROR_CODE | DW:H | R | L1 main error code |
| e(8) | PN_WRAN | FTD_SB_WPH_L1_EXTRA_ERROR_CODE | DW:H | R | L1 extra error code |

[/FTD_FIELDS]

Display 50.11: WPH L1 Received Error Timestamp (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This display presents error information (specifically, timestamps & extra information) received from L1 by WPH

Data Display:

+++++++++++++++++++++++++++

+ WPH L1 received error   +

+ timestamps              +

+                         +

+ DSP timestamp  aaaaaaaa +

+                         +

+ Additional DSP debug    +

+ info                    +

+ bbbb  cccc  dddd  eeee  +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | PN_WRAN | FTD_SB_WPH_L1_TIMESTAMP | DW:H | R | Error code DSP timestamp |
| b(4) | PN_WRAN | FTD_SB_WPH_L1_EXTRA_DEBUG_INFO1 | W:H | R | Additional DSP debug information |
| c(4) | PN_WRAN | FTD_SB_WPH_L1_EXTRA_DEBUG_INFO2 | W:H | R | Additional DSP debug information |
| d(4) | PN_WRAN | FTD_SB_WPH_L1_EXTRA_DEBUG_INFO3 | W:H | R | Additional DSP debug information |
| e(4) | PN_WRAN | FTD_SB_WPH_L1_EXTRA_DEBUG_INFO4 | W:H | R | Additional DSP debug information |

[/FTD_FIELDS]

Display 50.12: WPH L1 PId list before error (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This provides access to the PId list immediately preceeding the event which generated the error. The user is prompted to enter the offset, where 0 corresponds to the most recently saved PId. The specified PID is shown together with the seven immediately preceeding it (4 PID indices are shown to indicate what's on the display, and the ordering). Entering values which are too large will result in the oldest 8 PIDs being shown.

Data Display:

+++++++++++++++++++++++++++

+ WPH L1 PID list before  +

+ error                   +

+                         +

+ PID    ind PID          +

+ aaaa << ii eeee         +

+ bbbb << jj ffff         +

+ cccc kk >> gggg         +

+ dddd ll >> hhhh         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_WPH_L1_GET_PID_INDEX | DW:D | R | callback function subblock identity |
| a(4) | PN_WRAN | FTD_SB_WPH_L1_PID0 | W:H | R | L1 Process id |
| i(2) | PN_WRAN | FTD_SB_WPH_L1_PID_INDEX_ID1 | B:D | R | This indicates the index of the value being pointed at. |
| e(4) | PN_WRAN | FTD_SB_WPH_L1_PID4 | W:H | R | L1 Process id |
| b(4) | PN_WRAN | FTD_SB_WPH_L1_PID1 | W:H | R | L1 Process id |
| j(2) | PN_WRAN | FTD_SB_WPH_L1_PID_INDEX_ID2 | B:D | R | This indicates the index of the value being pointed at. |
| f(4) | PN_WRAN | FTD_SB_WPH_L1_PID5 | W:H | R | L1 Process id |
| c(4) | PN_WRAN | FTD_SB_WPH_L1_PID2 | W:H | R | L1 Process id |
| k(2) | PN_WRAN | FTD_SB_WPH_L1_PID_INDEX_ID3 | B:D | R | This indicates the index of the value being pointed at. |
| g(4) | PN_WRAN | FTD_SB_WPH_L1_PID6 | W:H | R | L1 Process id |
| d(4) | PN_WRAN | FTD_SB_WPH_L1_PID3 | W:H | R | L1 Process id |
| l(2) | PN_WRAN | FTD_SB_WPH_L1_PID_INDEX_ID4 | B:D | R | This indicates the index of the value being pointed at. |
| h(4) | PN_WRAN | FTD_SB_WPH_L1_PID7 | W:H | R | L1 Process id |

[/FTD_FIELDS]

Display 50.13: WPH L1 Rx signal list before error (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This provides access to the Rx Signal list immediately preceeding the event which generated the error. The user is prompted to enter the offset, where 0 corresponds to the most recently saved Rx Signal. The specified Rx Signal is shown together with the seven immediately preceeding it (4 indices are shown to indicate which are on the display, and the ordering). Entering values which are too large will result in the oldest 8 Rx Signals being shown

Data Display:

+++++++++++++++++++++++++++

+ WPH L1 Rx signal list   +

+ before error            +

+                         +

+ ID     ind ID           +

+ aaaa << ii eeee         +

+ bbbb << jj ffff         +

+ cccc kk >> gggg         +

+ dddd ll >> hhhh         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_WPH_L1_GET_RXSIG_INDEX | DW:D | R | callback function subblock identity |
| a(4) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGNALID1 | W:H | R | This displays an internal L1 signal id |
| i(2) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGID_INDEX_ID1 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| e(4) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGNALID5 | W:H | R | This displays an internal L1 signal id |
| b(4) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGNALID2 | W:H | R | This displays an internal L1 signal id |
| j(2) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGID_INDEX_ID2 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| f(4) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGNALID6 | W:H | R | This displays an internal L1 signal id |
| c(4) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGNALID3 | W:H | R | This displays an internal L1 signal id |
| k(2) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGID_INDEX_ID3 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| g(4) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGNALID7 | W:H | R | This displays an internal L1 signal id |
| d(4) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGNALID4 | W:H | R | This displays an internal L1 signal id |
| l(2) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGID_INDEX_ID4 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| h(4) | PN_WRAN | FTD_SB_WPH_L1_RX_SIGNALID8 | W:H | R | This displays an internal L1 signal id |

[/FTD_FIELDS]

Display 50.14: WPH L1 Tx signal list before error (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This provides access to the Tx Signal list immediately preceeding the event which generated the error. The user is prompted to enter the offset, where 0 corresponds to the most recently saved Tx Signal. The specified Tx Signal is shown together with the seven immediately preceeding it (4 indices are shown to indicate which are on the display, and the ordering). Entering values which are too large will result in the oldest 8 Tx Signals being shown

Data Display:

+++++++++++++++++++++++++++

+ WPH L1 Tx signal list   +

+ before error            +

+                         +

+ ID     ind ID           +

+ aaaa << ii eeee         +

+ bbbb << jj ffff         +

+ cccc kk >> gggg         +

+ dddd ll >> hhhh         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_WPH_L1_GET_TXSIG_INDEX | DW:D | R | callback function subblock identity |
| a(4) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGNALID1 | W:H | R | This displays an internal L1 signal id |
| i(2) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGID_INDEX_ID1 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| e(4) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGNALID5 | W:H | R | This displays an internal L1 signal id |
| b(4) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGNALID2 | W:H | R | This displays an internal L1 signal id |
| j(2) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGID_INDEX_ID2 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| f(4) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGNALID6 | W:H | R | This displays an internal L1 signal id |
| c(4) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGNALID3 | W:H | R | This displays an internal L1 signal id |
| k(2) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGID_INDEX_ID3 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| g(4) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGNALID7 | W:H | R | This displays an internal L1 signal id |
| d(4) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGNALID4 | W:H | R | This displays an internal L1 signal id |
| l(2) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGID_INDEX_ID4 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| h(4) | PN_WRAN | FTD_SB_WPH_L1_TX_SIGNALID8 | W:H | R | This displays an internal L1 signal id |

[/FTD_FIELDS]

Display 50.15: WPH L1 list of previous warnings received before error (Version: 6.2, Status: Approved)

[Version history]

Version	Status	Description / Reference

6.2	Approved	Approved / SWAM ADB RL03110358493, Inspection 13-Nov-2003

6.1	Proposal	Added / ISA RM-RIM 010-2694

[/Version history]

This provides access to the previous warning list immediately preceeding the event which generated the error. The user is prompted to enter the offset, where 0 corresponds to the most recently saved warning. The specified warning is shown together with the one immediately preceeding it (2 indices are shown to indicate which are on the display). Entering values which are too large will result in the oldest warnings being shown

Data Display:

+++++++++++++++++++++++++++

+ WPH L1 list of previous +

+ warnings before error   +

+                         +

+ Ind            ee       +

+ Main error     aaaaaaaa +

+ Extra error    bbbbbbbb +

+ Ind            ff       +

+ Main error     cccccccc +

+ Extra error    dddddddd +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | PN_WRAN | FTD_SB_INPUT_WPH_L1_GET_WARN_INDEX | DW:D | R | callback function subblock identity |
| e(2) | PN_WRAN | FTD_SB_WPH_L1_WARNING_INDEX_ID1 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| a(8) | PN_WRAN | FTD_SB_WPH_L1_MAIN_ERROR_CODE1 | DW:H | R | This displays an internal L1 signal id |
| b(8) | PN_WRAN | FTD_SB_WPH_L1_EXTRA_ERROR_CODE1 | DW:H | R | This displays an internal L1 signal id |
| f(2) | PN_WRAN | FTD_SB_WPH_L1_WARNING_INDEX_ID2 | B:D | R | This is used as an aide-memoire for the user; it indicates the index of the signal id being pointed at. |
| c(8) | PN_WRAN | FTD_SB_WPH_L1_MAIN_ERROR_CODE2 | DW:H | R | This displays an internal L1 signal id |
| d(8) | PN_WRAN | FTD_SB_WPH_L1_EXTRA_ERROR_CODE2 | DW:H | R | This displays an internal L1 signal id |

- Group 61: Common EM (Energy Management) displays

Display 61.01: Basic EM information (Version: 3.16, Status: Approved)

+++++++++++++++++++++++++++

+   EM information        +

+ Version no    aaaaaaaaa +

+ Battery bars  b         +

+ Bat sw status ccc       +

+ Battery type  ddd       +

+ Volt / Temp   eeee ffff +

+ Cap / SbyH    gggg hhh  +

+ ChMo / CsSt   iii   jjjj+

+ Carger type   kkk       +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	B-field size changed / Inspection 09-Aug-02, SWAM ADB CH02071530815

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(9) | EM | EM_SB_FTD_VERSION | S | R | EM version number: Eg: 05.04.070 |
| b(2) | EM | EM_SB_FTD_NUMBER_OF_BAT_BARS | B:D | R | Number of battery bars (1 to 8 decimal) |
| c(3) | EM | EM_SB_FTD_STATUS | S | R | Battery monitor SW status: ??? Cha = Charging Mon = Monitoring Rst = Restart batmon4 Set = Settle after charging Res = Reset batmon4 Ser = Service Sto = Stopped Dis = Disable |
| d(3) | EM | EM_SB_FTD_BAT_TYPE | S | R | Battery type: ??? SB = Service battery Ni = Nickel battery Err = Error 4L1 = 4.1 v littium battery 4L2 = 4.2 v littium battery |
| e(4) | EM | EM_SB_FTD_INST_BAT_VOLT | W:D | R | Instantaneous battery voltage in mV (0 to 9999 decimal) |
| f(3) | EM | EM_SB_FTD_CURRENT_BAT_TEMP | W:D | R | Current battery temperature in kelvin (0 to 999 decimal) |
| g(4) | EM | EM_SB_FTD_BAT_CAPACITY | W:D | R | Battery capacity in mAh (0 to 9999 decimal) |
| h(3) | EM | EM_SB_FTD_REMAIN_STBY_HOURS | W:D | R | Remaining standby time to battery low in hours (0 to 999 decimal) |
| i(3) | EM | EM_SB_FTD_PRESENT_CHA_MODE | S | R | Present charging state: Not = Not charging Ini = Initialization TCh = Temperature check LiC = Lithium charging Dis = Disabled Dch = Discharging Dco = Disconnectted Pow = Power Col = Cold charging Hot = Hot charging Mai = Maintenance charging BEr = Battery error NiC = Nickel charging CPa = Control paused CDi = Control disabled |
| j(4) | EM | EM_SB_FTD_CS_STATE | S | R | CS state): ???? Idle NSPS SCAN SIGN = Signalling Call CSD HSCS = HSCSD GPRS Data EDGE |
| k(3) | EM | EM_SB_FTD_PRESENT_CHR_TYPE | S | R | Present charger type: ??? TEr = Temperature error 2Wi = 2 wire charger 3Wi = 3 wire charger AC7 = ACP7 charger VEr = Voltage error CEr = Current error |

Display 61.02: Basic BATMON4 information (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+   BATMON4 information   +

+ Batt voltage  aaaaa     +

+ Stand by time bbbbbb    +

+ TX-off Volt   ccccc     +

+ TX-on Volt    dddd      +

+ Misc flags    eeeee     +

+ VDmf /PowInd  ffff   ggg+

+ Stat/Cal V    hhh   iiii+

+ Blow/PrWarn   jjj    kkk+

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | EM | EM_SB_FTD_INST_BAT_VOLT | W:D | R | Measured instantaneous battery voltage in mV (0 to 9999 decimal). |
| b(6) | EM | EM_SB_FTD_REMAIN_STBY_MINUTES | DW:D | R | Remaining standby time estimate in minutes (0 to 999999 decimal). |
| c(4) | EM | EM_SB_FTD_EST_TX_OFF | W:D | R | Estimated Tx-Off battery voltage in mV (0 to 9999 decimal). |
| d(4) | EM | EM_SB_FTD_MEAS_TX_ON | W:D | R | Measured Tx-On synchronized battery voltage in mV (0 to 9999 decimal). |
| e(2) | EM | EM_SB_FTD_MISC_MON_FLAGS1 | B:H | R | Miscellaneous Battery Monitor flags in bitfields (00 to FF hexadecimal). |
| f(4) | EM | EM_SB_FTD_DMF_BAT_VOLT | W:D | R | Double median filtered battery voltage in mV (0 to 9999 decimal) |
| g(3) | EM | EM_SB_FTD_BAT_POW_IND | S | R | Battery Power Indication: "Ok" "Low" "LFB" (= Low For Boot) "Emp" (= Empty) |
| h(3) | EM | EM_SB_FTD_STATUS | S | R | Battery Monitor internal status: "Cha" = Charging "Mon" = Monitoring "Rst" = Restart batmon4 "Set" = Settle after charging "Res" = Reset batmon4 "Ser" = Service "Stp" = Stopped "Dis" = Disable |
| i(4) | EM | EM_SB_FTD_CALC_VOLT_LEVEL | W:D | R | Calculated voltage level "Check" voltage in mV (0 to 9999 decimal). |
| j(3) | EM | EM_SB_FTD_NUMBER_BAT_LOW_WARN | W:D | R | Number of battery low warnings (0 to 999 decimal). |
| k(3) | EM | EM_SB_FTD_WARN_COUNT | W:D | R | Present warning interval counter (0 to 999 decimal). |

Display 61.03: Advanced BATMON4 algorithm information (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+   Advanced BATMON4      +

+ Dmf voltage    aaaa     +

+ Misc int flags 1 b      +

+ Curr mon est 1 ccccc    +

+ Batmon4 volt   ddd      +

+ Misc int flags 2 e      +

+ Curr mon est 2 fffff    +

+ Phi/elapsed  gggg hhhhhh+

+ Dphi/SbyMin  iiiiijjjjjj+

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | EM | EM_SB_FTD_DMF_BAT_VOLT | W:D | R | Double median filtered battery voltage in mV (0 to 9999 decimal) |
| b | EM | EM_SB_FTD_MISC_MON_INT_FLAGS2 | B:H | R | Miscellaneous BatMon4 internal flags as 4 bit bitfield (0 to F hexadecimal). Fields (bit 0 = lsb): [0] : First Correction flag. [1] : Model Valid flag. [3-2] : Settle Delay Mode. |
| c(5) | EM | EM_SB_FTD_CURRENT_MON_EST1 | W :D | R | Total phone current estimate reported to the Battery Monitor by PSM module in 0.1 mV units (0 to 99999 decimal). |
| d(4) | EM | EM_SB_FTD_MON_VOLT | W:D | R | Current BatMon4 model voltage in mV (0 to 9999 decimal). |
| e | EM | EM_SB_FTD_MISC_MON_INT_FLAGS3 | B:H | R | Misc BatMon4 internal flags as 4 bit bitfield (0 to F hexadecimal). Bitfields (bit 0 = lsb): [0] : 0=Standby, 1=Non-Standby. [3-1] : <Future - TBD>. |
| f(5) |  | EM_SB_FTD_CURRENT_MON_EST2 | W :D | R | Total phone current (ILoad) estimated by BatMon4 in the Non-Standby algorithm part from measured voltage drop in 0.1 mV units (0 to 99999 decimal). |
| g(4) | EM | EM_SB_FTD_PHI_VARIABLE | W:D | R | Present BatMon4 algorithm Phi value in mV (0 to 9999 decimal). |
| h(6) | EM | EM_SB_FTD_ELAPSED_TIME | DW:D | R | Elapsed "Model Time" in minutes (0 to 999999 decimal). |
| i(5) | EM | EM_SB_FTD_DELTA_PHI_VARIABLE | W:D (Signed) | R | Present BatMon4 Phi value in mV (-9999 to +9999 signed decimal). |
| j(6) | EM | EM_SB_FTD_REMAIN_STBY_MINUTES | DW:D | R | BatMon4 remaining standby time estimate in minutes (0 to 999999 decimal). |

Display 61.04: Battery Monitor setup and log information (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+  Battery monitor setup  +

+ Batt impedance aaa      +

+ Charged amount bbbbb    +

+ Full treshold  cccc     +

+ Low treshold   ddddd    +

+ Boot voltage   eeee     +

+ Low warning V  ffff     +

+ Algorithm adj  gg       +

+ Algorithm err  hhhh     +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | EM | EM_SB_FTD_BAT_IMP | W:D | R | Battery impedance in mOhm (0 to 999 decimal). |
| b(5) | EM | EM_SB_FTD_CHARGED_AMOUNT | W:D (Signed) | R | Charged amount (-9999 to +9999 decimal) |
| c(4) | EM | EM_SB_FTD_BAT_FULL_THRES | W:D | R | Battery full voltage threshold used by the BatMon4 algorithm in mV (0 to 9999 decimal). |
| d(4) | EM | EM_SB_FTD_BAT_LOW_THRES | W:D | R | Battery low voltage threshold used by the BatMon4 algorithm in mV (0 to 9999 decimal). |
| e(4) | EM | EM_SB_FTD_BAT_BOOT_VOLT | W:D | R | "Boot" battery voltage used for Battery Monitor initialisation in mV (0 to 9999 decimal). |
| f(4) | EM | EM_SB_FTD_VOLT_AT_FIRST_BAT_LOW | W:D | R | Voltage level at first battery low warning in mV (0 to 9999 decimal). |
| g(2) | EM | EM_SB_FTD_MON_ALG_ADJ | B:H | R | Number of BatMon4 algorithm parameter adjustments since last reset/restart of the BatMon4 algorithm (0 to FF hexadecimal). |
| h(4) | EM | EM_SB_FTD_PARAM_ERRORS | W:H | R | BatMon4 algorithm parameter error mask as bitmap (0000 to FFFF hexadecimal). |

Display 61.05: PSM (Phone State Monitor) information 1 (Version: 3.16, Status: Approved)

+++++++++++++++++++++++++++

+  Phone State Monitor 1  +

+                         +

+ Ext act MSB   aaaaaaaa  +

+                         +

+ Ext act MSB 2 bbbbbbbb  +

+                         +

+ Int act LSB   dddddddd  +

+                         +

+ Int act LSB 2 cccccccc  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	Bit descriptions updated / Inspection 09-Aug-02, SWAM ADB CH02071530815

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | EM | EM_SB_FTD_ACTIVITY_BYTE4 | S | R | External activity information MSB |
| b(8) | EM | EM_SB_FTD_ACTIVITY_BYTE3 | S | R | External activity information 2nd MSB |
| c(8) | EM | EM_SB_FTD_ACTIVITY_BYTE2 | S | R | Internal activity information 2nd LSB |
| d(8) | EM | EM_SB_FTD_ACTIVITY_BYTE1 | S | R | Internal activity information LSB |
|  |  |  |  |  | MSB Bit31 = Data cable active Bit30 = Camera active Bit29 = Cover power active Bit28 = Bluetooth active Bit27-Bit17 = Not used Bit16 = Touchpad active Bit15 = USB active Bit14 = Flashlight active Bit13 = MIDI active Bit12 = ADSP active Bit11 = VRR active Bit10 = IHF active Bit9 = MP3 active Bit8 = GPS active Bit7 = Video active Bit6 = Radio active Bit5 = DC out active Bit4 = Irda active Bit3 = Buzzer active Bit2 = Vibra active Bit1 = Keyboard light active Bit0 = Display light active LSB |

Display 61.06: PSM (Phone State Monitor) information 2 (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+ Phone State Monitor 2   +

+                         +

+ Battery current aaaaa   +

+                         +

+ Phone current   bbbbb   +

+                         +

+ Feature current ccccc   +

+                         +

+ Discharge mAs   ddddd   +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | EM | EM_SB_FTD_BAT_CURRENT | W:D (Signed) | R | Battery current in mA (-2000 to +2000 decimal) |
| b(5) | EM | EM_SB_FTD_PHONE_CURRENT | W:D | R | Phone current in mA (0 to 2000 decimal) |
| c(5) | EM | EM_SB_FTD_FEATURE_CURRENT | W:D | R | Feature current in mA (0 to 2000 decimal) |
| d(5) | EM | EM_SB_FTD_DISCHARGE_AMOUNT | W:D | R | Discharge amount in mAs (0 to 2000 decimal) |

Display 61.07: PSM (Phone State Monitor) information 3 (Version: 3.16, Status: Approved)

+++++++++++++++++++++++++++

+ Phone State Monitor 3   +

+                         +

+ Cs idle       aaa       +

+ Active sw     bbb       +

+ Cs state      cccc      +

+ mCs protocol  ddddd     +

+ Batmon state  eeeee     +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	E-field description updated / Inspection 09-Aug-02, SWAM ADB CH02071530815

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | EM | EM_SB_FTD_CS_IDLEACTIVITY | B:D | R | CS idle activity (0 to 255 decimal) |
| b(3) | EM | EM_SB_FTD_ACTIVE_SW_FEATURE | B:D | R | Active sw features (0 to 255 decimal) |
| c(4) | EM | EM_SB_FTD_CS_STATE | S | R | CS state: ???? Idle NSPS SCAN SIGN = Signalling Call CSD HSCS = HSCSD GPRS Data EDGE |
| d(5) | EM | EM_SB_FTD_CS_PROTOCOL | S | R | CS protocol: ????? GSM TDMA CDMA AMPS PDC WCDMA NMT TETRA |
| e(5) | EM | EM_SB_FTD_BATMON4_STATE | S | R | Batmon4 state: IDLE TALK FEAT DATA TRANS PACKD |

Display 61.08: Basic charging information (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+   Basic charging        +

+ Charging state aaa      +

+ Charger type   bbb      +

+ Inst bat volt  cccc     +

+ Charger checks d        +

+ Chr st/Chr cur eeee ffff+

+ Full/Vchar     g   hhhhh+

+ Ichar/PWM      iiii  jjj+

+ Temp/Chr time  kkk   lll+

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | EM | EM_SB_FTD_PRESENT_CHA_MODE | S | R | Present charging state: Not = Not charging Ini = Initialization TCh = Temperature check LiC = Lithium charging Dis = Disabled Dch = Discharging Dco = Disconnected Pow = Power Col = Cold charging Hot = Hot charging Mai = Maintenance charging BEr = Battery error NiC = Nickel charging CPa = Control paused CDi = Control disabled |
| b(3) | EM | EM_SB_FTD_PRESENT_CHR_TYPE | S | R | Present charger type: ??? TEr = Temperature error 2Wi = 2 wire charger 3Wi = 3 wire charger AC7 = ACP7 charger VEr = Voltage error CEr = Current error |
| c(4) | EM | EM_SB_FTD_CHA_INST_BAT_VOLT | W:D | R | Instantaneous battery voltage in mV (0 to 9999 decimal) |
| d | EM | EM_SB_FTD_NUMBER_CHR_CHECKS | B:D | R | Number of charger checks (0 to 9 decimal) |
| e(4) | EM | EM_SB_FTD_CHR_REC_STATE | S | R | Charger recognition state: ???? NoCh = No charger connected ByCk = Battery type check BeCk = Battery temp check CkV7 = Check VCHAR for ACP7 CkI7 = Check ICHAR from ACP7 CkI8 = Check ICHAR from ACP8 or ACP9 Ck3W = Check for 3 wire charger MxCk = Done maximum number of charger checks ChPa = Charger info paused ChDi = Charger info disabled ChOK= Charger OK ChNo = Charger not OK |
| f(4) | EM | EM_SB_FTD_INST_CHR_CURRENT | W:D | R | Instantaneous charger current in mA (0 to 9999 decimal) |
| g | EM | EM_SB_FTD_BAT_FULL_FLAG | B:D | R | Battery full flag (0 or 1) |
| h(5) | EM | EM_SB_FTD_AVERAGE_CHR_VOLT | W:D | R | Average Vchar in mV (0 to 65535 decimal) |
| i(4) | EM | EM_SB_FTD_CALC_CHR_CURRENT | W:D | R | Calculated Ichar in mA (0 to 9999 decimal) |
| j(3) | EM | EM_SB_FTD_SW_PWM_VALUE | B:D | R | SW PWM value (0 to 255 decimal) |
| k(3) | EM | EM_SB_FTD_CURRENT_BAT_TEMP | W:D | R | Current battery temperature in kelvin (0 to 999 decimal) |
| l(4) | EM | EM_SB_FTD_CHA_TIME | W:D | R | Charge time in 'hhmm' format (85 minutes is e.g. shown as 125) |

Display 61.09: Extended charging information (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+    Extended charging    +

+ Pres chr type aaa       +

+ Prev chr type bbb       +

+ Const ch mode c         +

+ Pres chr mode ddd       +

+ Pr mod / Hpwm eee    fff+

+ Spwm / Opwm   ggg   hhhh+

+ Hpwmf/Over V  i      jjj+

+ Bchr/ Cwpwm   k      lll+

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | EM | EM_SB_FTD_PRESENT_CHR_TYPE | S | R | Present charger type: ??? TEr = Temperature error 2Wi = 2 wire charger 3Wi = 3 wire charger AC7 = ACP7 charger VEr = Voltage error CEr = Current error |
| b(3) | EM | EM_SB_FTD_PREVIOUS_CHR_TYPE | S | R | Previous charger type: ??? TEr = Temperature error 2Wi = 2 wire charger 3Wi = 3 wire charger AC7 = ACP7 charger VEr = Voltage error CEr = Current error |
| c | EM | EM_SB_FTD_CHA_MODE | B:D | R | Constant Voltage Charging method (0 to 9 decimal) |
| d(3) | EM | EM_SB_FTD_PRESENT_CHA_MODE | S | R | Present charging mode: ??? Not = Not charging Ini = Initialization TCh = Temperature check LiC = Lithium charging Dis = Disabled Dch = Discharging Dco = Disconnectted Pow = Power Col = Cold charging Hot = Hot charging Mai = Maintenance charging BEr = Battery error NiC = Nickel charging CPa = Control paused CDi = Control disabled |
| e(3) | EM | EM_SB_FTD_PREVIOUS_CHA_MODE | S | R | Previous charging mode: ??? Not = Not charging Ini = Initialization TCh = Temperature check LiC = Lithium charging Dis = Disabled Dch = Discharging Dco = Disconnectted Pow = Power Col = Cold charging Hot = Hot charging Mai = Maintenance charging BEr = Battery error NiC = Nickel charging CPa = Control paused CDi = Control disabled |
| f(3) | EM | EM_SB_FTD_HW_PWM_VALUE | B:D | R | HW PWM value (0 to 255 decimal) |
| g(3) | EM | EM_SB_FTD_SW_PWM_VALUE | B:D | R | SW PWM value (0 to 255 decimal) |
| h(4) | EM | EM_SB_FTD_OS_PWM_BAT_VOLT | W:D | R | Open switch PWM battery voltage in mV (0 to 9999 decimal) |
| i | EM | EM_SB_FTD_PWM_FREQ | B:D | R | HW PWM frequency (0 to 3 decimal) |
| j(3) | EM | EM_SB_FTD_OVV_LIMIT | S | R | OVer Voltage limit ??? 3v6 = 3.6 volt 5v1 = 5.1 volt 5v3 = 5.3 volt |
| k | EM | EM_SB_FTD_BUB_CHARGING | B:D | R | Back-Up Battery charging status (0 to 2 decimal): 0 = software control (Should not happen) 1 = disabled 2 = enabled |
| l(4) | EM | EM_SB_FTD_CS_PWM_BAT_VOLT | W:D | R | Close switch PWM battery voltage in mV (0 to 9999 decimal) |

Display 61.10: Extended charging information 2 (Version: 3.16, Status: Approved)

+++++++++++++++++++++++++++

+ Open -dv det  aaa       +

+ Slow –dv det  bbb       +

+ -dv over time cccc      +

+ Charge amount ddd       +

+ Bat voltage   eee       +

+ Bat temp      fff       +

+ Fuzzy sum     ggg       +

+ Maint charge  hhh       +

+ Open dv       iii       +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	A small change in help display / Inspection 09-Aug-02, SWAM ADB CH02071530815

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(3) | EM | EM_SB_FTD_OS_DV_DETECT | B:D | R | Open switch -dv detect rule (0 to 100 decimal) |
| b(3) | EM | EM_SB_FTD_SLOW_DV_DETECT | B:D | R | Slow -dv detect rule (0 to 100 decimal) |
| c(3) | EM | EM_SB_FTD_DV_OVER_TIME_DETECT | B:D | R | -dv over time detect rule (0 to 100 decimal) |
| d(3) | EM | EM_SB_FTD_CHARGE_AMOUNT_DETECT | B:D | R | Charge amount detect rule (0 to 100 decimal) |
| e(3) | EM | EM_SB_FTD_BAT_VOLT_DETECT | B:D | R | Battery voltage detect rule (0 to 100 decimal) |
| f(3) | EM | EM_SB_FTD_BAT_TEMP_DETECT | B:D | R | Battery temperature detect rule (0 to 100 decimal) |
| g(4) | EM | EM_SB_FTD_FUZZY_SUM | W:D | R | Fuzzy sum (0 to 9999 decimal) |
| h(3) | EM | EM_SB_FTD_MAINTENANCE_CHA_TIME | W:D | R | Maintenance charging time in seconds (0 to 999 decimal) |
| i(3) | EM | EM_SB_FTD_OS_DV_OVER_TIME | W:D | R | Open switch dv over time in seconds (0 to 999 decimal) |

Display 61.11: Configuration parameters etc. (Version: 3.16, Status: Approved)

+++++++++++++++++++++++++++

+    Config parameters    +

+                         +

+ Time/bat bar  aaaaa     +

+ Bat footprint b         +

+ Conf bat bars c         +

+ Conf min curr ddd       +

+ Conf bat sfty e(4)      +

+ Conf bat emp  f(4)      +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	B-field size changed and description updated / Inspection 09-Aug-02, SWAM ADB CH02071530815

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | EM | EM_SB_FTD_CALC_BAT_BAR | W:D | R | Calculated time per battery bar in minutes (0 to 99999 decimal). |
| b(2) | EM | EM_SB_FTD_CONF_BAT_FOOTPRINT | B:D | R | Id of configured battery footprint. BxB = 1 BxC = 2 BxD = 3 BxN = 4 BL_4C = 11 BL_5C = 12 |
| c | EM | EM_SB_FTD_CONF_BAT_BARS | B:D | R | Configured number of batt. bars. |
| d(3) | EM | EM_SB_FTD_CONF_MIN_CURRENT | W:D | R | Configured minimum standby current for least power consuming phone protocol suppported in 0.1 mA units (0 to 999). |
| e(4) | EM | EM_SB_FTD_CONF_BAT_SAFETY_LEVEL | W:D | R | Configured Battery Monitor battery low "Safety Net" voltage level in mV (0 to 9999 decimal). |
| f(4) | EM | EM_SB_FTD_CONF_BAT_EMPTY_LEVEL | W:D | R | Configured Battery Monitor "Battery Empty" voltage level in mV (0 to 9999 decimal). |

Display 61.12: Varying EM information 1 (Version: 3.15, Status: Approved)

+++++++++++++++++++++++++++

+    Varying Em info 1    +

+ FT display 1            +

+    a(12)                +

+ FT display 2            +

+    b(12)                +

+ FT display 3            +

+    c(12)                +

+ FT display 4            +

+    d(12)                +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	29-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB CH02071530815

3.15	Proposal	11-Jul-02	AnttiE	Display added / RM-RIM tool 5687

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(12) | EM | EM_SB_FTD_STRING1 | S | R | Varying field test display 1 |
| b(12) | EM | EM_SB_FTD_STRING2 | S | R | Varying field test display 2 |
| c(12) | EM | EM_SB_FTD_STRING3 | S | R | Varying field test display 3 |
| d(12) | EM | EM_SB_FTD_STRING4 | S | R | Varying field test display 4 |

Display 61.13: Varying EM information 2 (Version: 3.15, Status: Approved)

+++++++++++++++++++++++++++

+    Varying Em info 2    +

+ FT display 5            +

+    a(12)                +

+ FT display 6            +

+    b(12)                +

+ FT display 7            +

+    c(12)                +

+ FT display 8            +

+    d(12)                +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	29-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB CH02071530815

3.15	Proposal	11-Jul-02	AnttiE	Display added / RM-RIM tool 5687

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(12) | EM | EM_SB_FTD_STRING5 | S | R | Varying field test display 5 |
| b(12) | EM | EM_SB_FTD_STRING6 | S | R | Varying field test display 6 |
| c(12) | EM | EM_SB_FTD_STRING7 | S | R | Varying field test display 7 |
| d(12) | EM | EM_SB_FTD_STRING8 | S | R | Varying field test display 8 |

- Group 62: Common general displays

Display 62.01: Reasons for SW resets (Version: 3.32, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Reason for sw resets    +

+                         +

+ Reset reason            +

+            aaaaaaaaaaaa +

+                         +

+ Task name  bbbbbbbbbbbb +

+ System name       ccccc +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.32	Approved	Approved / Inspection 06-Feb-03, many comments

3.31	Proposal	System field added, system specific DSP reset reasons removed / SWAM ADB AE03011346199

3.28	Proposal	Typo in description field corrected / Inspection 20-Dec-02, SWAM ADB TL02121648021

3.27	Proposal	Reset reasons for ADSP and GSM, TDMA, CDMA and WCDMA DSPs separated. DSP invalid msg reset reason removed / ISA RM-RIM tool 009-1419

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(12) | MON | FTD_SB_LAST_RESET_REASON | S | R | Last reset reason shown in ASCII text. The bold text on the left should be shown when HW_RESET_XXX on the right is returned: UNKNOWN: HW_RESET_UNKNOWN, PWR CHA: HW_RESET_POWER_OFF_CHARGER_CONNECTED, PWR KEY: HW_RESET_POWER_OFF_KEY_PRESSED, SW REQ: HW_RESET_SW_RESET_REQUESTED, INT ERR: HW_RESET_INT_ERROR, STACK: HW_RESET_STACK_OVERFLOW, WD REQ: HW_RESET_WD_RESET_REQUESTED, DIV ZERO: HW_RESET_DIV_BY_ZERO, PRODUCT: HW_RESET_PRODUCTION_RESET, ASSERT: HW_RESET_ASSERTION_FAILED, UNDEF: HW_RESET_UNDEFINED_INSTRUCTION, IO ABORT: HW_RESET_IO_ABORT, NULL ACC: HW_RESET_NULL_ACCESS, ADDERR: HW_RESET_ADDRESS_ERROR, ALIGN: HW_RESET_ALIGNMENT_ERROR, FLASH: HW_RESET_FLASH_WRITE_PROTECTION CBUS: HW_RESET_CBUS_ERROR OFFRTC: HW_RESET_POWER_OFF_RTC OSFATAL: HW_RESET_OS_FATAL_HANDLER FlaBlkOF: HW_RESET_FLASH_BLOCK_OVERFLOW ASIC WD: HW_RESET_ASIC_WD_RESET EPOC: HW_RESET_EPOC_RESET ADSP_START HW_RESET_ADSP_STARTUP_FAILED ADSP RESP HW_RESET_ADSP_NO_RESPONSE ADSP ERR HW_RESET_ADSP_ERROR CDSP_START HW_RESET_CDSP_STARTUP_FAILED CDSP RESP HW_RESET_CDSP_NO_RESPONSE CDSP ERR HW_RESET_CDSP_ERROR HW_RESET_RESERVED Note: reset reasons are not available in string format in some products. In that case a numeric value is shown, see os_trace_data.txt. |
| b(12) | MON | FTD_SB_TASK_BEFORE_RESET | S | R | Name of running task before reset Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt |
| c(5) | MON | FTD_SB_SYSTEM_BEFORE_RESET | S | R | Name of the active system (GSM, TDMA, CDMA, WCDMA) before reset |

[/FTD_FIELDS]

ADSP and CDSP reset reasons:
	STARTUP_FAILED:	DSP startup was attempted, but a timeout occurred
	NO RESPONSE:	DSP stopped responding
	ERROR:		DSP detected a fatal error and asked reset

Display 62.02: Counters for resets 1 (Version: 3.32, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Counters for reset 1    +

+                         +

+ Unknown/Charger   aa bb +

+ Power key/SW      cc dd +

+ Int/Stack         ee ff +

+ WD/Div            gg hh +

+ AWD               ii    +

+ Flash block       mm    +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.32	Approved	Approved / Inspection 06-Feb-03, many comments

3.28	Proposal	FTD_SB_DSP_DEAD_RESET removed / Remained by accident

3.27	Proposal	DSP counters moved to 62.04, DSP invalid msg counter removed / ISA RM-RIM tool 009-1419

3.19	Approved	Formats and a note about l-field added,  / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

3.12	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TL02080538357

3.12	Proposal	07-Jun-02	AnttiE	O-field added / RM tool 2793

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | MON | FTD_SB_UNKNOWN_RESET | W:D | R | UNKNOWN |
| bb | MON | FTD_SB_POFF_CHARGER_CNN | W:D | R | POWER_OFF_CHARGER_CONNECTED (charger connected when powering off) |
| cc | MON | FTD_SB_POFF_KEY | W:D | R | POWER_OFF_KEY_PRESSED (too fast power off/on) |
| dd | MON | FTD_SB_SW_RESET | W:D | R | SW_RESET_REQUESTED |
| ee | MON | FTD_SB_INTERNAL_ERROR | W:D | R | INT_ERROR |
| ff | MON | FTD_SB_STACK_OF_RESET | W:D | R | STACK_OVERFLOW |
| gg | MON | FTD_SB_WATCHDOG_RESET | W:D | R | WD_RESET_REQUESTED |
| hh | MON | FTD_SB_DIV_BY_0_RESET | W:D | R | DIV_BY_ZERO |
| ll | MON | FTD_SB_PPC_ASIC_WD_RESET | B:D | R | ASIC_WD_RESET |
| mm | MON | FTD_SB_FLASH_BLOCK_OVERFLOW | W:D | R | FLASH_BLOCK_OVERFLOW |

[/FTD_FIELDS]

Counters are stored to permanent memory.

These counters are not PPC counters, except FTD_SB_PPC_ASIC_WD_RESET

Display 62.03: Counters for resets 2 (Version: 3.19, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Counters for resets 2   +

+                         +

+ Prod/As/Un     aa bb cc +

+ IO/Null/Ad     dd ee ff +

+ Alig/FlashWrite   gg hh +

+ Cbus           ii       +

+ OSFatal        jj       +

+ Epoc           mm       +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

Version	Status	Description / Reference

3.19	Approved	Formats added, flash overflow reset and dsp startup failed counters removed / Inspection 30-Sep-02, SWAM ADB TL02092341741

3.13	Approved	27-Jun-02	AnttiE	Sub block id for ll field corrected / Inspection 14-Jun-02, SWAM ADB AE020625541901	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | MON | FTD_SB_PRODUCTION_RESET | W:D | R | PRODUCTION_RESET |
| bb | MON | FTD_SB_ASSERTFAIL_RESET | W:D | R | ASSERTION_FAILED |
| cc | MON | FTD_SB_UNDEFINSTR_RESET | W:D | R | UNDEFINED_INSTRUCTION |
| dd | MON | FTD_SB_IO_ABORT_RESET | W:D | R | IO_ABORT |
| ee | MON | FTD_SB_NULLPOINTER_RESET | W:D | R | NULL_ACCESS |
| ff | MON | FTD_SB_ADDR_ERROR_RESET | W:D | R | ADDRESS_ERROR |
| gg | MON | FTD_SB_ALIGNMENT_RESET | W:D | R | ALIGNMENT_ERROR |
| hh | MON | FTD_SB_FLASH_WRITE_RESET | W:D | R | FLASH_WRITE_PROTECTION |
| ii | MON | FTD_SB_CBUS_RESET | W:D | R | CBUS_ERROR |
| jj | MON | FTD_SB_OS_FATAL_RESET | W:D | R | OS_FATAL_HANDLER |
| mm | MON | FTD_SB_EPOC_RESET | W:D | R | EPOC_RESET |

[/FTD_FIELDS]

Counters are stored to permanent memory.

These counters are not PPC counters.

Display 62.04: Counters for DSP resets  (Version: 3.32, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Counters for DSP resets +

+                         +

+ GSM            aa bb cc +

+ TDMA           dd ee ff +

+ CDMA           gg hh ii +

+ WCDMA          jj kk ll +

+ ADSP           mm nn oo +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.32	Approved	Approved / Inspection 06-Feb-03, many comments

3.27	Proposal	Display added  / ISA RM-RIM tool 009-1419

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(2) | MON | FTD_SB_GSM DSP_STARTUP_FAILED | W:D | R | GSM DSP startup fail counter |
| b(2) | MON | FTD_SB_GSM DSP_NO_RESPONSE | W:D | R | GSM DSP no response (dead) counter |
| c(2) | MON | FTD_SB_GSM DSP_ERROR | W:D | R | GSM DSP error counter |
| d(2) | MON | FTD_SB_TDMA DSP_STARTUP_FAILED | W:D | R | TDMA DSP startup fail counter |
| e(2) | MON | FTD_SB_TDMA DSP_NO_RESPONSE | W:D | R | TDMA DSP no response (dead) counter |
| f(2) | MON | FTD_SB_TDMA DSP_ERROR | W:D | R | TDMA DSP error counter |
| g(2) | MON | FTD_SB_CDMA DSP_STARTUP_FAILED | W:D | R | CDMA DSP startup fail counter |
| h(2) | MON | FTD_SB_CDMA DSP_NO_RESPONSE | W:D | R | CDMA DSP no response (dead) counter |
| i(2) | MON | FTD_SB_CDMA DSP_ERROR | W:D | R | CDMA DSP error counter |
| j(2) | MON | FTD_SB_WCDMA DSP_STARTUP_FAILED | W:D | R | WCDMA DSP startup fail counter |
| k(2) | MON | FTD_SB_WCDMA DSP_NO_RESPONSE | W:D | R | WCDMA DSP no response (dead) counter |
| l(2) | MON | FTD_SB_WCDMA DSP_ERROR | W:D | R | WCDMA DSP error counter |
| m(2) | MON | FTD_SB_ADSP_STARTUP_FAILED | W:D | R | ADSP startup fail counter |
| n(2) | MON | FTD_SB_ADSP_NO_RESPONSE | W:D | R | ADSP no response (dead) counter |
| o(2) | MON | FTD_SB_ADSP_ERROR | W:D | R | ADSP DSP error counter |

[/FTD_FIELDS]

Counters are stored to permanent memory.
These counters are not PPCs.
ADSP and CDSP reset reasons:
	STARTUP_FAILED:	DSP startup was attempted, but a timeout occurred
	NO RESPONSE:	DSP stopped responding
	ERROR:		DSP detected a fatal error and asked reset

Display 62.06: Lights status control (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

If selected, lights are always on, otherwise light status is controlled by UI.

Data Display:

+++++++++++++++++++++++++++

+ Lights status control   +

+                         +

+ LIGHTS        XXX       +

+                         +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | LIGHT | FTD_SB_LIGHTS_TOGGLE | N/A | R | Toggle lights on/off |
| xxx | LIGHT | FTD_SB_LIGHTS_STATUS |  | R | Status of lights (ON, OFF) |

Display 62.07: Disabling R&D Test Displays (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

This display can be used to disable R&D Test Displays item from the phone main menu. To disable displays execute this display by selecting R&D test Displays from main menu and entering the display number to the query.

Data Display:

+++++++++++++++++++++++++++

+ R&D Test display        +

+                         +

+ Disable R&D test        +

+  display                +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | GARBAGE | FTD_SELECT_STATE | N/A | R | Disable R&D Test Displays Value: UI_FTD_DISPLAY_OFF |

Display 62.08: Reliability Test Display (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

This display can be used to set phone into test state for reliability stressing. To activate this functionality execute this display by selecting R&D test Displays from main menu and entering the display number to the query.

Data Display:

+++++++++++++++++++++++++++

+ Reliability test disp   +

+                         +

+ Reliability stressing   +

+  on                     +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Message ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | MTC | MTC_STATE_REQ | N/A | R | Set MS to test state Value: MTC_TEST |

Display 62.10: Information about MCU and DSP software versions (Version: 3.23, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ MCU and DSP info        +

+                         +

+ MCU version      aaaaa  +

+ MCU date         bbbbbb +

+ MCU CheckSum     cccc   +

+                         +

+ DSP version             +

+            dddddddddddd +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.23	Approved	Sub block id for c-field corrected / Problem found during implementation by Marko Haajo

3.19	Approved	Abbreviations for c and d fields corrected in data table, approved / Inspection 30-Sep-02, SWAM ADB MH02100437478

3.18	Proposal	B-field moved to 62.12. Display renumbered 62.04 -> 62.10 / ISA RM-RIM tool 8815

Version	Status	Date	Author	Description

3.12	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TL02080538357

3,12	Proposal	07-Jun-02	AnttiE	D-field updated / RM tool 2793

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | INFO | FTD_SB_MCU_VERSION |  | R | Version number of MCU SW (e.g. 2.081) |
| b(6) | INFO | FTD_SB_VERSION_DATE |  | R | Date of version.c Format of the date should be DDMMYY. |
| c(4) | INFO | FTD_SB_MCU_CHECKSUM |  | R | MCU SW checksum |
| d(12) | INFO | FTD_SB_DSP_VERSION |  | R | Version of currently active DSP software (GSM, TDMA, …) |

Display 62.11: Information about HW version (Version: 3.18, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ HW version info         +

+                         +

+ HW: aaaa                +

+                         +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.18	Approved	Display renumbered 62.05 -> 62.11 / ISA RM-RIM tool 8815

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | INFO | FTD_SB_HW_VERSION |  | R | Hardware version |

Display 62.12: Information about PPM (Version: 3.24, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ PPM info                +

+                         +

+ PPM           aaaaaa    +

+ TEXTS         bbbbbbb   +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.24	Approved	Approved / Inspection 11-Nov-02, SWAM ADB MH02110837487

3.18	Proposal	Display added. A-field originates from 62.04 / ISA RM-RIM tool 8815

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(6) | INFO | FTD_SB_PPM_VERSION | S | R | PPM version (e.g. 2.081B) |
| b(7) | INFO | FTD_SB_TEXT_VERSION | S | R | Language package version |

- Group 63: Common OS status displays

Display 63.01: OS Block usage statistics (Blocks 1-8) (Version: 3.19, Status: Approved)

+++++++++++++++++++++++++++

+ OS block usage stat 1-8 +

+ R set1/F set1 aa     bb +

+ R set2/F set2 cc     dd +

+ R set3/F set3 ee     ff +

+ R set4/F set4 gg     hh +

+ R set5/F set5 ii     jj +

+ R set6/F set6 kk     ll +

+ R set7/F set7 mm     nn +

+ R set8/F set8 oo     pp +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

1. row: Block set 1, block set 2

2. row: Block set 3, block set 4

3. row: Block set 5, block set 6

4. row: Block set 7, block set 8

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | MON | FTD_SB_BLOCK_RES_SET1 | B:H | R | Number of reserved blocks, set 1 |
| bb | MON | FTD_SB_FREE_BL_WORST_CASE_SET1 | B:H | R | Number of free blocks in worst case, set 1 |
| cc | MON | FTD_SB_BLOCK_RES_SET2 | B:H | R | Number of reserved blocks, set 2 |
| dd | MON | FTD_SB_FREE_BL_WORST_CASE_SET2 | B:H | R | Number of free blocks in worst case, set 2 |
| ee | MON | FTD_SB_BLOCK_RES_SET3 | B:H | R | Number of reserved blocks, set 3 |
| ff | MON | FTD_SB_FREE_BL_WORST_CASE_SET3 | B:H | R | Number of free blocks in worst case, set 3 |
| gg | MON | FTD_SB_BLOCK_RES_SET4 | B:H | R | Number of reserved blocks, set 4 |
| hh | MON | FTD_SB_FREE_BL_WORST_CASE_SET4 | B:H | R | Number of free blocks in worst case, set 4 |
| ii | MON | FTD_SB_BLOCK_RES_SET5 | B:H | R | Number of reserved blocks, set 5 |
| jj | MON | FTD_SB_FREE_BL_WORST_CASE_SET5 | B:H | R | Number of free blocks in worst case, set 5 |
| kk | MON | FTD_SB_BLOCK_RES_SET6 | B:H | R | Number of reserved blocks, set 6 |
| ll | MON | FTD_SB_FREE_BL_WORST_CASE_SET6 | B:H | R | Number of free blocks in worst case, set 6 |
| mm | MON | FTD_SB_BLOCK_RES_SET7 | B:H | R | Number of reserved blocks, set 7 |
| nn | MON | FTD_SB_FREE_BL_WORST_CASE_SET7 | B:H | R | Number of free blocks in worst case, set 7 |
| oo | MON | FTD_SB_BLOCK_RES_SET8 | B:H | R | Number of reserved blocks, set 8 |
| pp | MON | FTD_SB_FREE_BL_WORST_CASE_SET8 | B:H | R | Number of free blocks in worst case, set 8 |

[/FTD_FIELDS]

Note! The max value of the above fields is FF (255) even thought the OS limits the maximum number of blocks in the block set to FFFF (65535).. FTD will show FF in case of the field value exceeds FF.

Display 63.02: OS Block usage statistics (Blocks 9-16) (Version: 3.19, Status: Approved)

‘+++++++++++++++++++++++++++

+ OS block usage stat 9-16+

+ R set9/F set9 aa     bb +

+ R se10/F se10 cc     dd +

+ R se11/F se11 ee     ff +

+ R se12/F se12 gg     hh +

+ R se13/F se13 ii     jj +

+ R se14/F se14 kk     ll +

+ R se15/F se15 mm     nn +

+ R se16/F se16 oo     pp +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

1. row: Block set 9,  block set 10

2. row: Block set 11, block set 12

3. row: Block set 13, block set 14

4. row: Block set 15, block set 16

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | MON | FTD_SB_BLOCK_RES_SET9 | B:H | R | Number of reserved blocks, set 9 |
| bb | MON | FTD_SB_FREE_BL_WORST_CASE_SET9 | B:H | R | Number of free blocks in worst case, set 9 |
| cc | MON | FTD_SB_BLOCK_RES_SET10 | B:H | R | Number of reserved blocks, set 10 |
| dd | MON | FTD_SB_FREE_BL_WORST_CASE_SET10 | B:H | R | Number of free blocks in worst case, set 10 |
| ee | MON | FTD_SB_BLOCK_RES_SET11 | B:H | R | Number of reserved blocks, set 11 |
| ff | MON | FTD_SB_FREE_BL_WORST_CASE_SET11 | B:H | R | Number of free blocks in worst case, set 11 |
| gg | MON | FTD_SB_BLOCK_RES_SET12 | B:H | R | Number of reserved blocks, set 12 |
| hh | MON | FTD_SB_FREE_BL_WORST_CASE_SET12 | B:H | R | Number of free blocks in worst case, set 12 |
| ii | MON | FTD_SB_BLOCK_RES_SET13 | B:H | R | Number of reserved blocks, set 13 |
| jj | MON | FTD_SB_FREE_BL_WORST_CASE_SET13 | B:H | R | Number of free blocks in worst case, set 13 |
| kk | MON | FTD_SB_BLOCK_RES_SET14 | B:H | R | Number of reserved blocks, set 14 |
| ll | MON | FTD_SB_FREE_BL_WORST_CASE_SET14 | B:H | R | Number of free blocks in worst case, set 14 |
| mm | MON | FTD_SB_BLOCK_RES_SET15 | B:H | R | Number of reserved blocks, set 15 |
| nn | MON | FTD_SB_FREE_BL_WORST_CASE_SET15 | B:H | R | Number of free blocks in worst case, set 15 |
| oo | MON | FTD_SB_BLOCK_RES_SET16 | B:H | R | Number of reserved blocks, set 16 |
| pp | MON | FTD_SB_FREE_BL_WORST_CASE_SET16 | B:H | R | Number of free blocks in worst case, set 16 |

Display 63.03: OS block allocation errors (Version: 3.32, Status: Approved)

+++++++++++++++++++++++++++

+  OS block alloc errors  +

+ Double deallo aaaaaa    +

+ Failed deallo bbb       +

+ D deallo name cccccccc  +

+ Overwrites    dd        +

+ Out of memory ee        +

+ Block pointer fff       +

+ Mod blk point gg        +

+ Big blk count hh        +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.32	Approved	Added a note about task name unavailability / Inspection 06-Feb-03, SWAM ADB PA03013144161

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | MON | FTD_SB_POINTER_TO_DEALLOC | DW:H | R | Pointer to memory where double deallocation was called, in hex format. Valid only when double dellocation has happened. |
| bbb | MON | FTD_SB_PPC_DMM_OVER_DEALLOC | W:D | R | Counter for failed deallocations. |
| c(12) | MON | FTD_SB_TASK_NAME_DOUBLE_DEALLOC | S | R | Name of task that last tried to double deallocate a block. Valid only when double dellocation has happened. Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt |
| dd | MON | FTD_SB_DYNMEM_OVERWRITE | W:D | R | Counter for overwrites |
| ee | MON | FTD_SB_DYNMEM_EXHAUSTED | W:D | R | Counter for out of memory |
| fff | MON | FTD_SB_OS_INTERNAL_ERROR_COUNT | W:D | R | Counter for internal OS errors |
| gg | MON | FTD_SB_MODIFIED_POINTER | W:D | R | Counter for modified block pointer |
| hh | MON | FTD_SB_DYNMEM_BIG_BLOCK | W:D | R | Counter for too big block |

[/FTD_FIELDS]

Note: This display is only valid when the counter for failed deallocations is not zero.

Display 63.04: Memory status before reset (Version: 3.19, Status: Approved)

+++++++++++++++++++++++++++

+     Memory status       +

+     before reset        +

+                         +

+ Stack status aaaaaaaaa  +

+                         +

+                         +

+ Block status aaaaaaaaa  +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(12) | MON | FTD_SB_BLOCK_SET_STATUS_1 | s | R | Status of each block set before reset. First position contains the status of block set 1, second position the status of block set 2 and so on. Possible values for each block set are: 0 : status OK 1 : block set full 2 : (de)allocation error or total memory corruption Note: This display is only valid when a unknown or a stack overflow interrupt has occurred. |
| b(12) | MON | FTD_SB_BLOCK_SET_STATUS_2 | s | R | See FTD_SB_BLOCK_SET_STATUS_1 description. |

Display 63.05: OS pre-reset error status (Version: 3.32, Status: Approved)

+++++++++++++++++++++++++++

+ OS pre-reset error stat +

+                         +

+ Status code   st        +

+ Os func id    fn        +

+ Task name     aaaaaa    +

+                         +

+ Task 1        xxxxxxxx  +

+ Task 2        yyyyyyyy  +

+ Task 3        zzzzzzzz  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.32	Approved	Corrected a typo in x-field sub block id / Inspection 06-Feb-03, SWAM ADB PA03013144161

3.31	Approved	Added a note about task name unavailability / Meeting 13-Jan-03, SWAM ADB AE03011346199

3.19	Approved	Formats added, field descriptions improved / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| st | MON | FTD_SB_OS_PRE_RESET_ERROR_STATUS | B:H | R | Status code byte, 00 == no error, other values in os_trace_data.txt (OS_RESPONSES) |
| fn | MON | FTD_SB_OS_PRE_RESET_ERROR_FUNCTION_ID | B:D | R | OS function id where error/warn/remark occurred, values in os_trace_data.txt (MONITOR_HOOK_IDS) |
| a(6) | MON | FTD_SB_OS_PRE_RESET_ERROR_TASK | S | R | Task name Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt |
| x(8) | MON | FTD_SB_OS_PRE_RESET_ERROR_PARAMETER_1 | DW:H | R | Parameter 1, 32-bit hex number. See dct4_osmo_interpretation.txt |
| y(8) | MON | FTD_SB_OS_PRE_RESET_ERROR_PARAMETER_2 | DW:H | R | Parameter 2, 32-bit hex number. See dct4_osmo_interpretation.txt |
| z(8) | MON | FTD_SB_OS_PRE_RESET_ERROR_PARAMETER_3 | DW:H | R | Parameter 3, 32-bit hex number. See dct4_osmo_interpretation.txt |

Display 63.06: OS current error status (Version: 3.31, Status: Approved)

+++++++++++++++++++++++++++

+ Os current error status +

+                         +

+ Status code   st        +

+ Os func id    fn        +

+ Task name     aaaaaa    +

+                         +

+ Task 1        xxxxxxxx  +

+ Task 2        yyyyyyyy  +

+ Task 3        zzzzzzzz  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.31	Approved	Added a note about task name unavailability / Meeting 13-Jan-03, SWAM ADB AE03011346199

3.19	Approved	Formats added, field descriptions improved / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| st | MON | FTD_SB_OS_CURRENT_ERROR_STATUS | B:H | R | Status code byte, 00 == no error, other values in os_trace_data.txt (OS_RESPONSES) |
| fn | MON | FTD_SB_OS_CURRENT_ERROR_FUNCTION_ID | B:D | R | OS function id where error/warn/remark occurred, values in os_trace_data.txt (MONITOR_HOOK_IDS) |
| a(6) | MON | FTD_SB_OS_CURRENT_ERROR_TASK | S | R | Task name Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt |
| x(8) | MON | FTD_SB_OS_CURRENT_ERROR_PARAMETER_1 | DW:H | R | Parameter 1, 32-bit hex number. See dct4_osmo_interpretation.txt |
| y(8) | MON | FTD_SB_OS_CURRENT_ERROR_PARAMETER_2 | DW:H | R | Parameter 2, 32-bit hex number. See dct4_osmo_interpretation.txt |
| z(8) | MON | FTD_SB_OS_CURRENT_ERROR_PARAMETER_3 | DW:H | R | Parameter 3, 32-bit hex number. See dct4_osmo_interpretation.txt |

Display 63.07: OS pre-reset warning status (Version: 3.31, Status: Approved)

+++++++++++++++++++++++++++

+ Os pre-reset warn status+

+                         +

+ Status code   st        +

+ Os func id    fn        +

+ Task name     aaaaaa    +

+                         +

+ Task 1        xxxxxxxx  +

+ Task 2        yyyyyyyy  +

+ Task 3        zzzzzzzz  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.31	Approved	Added a note about task name unavailability / Meeting 13-Jan-03, SWAM ADB AE03011346199

3.19	Approved	Formats added, field descriptions improved / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| st | MON | FTD_SB_OS_PRE_RESET_WARN_STATUS | B:H | R | Status code byte, 00 == no error, other values in os_trace_data.txt (OS_RESPONSES) |
| fn | MON | FTD_SB_OS_PRE_RESET_WARN_FUNCTION_ID | B:D | R | OS function id where error/warn/remark occurred, values in os_trace_data.txt (MONITOR_HOOK_IDS) |
| a(6) | MON | FTD_SB_OS_PRE_RESET_WARN_TASK | S | R | Task name Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt. |
| x(8) | MON | FTD_SB_OS_PRE_RESET_WARN_PARAMETER_1 | DW:H | R | Parameter 1, 32-bit hex number. See dct4_osmo_interpretation.txt |
| y(8) | MON | FTD_SB_OS_PRE_RESET_WARN_PARAMETER_2 | DW:H | R | Parameter 2, 32-bit hex number. See dct4_osmo_interpretation.txt |
| z(8) | MON | FTD_SB_OS_PRE_RESET_WARN_PARAMETER_3 | DW:H | R | Parameter 3, 32-bit hex number. See dct4_osmo_interpretation.txt |

Display 63.08: OS current warning status (Version: 3.31, Status: Approved)

+++++++++++++++++++++++++++

+ Os current warning status +

+                         +

+ Status code   st        +

+ Os func id    fn        +

+ Task name     aaaaaa    +

+                         +

+ Task 1        xxxxxxxx  +

+ Task 2        yyyyyyyy  +

+ Task 3        zzzzzzzz  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.31	Approved	Added a note about task name unavailability / Meeting 13-Jan-03, SWAM ADB AE03011346199

3.19	Approved	Formats added, field descriptions improved / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| st | MON | FTD_SB_OS_CURRENT_WARN_STATUS | B:H | R | Status code byte, 00 == no error, other values in os_trace_data.txt (OS_RESPONSES) |
| fn | MON | FTD_SB_OS_CURRENT_WARN_FUNCTION_ID | B:D | R | OS function id where error/warn/remark occurred, values in os_trace_data.txt (MONITOR_HOOK_IDS) |
| a(6) | MON | FTD_SB_OS_CURRENT_WARN_TASK | S | R | Task name Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt |
| x(8) | MON | FTD_SB_OS_CURRENT_WARN_PARAMETER_1 | DW:H | R | Parameter 1, 32-bit hex number. See dct4_osmo_interpretation.txt |
| y(8) | MON | FTD_SB_OS_CURRENT_WARN_PARAMETER_2 | DW:H | R | Parameter 2, 32-bit hex number. See dct4_osmo_interpretation.txt |
| z(8) | MON | FTD_SB_OS_CURRENT_WARN_PARAMETER_3 | DW:H | R | Parameter 3, 32-bit hex number. See dct4_osmo_interpretation.txt |

Display 63.09: OS pre-reset remark status (Version: 3.31, Status: Approved)

+++++++++++++++++++++++++++

+ Os pre-reset remark status +

+                         +

+ Status code   st        +

+ Os func id    fn        +

+ Task name     aaaaaa    +

+                         +

+ Task 1        xxxxxxxx  +

+ Task 2        yyyyyyyy  +

+ Task 3        zzzzzzzz  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.31	Approved	Added a note about task name unavailability / Meeting 13-Jan-03, SWAM ADB AE03011346199

3.19	Approved	Formats added, field descriptions improved / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| st | MON | FTD_SB_OS_PRE_RESET_REM_STATUS | B:H | R | Status code byte, 00 == no error, other values in os_trace_data.txt (OS_RESPONSES) |
| fn | MON | FTD_SB_OS_PRE_RESET_REM_FUNCTION_ID | B:D | R | OS function id where error/warn/remark occurred, values in os_trace_data.txt (MONITOR_HOOK_IDS) |
| a(6) | MON | FTD_SB_OS_PRE_RESET_REM_TASK | S | R | Task name Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt |
| x(8) | MON | FTD_SB_OS_PRE_RESET_REM_PARAMETER_1 | DW:H | R | Parameter 1, 32-bit hex number. See dct4_osmo_interpretation.txt |
| y(8) | MON | FTD_SB_OS_PRE_RESET_REM_PARAMETER_2 | DW:H | R | Parameter 2, 32-bit hex number. See dct4_osmo_interpretation.txt |
| z(8) | MON | FTD_SB_OS_PRE_RESET_REM_PARAMETER_3 | DW:H | R | Parameter 3, 32-bit hex number. See dct4_osmo_interpretation.txt |

Display 63.10: OS current remark status (Version: 3.31, Status: Approved)

+++++++++++++++++++++++++++

+ Os current remark status +

+                         +

+ Status code   st        +

+ Os func id    fn        +

+ Task name     aaaaaa    +

+                         +

+ Task 1        xxxxxxxx  +

+ Task 2        yyyyyyyy  +

+ Task 3        zzzzzzzz  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.31	Approved	Added a note about task name unavailability / Meeting 13-Jan-03, SWAM ADB AE03011346199

3.19	Approved	Formats added, field descriptions improved / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| st | MON | FTD_SB_OS_CURRENT_REM_STATUS | B:H | R | Status code byte, 00 == no error, other values in os_trace_data.txt (OS_RESPONSES) |
| fn | MON | FTD_SB_OS_CURRENT_REM_FUNCTION_ID | B:D | R | OS function id where error/warn/remark occurred, values in os_trace_data.txt (MONITOR_HOOK_IDS) |
| a(6) | MON | FTD_SB_OS_CURRENT_REM_TASK | S | R | Task name Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt |
| x(8) | MON | FTD_SB_OS_CURRENT_REM_PARAMETER_1 | DW:H | R | Parameter 1, 32-bit hex number. See dct4_osmo_interpretation.txt |
| y(8) | MON | FTD_SB_OS_CURRENT_REM_PARAMETER_2 | DW:H | R | Parameter 2, 32-bit hex number. See dct4_osmo_interpretation.txt |
| z(8) | MON | FTD_SB_OS_CURRENT_REM_PARAMETER_3 | DW:H | R | Parameter 3, 32-bit hex number. See dct4_osmo_interpretation.txt |

Display 63.11: Heap system status (Version: 3.19, Status: Approved)

+++++++++++++++++++++++++++

+   Heap system status    +

+ Used bytes    (a7)      +

+ Used blocks   b(4)      +

+ Free bytes    c(7)      +

+ Free blocks   d(4)      +

+ Zero alloc    e(2)      +

+ Double alloc  f(2)      +

+ Invalid point g(2)      +

+ Co blk/Chk fa h(2)  i(2)+

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(7) | MON | FTD_SB_HEAP_USED_BYTES | DW:D | R | Used bytes |
| b(4) | MON | FTD_SB_HEAP_USED_BLOCKS | W:D | R | Used block count |
| c(7) | MON | FTD_SB_HEAP_FREE_BYTES | DW:D | R | Free bytes |
| d(4) | MON | FTD_SB_HEAP_FREE_BLOCKS | W:D | R | Free block count |
| ee | MON | FTD_SB_HEAP_ZERO_ALLOCATES | B:D | R | Count of zero allocations |
| ff | MON | FTD_SB_HEAP_DOUBLE_DEALLOC | B:D | R | Count of double deallocations |
| gg | MON | FTD_SB_HEAP_INVALID_POINTER | B:D | R | Count of invalid pointers |
| hh | MON | FTD_SB_HEAP_CORRUPT_BLOCK | B:D | R | Count of corrupt blocks |
| ii | MON | FTD_SB_HEAP_PNTRCHK_FAIL | DW :D | R | Total count of pointer check failures |

Display 63.12: Heap top users (Version: 3.31, Status: Approved)

+++++++++++++++++++++++++++

+     Heap top users      +

+ Task name     a(7)      +

+ Blocks used   b(4)      +

+ Task name     c(7)      +

+ Blocks used   d(4)      +

+ Task name     e(7)      +

+ Blocks used   f(4)      +

+ Task name     g(7)      +

+ Blocks used   h(4)      +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.31	Approved	Added a note about task name unavailability / Meeting 13-Jan-03, SWAM ADB AE03011346199

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(7) | MON | FTD_SB_HEAP_MEM_OWNER_TASK_1 | S | R | Task name |
| b(4) | MON | FTD_SB_HEAP_MEM_OWNER_BLOCKS_1 | B:D | R | Number of blocks used |
| c(7) | MON | FTD_SB_HEAP_MEM_OWNER_TASK_2 | S | R | Task name |
| d(4) | MON | FTD_SB_HEAP_MEM_OWNER_BLOCKS_2 | B:D | R | Number of blocks used |
| e(7) | MON | FTD_SB_HEAP_MEM_OWNER_TASK_3 | S | R | Task name |
| f(4) | MON | FTD_SB_HEAP_MEM_OWNER_BLOCKS_3 | B:D | R | Number of blocks used |
| g(7) | MON | FTD_SB_HEAP_MEM_OWNER_TASK_4 | S | R | Task name |
| h(4) | MON | FTD_SB_HEAP_MEM_OWNER_BLOCKS_4 | B:D | R | Number of blocks used |

[/FTD_FIELDS]

Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt.

Display 63.13: Failed assertions (Version: 3.19, Status: Approved)

+++++++++++++++++++++++++++

+    Failed assertions    +

+                         +

+ Filename     aaaaaaaaaaa+

+                         +

+  Line        bbbbb      +

+                         +

+ Counter for             +

+ failed asser  ccccc     +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(12) | MON | FTD_SB_ASSERT_LATEST_FILE | S | R | Filename, where assertion happened |
| b(5) | MON | FTD_SB_ASSERT_LATEST_LINE | DW:D | R | Line, where assertion happened |
| c(5) | MON | FTD_SB_ASSERT_TOTAL_COUNT | W:D | R | Counter for failed assertions |

Display 63.14-63.29: Reserved for future use (Version: 1, Status: - )

These displays are reserved for future OS displays.

Display 63.30: Information of OS_SYSTEM_STACK (Version: 3.19, Status: Approved)

+++++++++++++++++++++++++++

+    OS_SYSTEM_STACK      +

+                         +

+ FIQ stack     aaaa      +

+                         +

+ IRQ stack     bbbb      +

+                         +

+ FIQ reset     e         +

+                         +

+ IRQ reset     f         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

Version	Status	Description / Reference

3.19	Approved	Sub block naming convention for e- and f-fields changed, formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | MON | FTD_SB_STACK_STATUS_ SYSTEM_1 | DW:D | R | FIQ stack status (OS_SYSTEM_STACK_1) |
| bbbb | MON | FTD_SB_STACK_STATUS_ SYSTEM_2 | DW:D | R | IRQ stack status (OS_SYSTEM_STACK_2) |
| e | MON | FTD_SB_STACK_PRE_ RESET_STATUS_SYSTEM_1 | B:D | R | Shows the status of FIQ stack just before reset. 0 : status OK, no overflow 1 : status not OK, stack overflow |
| f | MON | FTD_SB_STACK_PRE_ RESET_STATUS_SYSTEM_2 | B:D | R | Shows the status of IRQ stack just before reset: 0 : status OK, no overflow 1 : status not OK, stack overflow |

[/FTD_FIELDS]

Note! Values are not stored to permanent memory.

Display 63.31: Select task set (Version: 3.19, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.19	Approved	Approved / Inspection 30-Sep-02, SWAM ADB TL02092341741

3.18	Proposal	Display added, old display with the same number removed / ISA RM-RIM tool 9425

[/Version history]

This displays toggles task set that is shown in displays 63.32 – 63.99. To toggle the set:

- Press menu-button.

- Scroll in the main menu to loop to field test display item.

- Press Select-button

- Select this display in executive mode by entering 6331 at the query prompt.

- Confirm with the Ok button.

- Task set is toggled and the resulting set is shown in the display

Note 1: If the display is selected using arrow buttons, then the task set selection is not toggled.

Note 2: This display may have been configured off in products that have less than 69 tasks.

***************************

*     Execute   display   *

* This displays toggles task          *

* set that is shown in displays    *

* 63.32 – 63.99. Use options to     *

* change.                 *

* 	                *

*                         *

*  Current task     a     *

*                         *

***************************

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | MON | FTD_SB_TASK_SET_TOGGLE | N/A | R | Toggle OS task set. |
| a | MON | FTD_SB_TASK_SET_STATUS | B:D | R | Current task set selection for displays 63.32 – 63.99 1 = tasks 0 – 67 2 = tasks 68 - 135 |

Displays 63.32 – 63.99: Information about tasks (Version: 3.31, Status: Approved)

[Version history]

Version	Status	Description / Reference

3.31	Approved	Added a note about task name unavailability / Meeting 13-Jan-03, SWAM ADB AE03011346199

3.19	Approved	Approved / Inspection 30-Sep-02, SWAM ADB TL02092341741

3.18	Proposal	Sub block naming convention modified / Message macro problem, Toni Laivo

Added support for two task sets / ISA RM-RIM tool 9425.

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

These displays show information either about tasks 0 … 67 or tasks 68 – 136 depending on which task set is currently selected, see display 63.31.

+++++++++++++++++++++++++++

+    Task information     +

+                         +

+ Task name     a(12)     +

+ Stack worst   b(5)      +

+ W mailbox     c(5)      +

+ F mailbox     d(5)      +

+ stack overflow e        +

+                         +

+                         +

+++++++++++++++++++++++++++

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(12) | MON | FTD_SB_TASKNAME_X | S | R | Task Name (ASCII) Note: task names are not available in some products. In that case task number is shown, see os_trace_data.txt |
| b(5) | MON | FTD_SB_STACK_STATUS_TASK_X | DW:D | R | Number tells how many stack memory locations have been empty in the worst case. So, if number is zero, stack has been full. Values are not stored to permanent memory. |
| c(5) | MON | FTD_SB_MSGBUF _STATUS_TASK_X | B:D | R | Worst case Mailbox status of task x (x is the task id) i.e. how much there is queue "places" used in worst case. |
| d(5) | MON | FTD_SB_FASTBUF_STATUS_TASK_X | B:D | R | Fast mailbox status of task x (x is the task id) i.e. how much there is queue "places" used in worst case. |
| e | MON | FTD_SB_STACK_PRE_RESET_STATUS_X | B:D | R | Tells whether stack overflow occurred before reset was done. (x is the task id) 0 : status OK, no overflow 1 : status not OK, stack overflow |

[/FTD_FIELDS]

where x = 0 … 67 and corresponds either to task x or task x + 68 depending on selected task set.

- Group 64: Common Audio displays

Display 64.01: Read DSP Memory (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Read DSP memory         +

+                         +

+ DSP fields   aaaaa bbbb +

+                         +

+ DSP 1-7      cccc  dddd +

+              eeee  ffff +

+              gggg  hhhh +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Formats added / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | DSP_AUDIO | FTD_SB_READ_DSP_ADDR | DW:H | R | Address field for reading DSP memory. Hex format, 24 Bits. |
| a(6) | DSP_AUDIO | FTD_SB_READ_DSP_ADDR_1 | DW:H | R | Address field for reading DSP memory. Hex format, 24 bits. |
| b(4) | DSP_AUDIO | FTD_SB_READ_DSP_MEM_1 | W:H | R | Current contents of address a(6) in DSP memory |
| c(4) | DSP_AUDIO | FTD_SB_READ_DSP_ MEM_2 | W:H | R | Current contents of address a(6) + 1 in DSP memory |
| d(4) | DSP_AUDIO | FTD_SB_READ_DSP_ MEM_3 | W:H | R | Current contents of address a(6) + 2 in DSP memory |
| e(4) | DSP_AUDIO | FTD_SB_READ_DSP_ MEM_4 | W:H | R | Current contents of address a(6) + 3 in DSP memory |
| f(4) | DSP_AUDIO | FTD_SB_READ_DSP_ MEM_5 | W:H | R | Current contents of address a(6) + 4 in DSP memory |
| g(4) | DSP_AUDIO | FTD_SB_READ_DSP_ MEM_6 | W:H | R | Current contents of address a(6) + 5 in DSP memory |
| h(4) | DSP_AUDIO | FTD_SB_READ_DSP_ MEM_7 | W:H | R | Current contents of address a(6) + 6 in DSP memory |

[/FTD_FIELDS]

When display is selected, address fields can be edited. Keyboard is in "text"-mode, so hex numbers 0-F can be written. Data display shows the addresses and their contents. After editing of addresses is completed, request for reading specified memory locations is sent. These values are updated 5-10 times/second.

Display 64.02: Signal generators (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Signal generators       +

+                         +

+ Signal level: -aa.adB   +

+ Frequency:     bbbb     +

+ Routing:       cc       +

+ Wave form:     dd       +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Formats added / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | DSP_AUDIO | FTD_SB_BASIC_SIGGEN_INPUT | DW:D | R | Test input value (for frequence) |
| aa.a | DSP_AUDIO | FTD_SB_BASIC_SIGGEN_LEVEL | S | R | Level of the test signal, dB from maximum. Decimal format. |
| bbbb | DSP_AUDIO | FTD_SB_BASIC_SIGGEN_FREQ | W:D | R | Frequency of the test signal. Decimal format. |
| cc | DSP_AUDIO | FTD_SB_BASIC_SIGGEN_ROUTE | B:D | R | Routing of the test signal. |
| dd | DSP_AUDIO | FTD_SB_BASIC_SIGGEN_WAVE_FORM | B:D | R | Wave form of the test signal. |

[/FTD_FIELDS]

This display can be used to control test signal generators in the DSP. When display is selected, fields can be edited. When editing is completed, a request to start the generators is sent. Because FTD-application is uncapable to handle more than one iput value per FTD, only frequency input is used at the beginning.

Display 64.03: Accesory mode status (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Accessory mode status   +

+                         +

+ Mode:              aaa  +

+ Accessory mode:    bbb  +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Formats added / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | audio | COMM_FTD_DATA_AUD_ANADIGI_MODE | B:D | R | Mode: ANA – Analog / DIG - Digital |
| bbb | audio | COMM_FTD_DATA_AUD_ACCESSORY_MODE | B:D | R | Accessory mode: HP / HF / HDC5 / HDC9 / LPS1 / LPS3 / PPHF / PPHF_EXT / HFU_2 / HFU_2_EXT / ANCO_HP / ANCO_ANC / IHF / SMART |

Display 64.04: Downlink audio display (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Downlink audio          +

+                         +

+ UEM attenuator     aa   +

+ Peak value         bbbb +

+ Cut off counter    cccc +

+ Voice level        dddd +

+ Data lack          eeee +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Formats added, approved / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | DSP_AUDIO | FTD_SB_DNLINK_AUD_ATT | B:H | R | Downlink attenuator setting in UEM. |
| bbbb | DSP_AUDIO | FTD_SB_DNLINK_AUD_SIGNAL_PEAK | W:H | R | Peak value of the downlink signal since last request. Counter is reset after every request. |
| cccc | DSP_AUDIO | FTD_SB_DNLINK_AUD_CUT_OFF | W:H | R | Value of the downlink cut off counter since last request. Counter is reset after every request. |
| dddd | DSP_AUDIO | FTD_SB_DNLINK_VOICE_LEVEL | W:H | R | Dowlink voice level moving average in hex |
| eeee | DSP_AUDIO | FTD_SB_DNLINK_DATA_LACK | W:H | R | Counter for lack of downlink data |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

Display 64.05: Uplink audio display (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Uplink audio display    +

+                         +

+ UEM gain           aa   +

+ Peak value         bbbb +

+ Cut off count      cccc +

+ Voice level        dddd +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Formats added, approved / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | DSP_AUDIO | FTD_SB_UPLINK_AUD_GAIN | B:H | R | Uplink gain setting in UEM. |
| bbbb | DSP_AUDIO | FTD_SB_UPLINK_AUD_SIGNAL_PEAK | W:H | R | Peak value of the uplink signal since last request. Counter is reset after every request. |
| cccc | DSP_AUDIO | FTD_SB_UPLINK_AUD_CUT_OFF | W:H | R | Value of the uplink cut off counter since last request. Counter is reset after every request. |
| dddd | DSP_AUDIO | FTD_SB_UPLINK_VOICE_LEVEL | W:H | R | Uplink voice level instantaneous or moving average in hex |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

Display 64.06: Microphone Active Gain Control (AGC) (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Microphone AGC          +

+                         +

+ Gain analog        aaa  +

+ Gain digital       bbb  +

+ Treshold low       ccc  +

+ Treshold up        ddd  +

+ Analog counter     eeee +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Sub block ids for a and b fields changed, description corrections, formats added / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | AUDIO | COMM_FTD_DATA_AUD_AIN_GAIN | W:D | R | Analog microphone gain in dB. Decimal point and sign is not shown. E.g. –10.5dB is shown as "105". 20*log10(Q11). |
| bbb | DSP_AUDIO | FTD_SB_AGC_DGAIN | W:D | R | Digital compensation gain in dB. Decimal point is not shown. 20*log10(Q11). |
| ccc | DSP_AUDIO | FTD_SB_AGC_ LOWER_TRESHOLD | W:D | R | Lower threshold for Microphone AGC in dB. Decimal point and sign is not shown. E.g. –10.5dB is shown as "105". 20*log10(Q11). |
| ddd | DSP_AUDIO | FTD_SB_AGC_ UPPER_TRESHOLD | W:D | R | Upper threshold for Microphone AGC in dB. Decimal point and sign is not shown. 20*log10(Q11). |
| eeee | DSP_AUDIO | FTD_SB_AGC_ GAIN_CONTROL_COUNTER | W:H | R | Counter for analog gain control. Hex format (Q11) |

Display 64.07: Acoustic Echo Canceller (AEC) (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Acoustic echo canceller +

+                         +

+ EAA/Ada/ERL aaa bbb ccc +

+ RxG/TcG/GLi ddd eee fff +

+ TxN/Sta/CNo ggg h   iii +

+ RX VAD      kkkk        +

+ TX VAD      llll        +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Sub block id for e field changed, description corrections, formats added / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | DSP_AUDIO | FTD_SB _AEC_ ACOUSTIC_ATT | W:D | R | Electro acoustic attenuation of echo in dB. Decimal point and sign is not shown. E.g. –10.5dB is shown as "105". 20*log10(Q11). |
| bbb | DSP_AUDIO | FTD_SB _AEC_ ADAPTIVE_ATT | W:D | R | Adaptive attenuation of echo in dB. Decimal point is not shown. 20*log10(Q11). |
| ccc | DSP_AUDIO | FTD_SB _AEC_ ECHO | W:D | R | Total echo return loss in dB. Decimal point is not shown. 20*log10(Q11). |
| ddd | DSP_AUDIO | FTD_SB _AEC_ RX_ATT | W:D | R | RX attenuation gain in dB. Decimal point and sign is not shown. 20*log10(Q11). |
| eee | DSP_AUDIO | FTD_SB _AEC_ TX_ATT | W:D | R | TX attenuation gain in dB. Decimal point and sign is not shown. 20*log10(Q11). |
| fff | DSP_AUDIO | FTD_SB _AEC_ ATT_GAIN_LIMIT | W:D | R | Gain limit for RX and TX attenuators in dB. Decimal point and sign is not shown. 20*log10(Q11). |
| ggg | DSP_AUDIO | FTD_SB _AEC_ TX_NOISE | W :D | R | TX Noise level in dB. Decimal point and sign is not shown. 20*log10(Q11). |
| h | DSP_AUDIO | FTD_SB _AEC_ ADAPTIVE_FILTERS | B:D | R | Adaptive filter status. |
| iii | DSP_AUDIO | FTD_SB _AEC_ COMFORT_NOISE | W:H | R | Comfort noise generation in different 12 sub bands. Hex format, 12 bits. |
| kkkk | DSP_AUDIO | FTD_SB _AEC_ RX_VAD | W:H | R | 16 last RX VAD decisions in hex format. |
| lllll | DSP_AUDIO | FTD_SB _AEC_ TX_VAD | W:H | R | 16 last TX VAD decisions in hex format. |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

Display 64.08: Dynamic/Multi-Band Range Controller (DRC/MDRC) (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ DRC/MDRC                +

+                         +

+ DsL	    aaa bbb ccc +

+             ddd eee fff +

+ Uspl/UsL    ggg hhh     +

+ NsL         iii         +

+ Dtb/Utb     jjj kkk     +

+ DNTb        ll          +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	Description corrections, formats added, approved / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa bbb ccc ddd eee fff | DSP_AUDIO | FTD_SB_DRC_DL_SIGNAL_LEVEL | W:D | R | Downlink signal level in dB. 10*log10(Q11). Decimal point and sign is not shown. E.g. –10.5dB is shown as "105". |
| ggg | DSP_AUDIO | FTD_SB_DRC_UL_SPEECH_LEVEL | W:D | R | Uplink speech level in dB. 10*log10(Q11). Decimal point and sign is not shown. E.g. –10.5dB is shown as "105". |
| hhh | DSP_AUDIO | FTD_SB_DRC_UL_SIGNAL_LEVEL | W:D | R | Uplink signal level in dB. 10*log10(Q11). Decimal point and sign is not shown. E.g. –10.5dB is shown as "105". |
| iii | DSP_AUDIO | FTD_SB_DRC_NOISE_LEVEL | W:D | R | Backround noise signal level in dB. 10*log10(Q11). Decimal point and sign is not shown. E.g. –10.5dB is shown as "105". |
| jjj | DSP_AUDIO | FTD_SB_DRC_DL_TBL | W:D | R | Current downlink DRC table(s). |
| kkk | DSP_AUDIO | FTD_SB_DRC_UL_TBL | W:D | R | Current uplink DRC table(s). |
| ll | DSP_AUDIO | FTD_SB_DRC_DL_NOISE_TBL | B:D | R | Current downlink noise dependent table. |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

Display 64.09: Control DSP audio enhancements switch 1 (Version: 3.15, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ DSP audio enhancement 1 +

+                         +

+ Entered:       aaaa     +

+ Switch 1:      bbbb     +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / SWAM ADB PK02071536518

3.15	Proposal	11-Jul-02	AnttiE	Minor change to sub block ids, back to proposal / SWAM ADB: AE02071054323

3.13	Approved	27-Jun-02	AnttiE	Sub block ids changed, description corrections, formats added, approved / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | DSP_AUDIO | FTD_SB_BASIC_ALG_1_VALUE_TO_SWITCH_1 | DW:H | R | Value to be written to switch1 |
| aaaa | DSP_AUDIO | FTD_SB_BASIC_ALG_1_VALUE_OF_SWITCH_ 1 | W:H | R | Value written to switch1. |
| bbbb | DSP_AUDIO | FTD_SB_BASIC_ALG_1_CURR_VALUE_SWITCH_ 1 | W:H | R | Current value of switch1. Updated 5-10 times per second. |

[/FTD_FIELDS]

This can be provided by Audio Server, but currently is not supported

To control DSP audio enhancements, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 6409 (TEST GROUP;TEST DISPLAY) at the query prompt.

- Test input display will activate, enter the control words in display. Keyboard is in "text" mode, so that hex numbers 0-F can be written.

- Confirm with the Ok button.

- If activation succeeded, the control word is then sent to the DSP.

- Status of the registers is read 5-10 times per second and filelds a and b are updated accordingly. This allows the user to see the current values in these registers.

Used together withdisplays 64.10 and 64.11, this display makes rapid on/off switching of audio DSP algorithms possible. Switching with arrow keys is possible only after this display or display 2 has been selected from the menu. This prevents accidental on/off switching of algorithms when browsing displays by arrow keys. Entered values are not saved to permanent memory.

Display 64.10: Control DSP audio enhancements switch 2 (Version: 3.15, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ DSP audio enhancement 2 +

+                         +

+ Entered:       aaaa     +

+ Switch 2:      bbbb     +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / SWAM ADB PK02071536518

3.15	Proposal	11-Jul-02	AnttiE	Minor change to sub block ids, back to proposal / SWAM ADB: AE02071054323

3.13	Approved	27-Jun-02	AnttiE	Sub block ids changed, description corrections, formats added, approved / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | DSP_AUDIO | FTD_SB_BASIC_ALG_1_VALUE_TO_SWITCH_2 | DW:H | R | Value to be written to switch2 |
| aaaa | DSP_AUDIO | FTD_SB_BASIC_ALG_1_VALUE_OF_SWITCH_ 2 | W:H | R | Value written to switch2. |
| bbbb | DSP_AUDIO | FTD_SB_BASIC_ALG_1_CURR_VALUE_SWITCH_ 2 | W:H | R | Current value of switch2. Updated 5-10 times per second. |

[/FTD_FIELDS]

This can be provided by Audio Server, but currently is not supported

To control DSP audio enhancements, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 6410 (TEST GROUP;TEST DISPLAY) at the query prompt.

- Test input display will activate, enter the control words in display. Keyboard is in "text" mode, so that hex numbers 0-F can be written.

- Confirm with the Ok button.

- If activation succeeded, the control word is then sent to the DSP.

- Status of the registers is read 5-10 times per second and filelds a and b are updated accordingly. This allows the user to see the current values in these registers.

Display 64.11: Control DSP audio enhancements switch 3 (Version: 3.15, Status:Approved)

Data Display:

+++++++++++++++++++++++++++

+ DSP audio enhancement 3 +

+                         +

+ Entered:       aaaa     +

+ Switch 3:      bbbb     +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / SWAM ADB PK02071536518

3.15	Proposal	11-Jul-02	AnttiE	Minor change to sub block ids, back to proposal / SWAM ADB: AE02071054323

3.13	Approved	27-Jun-02	AnttiE	Sub block ids changed, description corrections, formats added, approved / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | DSP_AUDIO | FTD_SB_BASIC_ALG_1_VALUE_TO_SWITCH_3 | DW:H | R | Value to be written to switch3 |
| aaaa | DSP_AUDIO | FTD_SB_BASIC_ALG_1_VALUE_OF_SWITCH_ 3 | W:H | R | Value written to switch3. |
| bbbb | DSP_AUDIO | FTD_SB_BASIC_ALG_1_CURR_VALUE_SWITCH_ 3 | W:H | R | Current value of switch3. Updated 5-10 times per second. |

[/FTD_FIELDS]

This can be provided by Audio Server, but currently is not supported

To control DSP audio enhancements, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 6411 (TEST GROUP;TEST DISPLAY) at the query prompt.

- Test input display will activate, enter the control words in display. Keyboard is in "text" mode, so that hex numbers 0-F can be written.

- Confirm with the Ok button.

- If activation succeeded, the control word is then sent to the DSP.

- Status of the registers is read 5-10 times per second and filelds a and b are updated accordingly. This allows the user to see the current values in these registers.

Display 64.12: SpCs (Version: 3.13, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ SpCs                    +

+                         +

+ Rx type           aaaa  +

+ Tx type           bbbb  +

+ decode mode       cc    +

+ encode mode       dd    +

+ spC type          eeeee +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.13	Approved	27-Jun-02	AnttiE	A and b field size changed, description corrections, formats added / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | DSP_AUDIO | FTD_SB_RX_TYPE | W:H | R | Uplink flags (Speech/Background noise) |
| b(4) | DSP_AUDIO | FTD_SB_TX_TYPE | W:H | R | Downlink flags ( BFI, SID, TAF) |
| c(2) | DSP_AUDIO | FTD_SB_AMR_DEC_BITRATE | B:H | R | AMR SpC Decoder bitrate (00-07) |
| d(2) | DSP_AUDIO | FTD_SB_AMR_ENC_BITRATE | B:H | R | AMR SpC Encoder bitrate (00-07) |
| e(5) | DSP_AUDIO | FTD_SB_SpC_TYPE | S | R | SpC type in string (G_FR, G_EFR, G_AMR, WAMR1, or WAMR2) |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

Display 64.13: SpC counters (Version: 3.16, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ SpC counters            +

+                         +

+ C_handovers       aaaaa +

+ C_mode changes    bbbbb +

+ C_format changes  ccccc +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

3.16	Approved	21-Aug-02	AnttiE	Counters are not PPC:s any more / SWAM ADB PK02071536518

3.13	Rejected	27-Jun-02	AnttiE	Formats added, rejected because of obvious PPC problems / Inspection 14-Jun-02, SWAM ADB PK02061333948

1	Proposal	24-Apr-02	IHu	Original status: Proposal

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(5) | DSP_AUDIO | FTD_SB_C_HANDOVERS | W:D | R | Number of codec handovers |
| b(5) | DSP_AUDIO | FTD_SB_C_MODE_CHANGES | W :D | R | Number of codec mode changes |
| c(5) | DSP_AUDIO | FTD_SB_C_FORMAT_CHANGES | W :D | R | Number of codec format changes |

[/FTD_FIELDS]

These values are updated 5-10 times/second.

- Group 65: Common PPC (Product Performance Counters) displays

Display 65.01: PPC Counters 1 (Version: 3.19, Status: Approved)

+++++++++++++++++++++++++++

+      PPC counters 1     +

+                         +

+ Ccall timer    aaaa     +

+ Dmem manag     bb       +

+ DSP res count  cc       +

+ SW cr count    dd       +

+ ASIC wdog res  ee       +

+ Bad Pof count  ff       +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

Version	Status	Description / Reference

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | MON | FTD_SB_PPC_CUMUL_CALL_TIMER | DW:D | R | PPC number 36 : Cumulative Call Timer. |
| bb | MON | FTD_SB_PPC_DMM_NO_BUF | W:D | R | PPC number 12 : Dynamic Memory Management - No Buffers. |
| cc | MON | FTD_SB_PPC_DSP_RESET | W:D | R | PPC number 16 : DSP reset counter. |
| dd | MON | FTD_SB_PPC_SW_CRASH | B:D | R | PPC number 17 : SW crash counter. |
| ee | MON | FTD_SB_PPC_ASIC_WD_RESET | B:D | R | PPC number 18 : ASIC watchdog reset counter. |
| ff | MON | FTD_SB_BAD_POWER_OFF | W:D | R | PPC number 49 : Bad Power Off Counter. |

Display 65.02: PPC Counters 2 (Version: 3.19, Status: Approved)

+++++++++++++++++++++++++++

+      PPC counters 2     +

+ PA temp        aa       +

+ PA batt        bb       +

+ Batt temp      cc       +

+ Chr full       dd       +

+ Inv chr conn   eeee     +

+ Inv C/ Inv B   ff    gg +

+ I Btemp/Bsize  hh    ii +

+ Ccon / Cdis    kk    ll +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

Version	Status	Description / Reference

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | MON | FTD_SB_PPC_PA_OVER_T_PWR_OFF | B:D | R | PPC number 20 : PA Over Temperature Power off counter. |
| bb | MON | FTD_SB_PPC_PA_OVER_T_BATT_TEMP | B:D | R | PPC number 21 : PA Over Temperature Power off Battery temperature. |
| cc | MON | FTD_SB_PPC_BATT_OVER_T_PWR_OFF | B:D | R | PPC number 22 : Battery Over Temperature Power off counter. |
| dd | MON | FTD_SB_PPC_BATT_OVER_T_PA_TEMP | B:D | R | PPC number 23 : Battery Over Temperature Power off PA temperature |
| Eeee | MON | FTD_SB_PPC_BATT_CHARGED_FULL | DW:D | R | PPC number 24 : Battery Charged to Full Counter. |
| ff | MON | FTD_SB_PPC_INV_CHA_CONN | B:D | R | PPC number 27 : Invalid Charger Connected counter. |
| gg | MON | FTD_SB_PPC_INV_BATT_CONN | B:D | R | PPC number 29 : Invalid Battery Connection counter. |
| hh | MON | FTD_SB_PPC_INV_BATT_CONN_BTEMP | W:D | R | PPC number 30 : Invalid Battery Connection BTEMP. |
| ii | MON | FTD_SB_PPC_INV_BATT_CONN_BSIZE | W:D | R | PPC number 31 : Invalid Battery Connection BSIZE. |
| jjjj | MON | FTD_SB_PPC_CHA_CONN | W:D | R | PPC number 32 : Charger connection counter. |
| kkkk | MON | FTD_SB_PPC_CHA_DISCONN | W:D | R | PPC number 33 : Charger disconnection counter. |

Display 65.03: PPC Counters 3 (Version: 3.19, Status: Approved)

+++++++++++++++++++++++++++

+      PPC counters 3     +

+                         +

+ Dropped calls  aaaa     +

+                         +

+ Dropped calls           +

+ rssi          -bbb      +

+                         +

+ Dropped call            +

+ BER %          cccc     +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Formats added / Inspection 30-Sep-02, SWAM ADB TL02092341741

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | MON | FTD_SB_PPC_DROPPED_CALL | DW:D | R | PPC number 40 : Total number of dropped calls. |
| -bbb | MON | FTD_SB_PPC_DROPPED_CALL_RSSI | B:D | R | PPC number 41 : Dropped call rssi (in dBm). |
| cccc | MON | FTD_SB_PPC_DROPPED_CALL_BER | B:D | R | PPC number 43 : Dropped call ber(in %). |

Display 65.04: Camera PPCs (Version: 3.27, Status: Approved)

+++++++++++++++++++++++++++

+  Camera PPC counter     +

+                         +

+ App started    a(7)     +

+ Cont int fail  b(4)     +

+ Viewfinder run c(7)     +

+ Data int fail  d(4)     +

+ Pic landscape  e(4)     +

+ Pic portrait   f(4)     +

+ Discar images  g(4)     +

+ CCP / twilight h(2) I(4)+

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.19	Approved	Sub block id for f-field changed / Monitor server inspection, SWAM ADB AE02093045228

Version	Status	Date	Author	Description

3.15	Approved	21-Aug-02	AnttiE	Approved / Inspection 09-Aug-02, SWAM ADB TL02080538357, KK02071728766

3.15	Proposal	11-Jul-02	AnttiE	Display added / ISA RM-RIM tool 5533

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(7) | MON | FTD_SB_PPC_CAMERA_APP_START | DW:D | R | Camera application started |
| b(4) | MON | FTD_SB_PPC_CAMERA_CONTROL_FAILED | W:D | R | Camera control interface failed |
| c(7) | MON | FTD_SB_PPC_CAMERA_VIEW_RUNTIME | DW:D | R | Camera viewfinder runtime counter |
| d(4) | MON | FTD_SB_PPC_CAMERA_DATA_FAILED | W:D | R | Camera data interface failed |
| e(4) | MON | FTD_SB_PPC_PICTURE_LANDSCAPE | W:D | R | Pictures taken landscape |
| f(4) | MON | FTD_SB_PPC_PICTURE_PORTRAIT | W:D | R | Pictures taken portrait |
| g(4) | MON | FTD_SB_PPC_PICTURE_DISCARDED | W:D | R | Discarded images |
| h(2) | MON | FTD_SB_PPC_COMPACT_CAMERA_PORT_STUCK | B:D | R | CCP stuck counter |
| I(4) | MON | FTD_SB_PPC_PICTURE_TWILIGHT | W:D | R | Pictures taken twilight |

Display 65.05: HW UI counters (Version: 3.28, Status: Approved)

+++++++++++++++++++++++++++

+     HW UI counter       +

+                         +

+ Flip open      a(8)     +

+                         +

+                         +

+ EL on time     b(8)     +

+                         +

+ Keypad Chinese character+

+ input counter  c(8)     +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.28	Approved	PPC counter ids added to field descriptions / Antti E

3.25	Approved	Approved / Inspection 11-Nov-02, SWAM ADB JA02112040764

3.23	Proposal	Display added / ISA RM-RIM tool 001-1

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | MON | FTD_SB_PPC_FLIP_OPEN_TIMES | DW:D | R | PPC 1014: Flip open counter |
| b(8) | MON | FTD_SB_PPC_EL_ON_TIME | DW:D | R | PPC 1015: EL on time counter (in second) |
| c(8) | MON | FTD_SB_PPC_KEYPAD_CHINESE_CHARACTER_INPUT | DW:D | R | PPC 1016: Keypad Chinese Character Input Counter Note: repeated in display 76.06. |

- Group 73: Common Accessory displays

Please refer Accessory Server specific documentation for more information about the data items.

Display 73.01: Accessory mode information 1 (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Accessory mode info 1   +

+                         +

+ Accessory mode     aa   +

+ Accessory detect   bb   +

+ Last head Int      cccc +

+ Last hook int      dddd +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | Accessory | ACC_SB_FTD_ACC_MODE |  | R | Accessory mode (byte, hex) |
| bb | Accessory | ACC_SB_FTD_ACC_DET_ON |  | R | Accessory detection in progress flag: 0 / 1 (byte, hex) |
| cccc | Accessory | ACC_SB_FTD_ACC_HEADINT |  | R | Last HeadInt channel reading (word, hex) |
| dddd | Accessory | ACC_SB_FTD_ACC_HOOKINT |  | R | Last HookInt channel reading (word, hex) |

Display 73.02: Accessory mode information 2 (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Accessory mode info 2   +

+                         +

+ Loop counter       aa   +

+ P&P hf wrong inst  bb   +

+ P&P hf power down  cc   +

+ Charger disconnect dd   +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | Accessory | ACC_SB_FTD_ACC_COUNTER |  | R | Accessory detection loop counter value (byte, hex) |
| bb | Accessory | ACC_SB_FTD_ACC_PPHF_WI |  | R | Plug and play handsfree wrong installation flag: 0 / 1 (byte, hex) |
| cc | Accessory | ACC_SB_FTD_ACC_PPHF_DOWN |  | R | Plug and play handsfree power down flag: 0 / 1 (byte, hex) |
| dd | Accessory | ACC_SB_FTD_ACC_CHRG_DC |  | R | Charger disconnection detected flag: 0 / 1 (byte, hex) |

Display 73.03: Accessory mode information 3 (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Accessory mode info 3   +

+                         +

+ Headset btn down     aa +

+ HFU car kit state    bb +

+ HFU car kit state up cc +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | Accessory | ACC_SB_FTD_ACC_HD_BTN_DOWN |  | R | Headset detected in button down state flag: 0 / 1 (byte, hex) |
| bb | Accessory | ACC_SB_FTD_ACC_HFU2_INFO |  | R | HFU-2x car kit state indication data (byte, hex) |
| cc | Accessory | ACC_SB_FTD_ACC_HFU2_SU_INFO |  | R | HFU-2x car kit start up indication data (byte, hex) |

Display 73.04: Vibra information (Version: 1, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Vibra information       +

+                         +

+ Vibra state         aa  +

+ Vibra intensity     bb  +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | Accessory | ACC_SB_FTD_ACC_VBR_STATE |  | R | Vibra state (off/on) flag: 0 / 1 (byte, hex) |
| bb | Accessory | ACC_SB_FTD_ACC_VBR_INT |  | R | Vibra intensity (relative clock frequency) data: 0x00 - 0x64 (byte, hex) |

Display 73.06: Ambient light sensor information 1 (Version: 1, Status: Approved )

Data Display:

+++++++++++++++++++++++++++

+ Ambient light sensor    +

+  info 1                 +

+                         +

+ Luminance level    aa   +

+ Calibrated value   bbbb +

+ Temp comp value    cccc +

+ Raw AD value       dddd +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aa | Accessory | ACC_SB_FTD_ACC_ALI_LUM | B:H | R | Ambient light luminance level (byte, hex) |
| bbbb | Accessory | ACC_SB_FTD_ACC_ALI_CAL_VAL | W:H | R | Ambient light calibrated value (word, hex) |
| cccc | Accessory | ACC_SB_FTD_ACC_ALI_TEMP_VAL | W:H | R | Ambient light temperature compensated value (word, hex) |
| dddd | Accessory | ACC_SB_FTD_ACC_ALI_RAW_VAL | W:H | R | Ambient light raw AD value (word, hex) |

Display 73.07: Ambient light sensor information  2 (Version: 1, Status: Approved )

Data Display:

+++++++++++++++++++++++++++

+ Ambient light sensor    +

+  info 2                 +

+                         +

+ CalConv cal value  aaaa +

+ VCXO temperature   bbbb +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(4) | ACCESSORY | ACC_SB_FTD_ACC_ALI_CALCONV | W:H | R | Ambient light CalConv –calibration value (word, hex) |
| b(4) | ACCESSORY | ACC_SB_FTD_ACC_ALI_VCXOTEMP | B:D | R | VCXO Temperature (byte, decimal) |

- Group 74: Common Memory Management displays

+++++++++++++++++++++++++++

+ MeM component versions  +

+                         +

+ PMM release ID          +

+           a(11)         +

+ PERM release ID         +

+           b(11)         +

+ NVD release ID          +

+           c(11)         +

+                         +

+++++++++++++++++++++++++++

Display 74.01: Information of MeM component versions (Version: 1, Status: Approved)

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(11) | PERM | PERM_SB_FTD_PMM_VERSION | S | R | PMM release ID |
| b(11) | PERM | PERM_SB_FTD_PERM_VERSION | S | R | PERM release ID |
| c(11) | NVD | PERM_SB_FTD_NVD_VERSION | S | R | NVD release ID |

Display 74.02: Information of PMM Plain area (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+     PMM Plain area      +

+ Memory used    a(7)     +

+ % used         b(4)     +

+ Memory release c(7)     +

+ % released     d(4)     +

+ Memory unused  e(7)     +

+ % unused       f(4)     +

+ Number of bloc g(3)     +

+ Block size     h(7)     +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(7) | PERM | PERM_SB_FTD_PMM_PLAIN_TOTAL_MEMORY_USED |  | R | Memory Used |
| b(4) | PERM | PERM_SB_FTD_PMM_PLAIN_TOTAL_MEMORY_USED_PC | B:D | R | % Used |
| c(7) | PERM | PERM_SB_FTD_PMM_PLAIN_TOTAL_MEMORY_RELEASED |  | R | Memory Released |
| d(4) | PERM | PERM_SB_FTD_PMM_PLAIN_TOTAL_MEMORY_RELEASED_PC | B:D | R | % Released |
| e(7) | PERM | PERM_SB_FTD_PMM_PLAIN_TOTAL_MEMORY_UNUSED |  | R | Memory Unused |
| f(4) | PERM | PERM_SB_FTD_PMM_PLAIN_TOTAL_MEMORY_UNUSED_PC | B:D | R | % Unused |
| g(3) | PERM | PERM_SB_FTD_PMM_PLAIN_TOTAL_PAGE_NUMBER | W:D | R | Number of Blocks (configuration) |
| h(7) | PERM | PERM_SB_FTD_PMM_PLAIN_PAGE_SIZE |  | R | Block size (configuration) |

Display 74.03: Information of PMM Classified area (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+   PMM classified area   +

+ Memory used    a(7)     +

+ % used         b(4)     +

+ Memory release c(7)     +

+ % released     d(4)     +

+ Memory unused  e(7)     +

+ % unused       f(4)     +

+ Number of bloc g(3)     +

+ Block size     h(7)     +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaaaaa | PERM | PERM_SB_FTD_PMM_CLASS_TOTAL_MEMORY_USED |  | R | Memory Used |
| bbbb | PERM | PERM_SB_FTD_PMM_CLASS_TOTAL_MEMORY_USED_PC | B:D | R | % Used |
| ccccccc | PERM | PERM_SB_FTD_PMM_CLASS_TOTAL_MEMORY_RELEASED |  | R | Memory Released |
| dddd | PERM | PERM_SB_FTD_PMM_CLASS_TOTAL_MEMORY_RELEASED_PC | B:D | R | % Released |
| eeeeeee | PERM | PERM_SB_FTD_PMM_CLASS_TOTAL_MEMORY_UNUSED |  | R | Memory Unused |
| ffff | PERM | PERM_SB_FTD_PMM_CLASS_TOTAL_MEMORY_UNUSED_PC | B:D | R | % Unused |
| gggg | PERM | PERM_SB_FTD_PMM_CLASS_TOTAL_PAGE_NUMBER |  | R | Number of Blocks (configuration) |
| hhhhhhh | PERM | PERM_SB_FTD_PMM_CLASS_PAGE_SIZE |  | R | Block size (configuration) |

Display 74.04: Information of PMM Protected area (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+   PMM protected area    +

+ Memory used    a(7)     +

+ % used         b(4)     +

+ Memory release c(7)     +

+ % released     d(4)     +

+ Memory unused  e(7)     +

+ % unused       f(4)     +

+ Number of bloc g(3)     +

+ Block size     h(7)     +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaaaaa | PERM | PERM_SB_FTD_PMM_PROT_TOTAL_MEMORY_USED |  | R | Memory Used |
| bbbb | PERM | PERM_SB_FTD_PMM_PROT_TOTAL_MEMORY_USED_PC | B:D | R | % Used |
| ccccccc | PERM | PERM_SB_FTD_PMM_PROT_TOTAL_MEMORY_RELEASED |  | R | Memory Released |
| dddd | PERM | PERM_SB_FTD_PMM_PROT_TOTAL_MEMORY_RELEASED_PC | B:D | R | % Released |
| eeeeeee | PERM | PERM_SB_FTD_PMM_PROT_TOTAL_MEMORY_UNUSED |  | R | Memory Unused |
| ffff | PERM | PERM_SB_FTD_PMM_PROT_TOTAL_MEMORY_UNUSED_PC | B:D | R | % Unused |
| gggg | PERM | PERM_SB_FTD_PMM_PROT_TOTAL_PAGE_NUMBER |  | R | Number of Blocks (configuration) |
| hhhhhhh | PERM | PERM_SB_FTD_PMM_PROT_PAGE_SIZE |  | R | Block size (configuration) |

Display 74.05: PMM Storing information (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+   PMM protected area    +

+                         +

+ % Rec. foreg.  a(3)     +

+ % Rec. backg.  b(3)     +

+ Fragments      c(4)     +

+ Pool size      d(8)     +

+ Mem reserv.    e(8)     +

+ Swap size      f(8)     +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaa | PERM | PERM_SB_FTD_PMM_FOREGROUND_WRITE_PC | B:D | R | % Records written in Foreground |
| bbb | PERM | PERM_SB_FTD_PMM_BACKGROUND_WRITE_PC | B:D | R | % Records written in Background |
| cccc | PERM | PERM_SB_FTD_PMM_FRAGMENT_COUNTER |  | R | Total amount of fragments |
| dddddddd | PERM | PERM_SB_FTD_PMM_PLAIN_POOL_SIZE_QUERY |  | R | Remaining Pool size in Plain area |
| eeeeeeee | PERM | PERM_SB_FTD_PMM_PLAIN_MEM_TOTAL_RES_QUERY |  | R | Memory reservations in Plain area |
| ffffffff | PERM | PERM_SB_FTD_PMM_PLAIN_SWAP_SIZE |  | R | Swap size in Plain area |

Display 74.06: PMM Error information 1 (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+   PMM Error info 1      +

+                         +

+ Log adress     a(8)     +

+ Error code     b(8)     +

+ Write address  c(4)     +

+ Read address   d(8)     +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaaaaaa | PERM | PERM_SB_FTD_PMM_ERROR_LOG_PRESENT |  | R | Log address |
| bbbbbbbb | PERM | PERM_SB_FTD_PMM_ERROR_CODE |  | R | Error code |
| cccccccc | PERM | PERM_SB_FTD_PMM_ERROR_WRITE_ADDRESS |  | R | Write address |
| dddddddd | PERM | PERM_SB_FTD_PMM_ERROR_READ_ADDRESS |  | R | Read address |

Display 74.07: PMM Error information 2 (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+   PMM Error info 2      +

+                         +

+ Write error counter     +

+ a(8)                    +

+ Read error counter      +

+ b(8)                    +

+ Calling group ID        +

+ c(4)                    +

+ Calling index number    +

+ d(8)                    +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) | PERM | PERM_SB_FTD_PMM_DEV_FLASH_WRITE_ERROR_COUNTER |  | R | Write error counter |
| b(8) | PERM | PERM_SB_FTD_PMM_DEV_FLASH_READ_ERROR_COUNTER |  | R | Read error counter |
| c(4) | PERM | PERM_SB_FTD_PMM_CALLING _GROUP |  | R | Calling group ID |
| d(4) | PERM | PERM_SB_FTD_PMM_CALLING _INDEX |  | R | Calling Index number |

Display 74.08: Information of PMM Clean-up (Version: 1, Status: Approved)

+++++++++++++++++++++++++++

+    PMM clean-up         +

+                         +

+ Retry peak data read    +

+ a(4)                    +

+ Retry peak data write   +

+ b(4)                    +

+ Retry peak header read  +

+ c(3)                    +

+ Cleanup peak	d(3)  +

+ Writeback peak    e(3)  +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa | PERM | PERM_SB_FTD_PMM_ERROR_RETRY_DATA_READ |  | R | Retry peak on record data read |
| bbbb | PERM | PERM_SB_FTD_PMM_ERROR_RETRY_DATA_WRITE |  | R | Retry peak on record data write |
| ccc | PERM | PERM_SB_FTD_PMM_ERROR_RETRY_HEADER_READ |  | R | Retry peak on record header read |
| ddd | PERM | PERM_SB_FTD_PMM_PEAK_CLEANUP_MAILBOX |  | R | Cleanup peak (in mailbox) |
| eee | PERM | PERM_SB_FTD_PMM_PEAK_WRITEBACK_MAILBOX |  | R | Writeback peak (in mailbox) |

Group 81: Multimode protocol displays

Display 81.01: Force protocol between GSM and WCDMA (Version: 3.26, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Force protocol between  +

+ GSM and WCDMA           +

+                         +

+ 0-dual                  +

+ 1-GSM                   +

+ 2-WCDMA                 +

+                         +

+ a	                +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.26	Approved	Approved / Inspection 11-Nov-02, SWAM ADB AE02112043720

3.25	Proposal	TDMA-GSM and WCDMA-GSM specifications separated. Rewritten to match implementation. Mode changed to R,I,O / Inspection 11-Nov-02, SWAM ADB AE02112043720, AE02111948909

Version	Status	Date	Author	Description

1	Approved	24-Apr-02	IHu	Original status: Approved

[/Version history]

To change Force Protocol status, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in input mode by entering 8101 (Multimode protocol displays; Force protocol) to the query prompt.

- Test input prompt will activate, enter the Force Protocol status code (see the table below) in display.

- Confirm with the Ok button.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| INPUT | GSS | FTD_SB_SELECTED_RAT_SET | DW:D | R,I,O | Force protocol 0 = dual mode 1 = GSM 2 = WCDMA |
| a | GSS | FTD_SB_SELECTED_RAT_READ | B:D | R,I,O | Forced protocol: 0 = dual mode 1 = GSM 2 = WCDMA 255 = unknown |

Display 81.02: Toggle Integrity Protection Mode (Version: 3.26, Status: Approved)

Data Display:

+++++++++++++++++++++++++++

+ Toggle intergrity       +

+ protection mode on/off  +

+                         +

+ aaa                     +

+                         +

+                         +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

[Version history]

Version	Status	Description / Reference

3.26	Approved	Approved / Inspection 11-Nov-02, SWAM ADB AE02112043720

3.25	Proposal	Mode changed to R,I,O / Inspection 11-Nov-02, SWAM ADB AE02111948909

3.21	Approved	Sub block ids truncated / Inspection 09-Oct-02, SWAM ADB TA02101040863

3.20	Approved	Display added / ISA RM-RIM tool 009-31 & Inspection 09-Oct-02, SWAM ADB AE02100942067

[/Version history]

To change integrity protection mode, perform following steps:

- Press the Menu button.

- Scroll in the main menu loop to field test display item.

- Press the Select button.

- Select this display in executive mode by entering 8102 (Multimode protocol displays;toggle integrity protection mode) to the query prompt.

- Confirm with the Ok button.

[FTD_FIELDS]

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| EXE | GSS | FTD_SB_GSS_INT_PRO_MODE_TOGGLE | N/A | R,I,O | Toggle integrity protection mode. |
| a(3) | GSS | FTD_SB_GSS_INT_PRO_MODE | S | R,I,O | Current integrity protection mode: ON : Enabled OFF : Disabled |

- Group 99: Charlie specific displays

Display 99.01: PPC Counters

Data Display:

+++++++++++++++++++++++++++

+ PPC Counters            +

+                         +

+ Reflash  aaaa           +

+ Ins SIM  bbbb           +

+ SW reset cccc           +

+                         +

+ SYMBIAN panic prog dddd +

+ SYMBIAN panic data eeee +

+ SYMBIAN panic reas ffff +

+++++++++++++++++++++++++++

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| aaaa |  |  |  |  | PPC number 50 : Reflash Counter |
| bbbb |  |  |  |  | PPC number 94 : Insert SIM Card Error counter |
| cccc |  |  |  |  | PPC number 327 : SW reset reason |
| dddd |  |  |  |  | PPC number 331 : SYMBIAN Panic program counter |
| eeee |  |  |  |  | PPC number 332 : SYMBIAN Panic data abort counter |
| ffff |  |  |  |  | PPC number 333 : SYMBIAN Panic reason counter |

Display 99.04: Einfo on FTD

Data Display:

+++++++++++++++++++++++++++

+ Einfo                   +

+                         +

+ APE SW         aaaaaaaa +

+ CellMo SW      bbbbbbbb +

+ APE image      cccccccc +

+ HWId           dddddddd +

+                         +

+                         +

+                         +

+++++++++++++++++++++++++++

| Abbr. | Server | Sub Block ID | Format | Mode | Description |
| --- | --- | --- | --- | --- | --- |
| a(8) |  |  |  |  | APE SW (AMCU, ADSP) |
| b(8) |  |  |  |  | CellMo SW (CMCU, CDSP) |
| c(8) |  |  |  |  | APE variant image version |
| d(8) |  |  |  |  | HWIDn versionro |

# REFERENCES

1. Original GSM Field Test Display Specification:

Part:	HD9413:SPEC.BASE

Item:	HD9413:FIELD_TEST_DISPLAY_SPEC.BASE-SPEC

File:	hd94x_field_test_display_spec.txt

2. Original TDMA Field Test Display Specification:

DAMPS_CS_SW: IS136_FT_DISPLAY_SPEC.BASE-SPEC.

3. Field Test Display – Application configuration:

Workset:	ISA:WS_ISA

Part:	ISA:DOCUMENTS.BASE

Item: 	ISA:USER_GUIDES.BASE

File: 	FTD_Appl_conf_guide.doc

4. Product Performance counters specification:

Doc No: DSP00033-EN

Database: WinTesla-Camberley (UK)

Filename: document\wintesla\ppc_spec.doc

5. BT102 locals message description, v10.2 27.9.2001. Proposal. Thomas Busse. \\trenfs05\Calypso\sw\System\Bluetooth\Documents\Bochum\BT102_loc_spec.doc

6. BT102 – Product performance counter, v8.0 15.8.2001. Approved. Thomas Busse

\\trenfs05\Calypso\sw\System\Bluetooth\Documents\Bochum\BT102_ppc_spec.doc

7. BT102 FTD event message description, v4.0 29.3.2001. Approved. Daniel Bencak, Dietmar Schepp \\trenfs05\Calypso\sw\System\Bluetooth\Documents\Bochum\BT102_ftd_event_msg.doc